3GPP RAN WG4 Meeting #114	R4-2501042

Athens, GR, February 17th – February 21st, 2025

Title: 	Discussion on UE to UE CLI handling

Source: 	CATT

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 made fruitful progress of the UE to UE CLI requirements in R19 SBFD WI. Some agreements were achieved and captured in the way forward [1]. In this contribution, we will further discuss the UE to UE CLI handling.

Discussion

Measurement methods

Regarding measurement methods, the recommended WF from RAN4#112bis was agreed in last meeting as below:

RAN4#113 Agreement: - Recommended WF - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

In RAN1#119 meeting, Method #3 was also supported [2]:

RAN1#119 Agreement: For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Therefore RAN4 should also specify L1 CLI-RSSI measurement requirements for Method #3. For both Method #1 and Method #3, since UE measures power in different frequency subbands over indicated OFDM symbols to derive RSSI results irrelative with the existence of reference signals, RAN4 can define a single set of measurement requirements, including measurement period and performance requirements, for both Method #1 and Method #3.

Proposal 1: RAN4 to specify L1 CLI-RSSI measurement requirements for Method #3, and define a single set of measurement requirements for both Method #1 and Method #3.

Rx beam

In regards to the assumption of Rx beam, RAN1 has made an agreement in RAN1#118bis and RAN4 agreed to capture RAN1 agreements into RRM specification. But the way of capturing RAN1 agreements is still undetermined.

RAN4#113: - Agreements (from Thu AH session) - RAN4 to capture the Rx beam determination for L1 CLI measurement based on RAN1 agreement, FFS how to capture, e.g. specify the UE behaviour or use a reference to RAN1 spec. - Rx beam sweeping is not needed for L1 CLI measurement.

Considering RAN1 will capture their agreements into RAN1 specification in a detailed way, we prefer that RAN4 uses a reference to RAN1 spec rather than capturing the same contents in RAN4 spec.

Proposal 2: RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements.

Measurement resources

Regarding the impacts of measurement resources and reporting on measurement delay and accuracy requirements, some proposals were brought up in last meeting. In RAN1#119 meeting, some progress were made, which can help RAN4 to proceed. The related RAN1 conclusions are copied below:

RAN1#119 Agreement: Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19. Conclusion For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

According to current RAN1’s design, only wideband reporting is supported for both L1 CLI SRS-RSRP and RSSI measurements and there is no consensus of further discussing whether to support subband reporting in Rel-19. Besides, it is also clarified that only one wideband result is reported when one CLI-RSSI measurement resource is mapped into two DL-subbands. Therefore RAN4 does not need to consider defining accuracy requirements for subband reporting.

Proposal 3: RAN4 does not need to define measurement accuracy requirements for subband reporting for L1 CLI measurements.

Side conditions

Timing offset

The assumption of timing offset for L1 CLI measurements depends on specific scenarios and a LS was sent to RAN1 for clarification. In RAN1#119 meeting, the LS was discussed and the following agreement is sent back to RAN4 [3]:

RAN1 Agreement: Send reply LS to RAN4 with following information: When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

According to RAN1 agreement, intra-cell UE-to-UE L1 CLI measurement scenario is prioritized, and therefore the cell phase error can be removed from the Rel-16 SRS-RSRP side condition.

Proposal 4: Intra-cell UE-to-UE L1 CLI measurement is prioritized and cell phase error can be removed from Rel-16 CLI timing offset assumption.

SRS Es/Iot and BW

Based on the summary of simulation results [4], we observed that the cell phase error has a significant impact on L1 SRS-RSRP measurement results. If RAN4 agrees to remove cell phase error, which can significantly reduce the measurement error, especially for large SCS configurations. And then selection of number of measurement samples and SRS Es/Iot will have a limited effect on measurement performance. For SRS Es/Iot, it is acceptable to reuse the Rel-16 value, -1 dB.

In regards to the assumption of number of PRBs of SRS resources for L1 CLI measurements, RAN4 needs to consider the sizes of UL subband and total channel bandwidth. For most of SCS and band combinations, it is reasonable to reuse the assumption of 48 PRBs. However, when SCS equals 60 kHz in FR1 bands, there might be some issues.

According to agreements from RF groups, the channel bandwidths supporting SBFD shall be equal to or larger than 40 MHz except for n39, and the size of UL subbands will be chosen and declared from the Table 5.3.2-1 in TS 38.104 [5]. Hence, the applicable sizes of channel bandwidth for 60 kHz SCS in FR1 bands are highlighted below:

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

If SRS-RSRP measurements apply when the size of SRS BW is not smaller than 48 PRBs, which means the size of UL subband shall not be smaller than 48 PRBs. Furthermore, if the size of DL subbands is also included, assuming it is DU case and the sizes of UL subband and DL subband are the same, at least 96 PRBs are needed. And when guardband is required, larger channel bandwidths should be configured. Based on the above analysis, it is clear that up to 3 options are applicable for channel bandwidths configured with 60 kHz SCS in FR1 bands for DU case, and DUD configuration is not applicable in this case. If a UL subband smaller than 48 PRBs is configured, configuring some small bandwidths and DUD case become applicable from the network perspective, however the SRS-RSRP measurement performance requirement will not apply any more.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

As we discussed above, if cell phase error is removed, SRS-RSRP measurement error will be effectively reduced and the benefit of using a large number of PRBs is limited. Therefore in order to make performance requirements applicable for more channel BW configurations with SCS 60 kHz, we slightly prefer 24 PRBs between two SRS BW assumptions, i.e., 24 and 48 PRBs.

Proposal 5: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot = 1 dB.

- SRS BW: 24 PRBs.

Measurement period

A main issue for defining L1 CLI SRS-RSRP measurement period is to select the value of measurement sample. In our opinion, RAN4 needs to consider the NW configuration as well. In CSI-ReportConfig, network may configure the IE, timeRestrictionForChannelMeasurements, to restrict the measurement duration. As defined in L1-RSRP measurement requirements, number of measurement samples can be 1 if the mentioned IE is configured as ‘configured’. Otherwise, it equals to 3. Since RAN1 agreed to reuse CSI framework for L1 CLI measurement and reporting, this description can be reused for L1 CLI SRS-RSRP. Note that this is applicable for periodic and semi-persistent SRS resources. For aperiodic resources, the measurement sample is 1.

Proposal 6: For periodic and semi-persistent SRS resources, the number of measurement samples equals to 1, if timeRestrictionForChannelMeasurements is ‘configured’. Otherwise it equals to 3.

Proposal 7: For aperiodic resources, the measurement sample is 1.

Measurement reporting

Only aperiodic reporting is supported till now and no new reporting type is supported by RAN1. Therefore RAN4 can focus on aperiodic reporting and confirm the following recommended WF in RAN4#113:

- Recommended WF - For aperiodic reporting, L1-RSRP measurement reporting requirement for aperiodic reporting can be re-used as baseline.

Proposal 8: RAN4 to reuse L1-RSRP measurement reporting requirements for L1 CLI SRS-RSRP aperiodic reporting.

Scheduling restriction

Many proposals about scheduling restriction for L1 CLI measurements were brought up and a recommended WF was provided as follows:

- Recommended WF - For scheduling restriction, - For L1-SRS-RSRP, - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing.

Firstly, UE is not expected to perform UL transmissions during L1 CLI measurements in both FR1 and FR2 irrespective of Rx beam assumption.

Secondly, for DL receptions of CLI resources in FR1, scheduling restrictions are irrespective of Rx beam assumptions. However, the UE capabilities defined in R16, cli-RSSI-FDM-DL-r16 and cli-SRS-RSRP-FDM-DL-r16, should be considered. When UE does not support simultaneous receptions of CLI resources and DL signal/channel in a FDM manner, DL scheduling restrictions should be defined as what are defined in R16 scheduling restrictions.

Thirdly, for DL receptions of CLI resources in FR2, different Rx beam assumptions should be considered. According to RAN1 agreements, there are three Rx assumptions for CLI resource receptions in FR2:

- UE follows the ‘typeD’ TCI state(s) configured in CSI-AssociatedReportConfigInfo for L1 CLI measurements, if it is configured.

- If ‘typeD’ TCI state(s) is not configured but unifiedTCI-StateType is configured, UE uses the indicated DL only/joint TCI state specified in TS 38.214.

- Otherwise, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Apparently, DL scheduling restrictions are needed if the configured CLI measurement resources are not QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET. Similarly, whether the UE is capable of receiving CLI resources and DL signal/channel in a FDM manner should be considered in FR2.

In regards with the number of restricted symbols, since different measurement timings are assumed for L1 SRS-RSRP measurement and L1 CLI-RSSI measurement, different values might be used. For L1 SRS-RSRP measurement, the timing assumption is similar to the one in Rel-16 CLI measurement requirements. A small difference is that cell phase error can be removed, if agreed, from the timing error assumption. Therefore, in our opinion, the number of restricted symbols defined in R16 CLI scheduling restrictions can be reused for L1 SRS-RSRP measurements. For L1 CLI-RSSI measurements, the measurement is based on DL timing of serving cell. Hence, the number of restricted symbols equals the number of CLI resources.

Proposal 9: UL scheduling restrictions are needed for both FR1 and FR2 irrespective of Rx beam assumptions, and UL scheduling restrictions in Rel-16 can be reused.

Proposal 10: For DL receptions, when UE does not support cli-SRS-RSRP-FDM-DL or cli-RSSI-FDM-DL in FR1 and FR2, or CLI resources are not QCL-ed with TypeD to one of the latest received PDSCH or the latest monitored CORESET in FR2, scheduling restrictions are defined where the number of restricted symbols equals the number of CLI resources.

Measurement restriction

Regarding the measurement restriction, RAN4 agreed to reuse R16 measurement restriction requirements with an open issue:

RAN4#113 Agreement: - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

In our understanding, the exception cases mean that UE can receive resources of different types of measurements at the same time. For example, UE could simultaneously receive multiple resources in FR1 or multiple resources QCLed with TypeD in FR2 if frequency configurations are appropriate. If UE supports to perform measurements in parallel, it can reduce measurement delay. Therefore we slightly prefer to consider those exception cases.

Proposal 11: Prefer to consider the exception cases where UE can receive CLI resources and other resources at the same time, e.g., resources QCLed with TypeD in FR2.

Requirement applicability

In order to facilitate the discussion on this issue, two sub-issues are provided for further discussion:

- Recommended WF - The proposals are close to each other, further discuss the following issues and come up with combined the wording - Whether we define L1 CLI measurement resources to be confined within UL or DL subband or just DL active BWP - Whether we define L1 CLI measurement as intra-frequency

Since RAN1 agreed to support CLI-RSSI measurements in UL subband in RAN1#119, then it is reasonable to confine the L1 CLI measurement resources within DL active BWP.

For the second sub-bullet, a definition of L1 CLI measurement similar to that of L1-RSRP measurement can be expected in RAN1 spec, TS 38.215, where the following description is included in SSB-based L1-RSRP definition:

“If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency.”

Hence, we prefer to clarify that the L1 CLI measurement is applicable for RRC_CONNECTED intra-frequency in RRM spec.

Proposal 12: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 CLI measurement resources are confined within BW of DL active BWP.

Conclusions

This contribution discussed the impacts of L1 UE-to-UE CLI on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to specify L1 CLI-RSSI measurement requirements for Method #3, and define a single set of measurement requirements for both Method #1 and Method #3.

Proposal 2: RAN4 to use a reference to RAN1 spec to reflect the Rx beam assumption for L1 UE-to-UE CLI measurements.

Proposal 3: RAN4 does not need to define measurement accuracy requirements for subband reporting for L1 CLI measurements.

Proposal 4: Intra-cell UE-to-UE L1 CLI measurement is prioritized and cell phase error can be removed from Rel-16 CLI timing offset assumption.

Observation 1: L1 SRS-RSRP performance requirements will not apply for most of channel BW configurations with 60 kHz SCS in FR1, if 48 PRBs SRS BW is assumed.

Proposal 5: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- SRS Es/Iot = 1 dB.

- SRS BW: 24 PRBs.

Proposal 6: For periodic and semi-persistent SRS resources, the number of measurement samples equals to 1, if timeRestrictionForChannelMeasurements is ‘configured’. Otherwise it equals to 3.

Proposal 7: For aperiodic resources, the measurement sample is 1.

Proposal 8: RAN4 to reuse L1-RSRP measurement reporting requirements for L1 CLI SRS-RSRP aperiodic reporting.

Proposal 9: UL scheduling restrictions are needed for both FR1 and FR2 irrespective of Rx beam assumptions, and UL scheduling restrictions in Rel-16 can be reused.

Proposal 10: For DL receptions, when UE does not support cli-SRS-RSRP-FDM-DL or cli-RSSI-FDM-DL in FR1 and FR2, or CLI resources are not QCL-ed with TypeD to one of the latest received PDSCH or the latest monitored CORESET in FR2, scheduling restrictions are defined where the number of restricted symbols equals the number of CLI resources.

Proposal 11: Prefer to consider the exception cases where UE can receive CLI resources and other resources at the same time, e.g., resources QCLed with TypeD in FR2.

Proposal 12: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 CLI measurement resources are confined within BW of DL active BWP.

References

[1] R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4#113.

[2] Chair notes RAN1#119, RAN1#119.

[3] R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN1#119.

[4] R4-2420166, Summary of simulation results for L1-SRS-RSRP, Huawei, RAN4#113.

[5] TS 38.104, Base Station (BS) radio transmission and reception, V18.8.0.