3GPP RAN WG4 Meeting #114	R4-2501048

Athens, GR, February 17th – February 21st, 2025

Title: 	Discussion on the impacts of low band CA via switching on RRM requirements

Source: 	CATT

Agenda item:	7.8.3

Document for:	Discussion

Introduction

In RAN#106 meeting, a new WID [1] to support low band carrier aggregation via switching was approved and the objectives are as follows:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

According to the objectives, RRM group needs to identify possible impacts due to the support of low-low band CA via switching and defines corresponding core and performance requirements. In this contribution, we will present our views on these aspects.

Discussion

Potential impacts on RRM requirements

A brief description of the mechanism of the solution is provided in the WID as follows:

An enhancement of the 3GPP specification is needed to enable the following solution: - Device needs to support switching: when the SCell operation is triggered, UE needs to switch to the SCell, and during the operation period there is no simultaneous Tx/Rx between the PCell and the SCell - Device switches back to the PCell after the SCell operation is finished

According to this description and objectives, there will be two cases and UE will switch between these two cases based on a semi-static configuration configured by network. Furthermore, when UE works on one carrier, another carrier is disenabled. Currently, the CA combination CA_n5A-n29A is taken as an example for this WID.

- Case 1: Tx/Rx on FDD carrier 1 (n5) and no Rx on SDL carrier 2 (n29)

- Case 2: Rx on SDL carrier 2 (n29) and no Tx/Rx on FDD carrier 1 (n5)

Because carriers in both bands are co-located, synchronized and in a single TAG, therefore there are no DL/UL timing issues.

Proposal 1: CA_n5A-n29A has no impacts on DL/UL timing.

However, there are some impacts on RRM measurements. Considering n29 is a DL-only band, the cell configured in n5 shall be PCell when band n5 and n29 are aggregated. According to the above mechanism, UE cannot receive DL reference signals from PCell in n5 when UE works on the carrier in n29. In other words, UE cannot perform necessary RRM measurements on PCell, e.g., RLM. If the time duration that UE works on n29 carrier is long, then UE cannot meet the corresponding RRM requirements.

Observation 1: UE may not meet RRM requirements due to PCell being not measurable in a long time period.

In addition, since UE cannot perform UL transmissions when it works on n29 carrier, the HAQR feedback, UL data and measurement results that request UL resources to transmit need to be buffered and transmitted later when UE switches back to n5 PCell. From the physical layer perspective, though some UL transmissions do not need to be performed immediately, an upper-bound of delay time, k in RAN1 spec, exists and in general the max value for k is 8 slots. Similarly, when UE is configured to periodically perform RRM measurements, e.g., RLM, on PCell, the maximum time separation between two consecutive measurements is approximately hundreds of milliseconds. Hence, the time duration for UE working in n29 cannot be longer than a specific value. Therefore RAN4 need to discuss solutions from both network and UE sides to avoid the above situations.

Observation 2: When UE works in n29, UL transmissions can only be performed when UE switches back to n5.

Proposal 2: RAN4 to discuss solutions from both network and UE sides to mitigate the impacts when UE works in n29 for a long time period.

Another impact on RRM spec we identify is the interruption on serving cells due to the switching procedure. During the switching period, UE may not be able to continue receiving or transmitting data in some serving cells because of RF preparations and therefore an interruption should be allowed. Considering that specific values of the switching period depend on RF discussions, RRM group can discuss the interruption requirements due to switching after RF group has progress on the time duration of switching procedure. In addition, any RAN1 agreements that have impacts on RRM spec should also be considered.

Proposal 3: RAN4 to discuss interruption requirements due to switching procedure after RF has agreements on the time duration of switching.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: CA_n5A-n29A has no impacts on DL/UL timing.

Observation 1: UE may not meet RRM requirements due to PCell being not measurable in a long time period.

Observation 2: When UE works in n29, UL transmissions can only be performed when UE switches back to n5.

Proposal 2: RAN4 to discuss solutions from both network and UE sides to mitigate the impacts when UE works in n29 for a long time period.

Proposal 3: RAN4 to discuss interruption requirements due to switching procedure after RF has agreements on the time duration of switching.

References

[1] RP-243317, New WID on low band carrier aggregation via switching, RAN#106.