3GPP TSG RAN WG4 Meeting #114		R4-2501050

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Remaining issues on enabling HPUE CA with 1UL

Agenda Item:	4.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For the RAN task established in RAN#104, RAN4 was tasked to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL with a small online TU [1]. Most discussions took place offline during two quarters resulting in an agreement to enable such operation exceptionally [2] and the corresponding CRs [3][4]. However, the solution is indeed not complete where only MSD exemption and a generic description to allow such operation is captured in the specs [3][4]:

### 6.2A.0 General As an exception, HPUE applicability notes in 5.5A do not restrict power class indication for single configured UL CC with DL CA. UE can indicate power class for the single configured UL CC with DL CA as specified in Table 6.2.1-1 and if UE supports UL MIMO in this carrier, UE can indicate power class for the CA configuration as specified in Table 6.2D.1-1.

When UE supports higher power class than default power class for a CA configuration with a single UL CC with DL CA and applicability note for the supported power class is not present for this configuration in clause 5.5A. - if the corresponding higher power class MSD is not specified, reference sensitivity and exceptions for reference sensitivity of default power class shall be verified with output power limited to default power class - otherwise, the higher power class reference sensitivity and exceptions for reference sensitivity shall be verified with the power class of CA configuration supported by the UE.

And the following details are still required to be addressed [2]:

- Further discussion on UE signaling for DL CA with 1 UL - FFS whether and how to update the HPUE applicability notes in clause 5.5A, e.g. change “applicable” to “specified”. - FFS whether and how to make updates on configured transmission power.

In general, the open issues are related to the corresponding procedure and Pcmax.

In this contribution, we present our views on these open issues in order to conclude the solution.

# 2 Discussion

## 2.1 Procedure consideration

In short, the solution aims at allowing an exception under the current framework which enables a power boosting on top of the band combination power class at UE when operating in CA mode but with only 1 UL transmitted. By looking back into the history, several UE exceptional power boosting schemes have been specified in 5G NR:

- Power boosting for pi/2-BPSK

- Power boosting for uplink Tx switching for UL CA

- Power boosting for UL coverage enhancement

All above schemes involves both a separate UE capability indication and a corresponding enabling configuration by network:

- Power boosting for pi/2-BPSK: IE “powerBoosting-pi2BPSK” is used by UE to indicate the support of the power boosting, and IE “powerBoostPi2BPSK” is used by network to configure the power boosting.

- Power boosting for uplink Tx switching for UL CA: IE “uplinkTxSwitching-PowerBoosting” is used by UE to indicate the support of the power boosting, and IE “uplinkTxSwitchingPowerBoosting” is used by network to configure the power boosting.

- Power boosting for UL coverage enhancement: IEs “powerBoosting-pi2BPSK-QPSK-r18” and “powerBoosting-pi2BPSK-QPSK-Modified-r18” are used by UE to indicate the support of the power boosting, and IEs “powerBoostPi2BPSK-r18” is used by network to configure the power boosting.

Observation 1: For previously defined exceptions of power boosting, it is up to network to decide whether or not to enable the exception in fields for a specific UE.

In other words, network has the full control on whether or not to enable a UE’s power boosting in fields, and the exception of allowing HPUE CA with 1 UL should also follow such procedure, hence a corresponding enabling IE should be introduced.

Proposal 1: Similar to other exceptional power boosting schemes, network shall have full control on the HPUE CA with 1 UL hence a corresponding enabling IE shall be introduced.

For the UE capability signalling, it also can be observed that a separate UE capability IE is introduced for each power boosting scheme.

Observation 2: For all the previously defined exceptions of power boosting, a separate UE capability is used for a UE to indicate the support of each power boosting.

Moreover, if reusing an existing IE to indicate the support of HPUE CA with 1UL, e.g., ue-PowerClassPerBandPerBC-r17, a legacy UE may use this IE to indicate the higher power capability of the band, but without power boosting exception, while another UE may use the same IE to indicate the capability and with power boosting exception, then network cannot tell apart these two UE types.

Observation 3: Reusing the existing IE to indicate the support of the exceptional HPUE CA with 1UL may cause confusion at network.

Therefore, we propose to introduce a separate IE for UE to indicate such exception.

Proposal 2: RAN4 to introduce a separate UE capability IE to indicate its support of the exceptional HPUE CA with 1UL.

## 2.2 Updating Pcmax

Similarly, the exception of transmitting a higher power class then the band combination power class should be reflected in the configured transmit power calculation.

Proposal 3: Update Pcmax calculation to allow the exception.

The proposed changes are reflected in our companion CRs [5][6].

Proposal 4: RAN4 to approved the corresponding proposed changes in CRs [5][6].

# 3 Conclusion

In this contribution, we have the following observations and proposals for enabling HPUE CA with 1UL:

Observation 1: For previously defined exceptions of power boosting, it is up to network to decide whether or not to enable the exception in fields for a specific UE.

Observation 2: For all the previously defined exceptions of power boosting, a separate UE capability is used for a UE to indicate the support of each power boosting.

Observation 3: Reusing the existing IE to indicate the support of the exceptional HPUE CA with 1UL may cause confusion at network.

Proposal 1: Similar to other exceptional power boosting schemes, network shall have full control on the HPUE CA with 1 UL hence a corresponding enabling IE shall be introduced.

Proposal 2: RAN4 to introduce a separate UE capability IE to indicate its support of the exceptional HPUE CA with 1UL.

Proposal 3: Update Pcmax calculation to allow the exception.

Proposal 4: RAN4 to approved the corresponding proposed changes in CRs [5][6].

# Reference

[1] RP-241687, “Moderator's summary for offline discussion on CA Power Class Support”, Moderator (AT&T)

[2] R4-2420525, “WF on solution to enable HPUE max transmit power in DL CA”, Qualcomm

[3] R4-2420520, “CR to TS 38.101-1: DL CA HPUE”, Qualcomm et al.

[4] R4-2420524 “CR to TS 38.101-1: DL CA HPUE (Cat-A)”, Qualcomm et al.

[5] R4-2501051, “(Power_Limit_CA_DC-Core) CR to TS 38.101-1: DL CA HPUE with 1UL (Cat-F)”, CATT

[6] R4-2501052, “(Power_Limit_CA_DC-Core) CR to TS 38.101-1: DL CA HPUE with 1UL (Cat-A)”, CATT