3GPP TSG RAN WG4 Meeting #114		R4-2501063

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on BS in-band blocking for the frequency range 14.8GHz - 15.35GHz

Agenda Item:	7.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With the three reply LSs sent to ITU WP5D as planned, the study item on IMT parameters for NR for 4400 to 4800MHz, 7125 to 8400MHz and 14800 to 15350MHz was proposed to be extended with one quarter, aiming at concluding BS in-band blocking for 15GHz and aligning TR sections and correct where necessary [1], and got approved by RAN#106 [2].

Unlike 8GHz frequency range where band n104 can act a good reference point for related parameters including in-band blocking level [3][4], in-band blocking level for the 15GHz frequency range is still open as shown in the following issue from the approved way forward [5]:

Issue 4-2-X: In-band blocking Agreement For LS: Mention “To be specified" For TR: FFS for next meeting

In this contribution, we provide an example of estimating power level of the in-band interference based on the ACS interferer power level, and propose to leave it to the stage when specifying the band definition for this frequency range.

# 2 Discussion

Calculation of in-band interferer level

In order to calculate the in-band interferer level, we could firstly check the ACS interferer level. As replied in the third LS to WP5D, the typical channel bandwidth is 200MHz for this frequency range [6].

Starting from this parameter, one example is to presume 2 times of PRB number as that for 100MHz channel bandwith, i.e., 2*273 = 546 for SCS 30kHz, then the signal bandwidth could be computed as 196.6MHz, and BS Rx noise figure is set to 8dB, hence the noise floor of base station is -83.1dBm.

And assuming the allowed desensitization is 6dB, then the total base station noise would be -77.1dBm, which means the allowed base station interference is -78.3dBm. Since BS ACS is set to 30dB, it means that the interfering signal level for ACS PAI is -48.3dBm.

Usually the in-band interferer is stronger than that for ACS, and the difference between them is assumed to be ΔAI (dB), then the in-band interferer level PII can be calculated as the following equation:

PII = PAI  +  ΔAI

In one example, if the difference is 9dB, then the in-band interferer level is -48.3 + 9 = -39.3 dBm.

The following table illustrates the derivation described above.

| Parameters | Value | Unit |
| --- | --- | --- |
| Channel bandwidth | 200 | MHz |
| SCS | 30 | kHz |
| Number of PRBs | 546 | PCS |
| Signal bandwidth | 196.6 | MHz |
| BS Rx NF in dB | 8 | dB |
| Base station noise floor | -83.1 | dBm |
| Allowed desensitization | 6 | dB |
| Total base station noise | -77.1 | dBm |
| Allowed base station interference | -78.3 | dBm |
| Base station adjacent channel selectivity | 30.0 | dB |
| Interfering signal level (ACS) | -48.3 | dBm |
| Difference between in-band and ACS interferer (ΔAI) | 9.0 | dB |
| Interfering signal level (In-band) | -39.3 | dBm |

However, to specify an operating band for this frequency range in the future, the in-band interferer power level is one of the main tasks, therefore, this could be left to that stage, and for the time being, it is sufficient to capture the possible calculation method for in-band interferer levels into TR.

Proposal: For 14.8 – 15.35GHz frequency range, RAN4 to capture the possible calculation method for in-band interferer levels into TR.

# 3 Conclusion

In this contribution, we have the following proposal for BS in-band blocking for the frequency range 14.8G – 15.35GHz:

Proposal: For 14.8 – 15.35GHz frequency range, RAN4 to capture the possible calculation method for in-band interferer levels into TR.

# Reference

[1] RP-242943, “Status report for SI Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson

[2] RP-242944, “Revised SID: Study on IMT parameters for NR for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson

[3] R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT

[4] R4-2414281, “WF on 8GHz BS parameters”, Huawei

[5] R4-2420330, “WF on 15GHz BS parameters”, ZTE

[6] R4-2420385, “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson