3GPP TSG RAN WG4 Meeting #114		R4-2501072

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on REFSENS for LP-WUS

Agenda Item:	7.26.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For LP-WUS REFSENS, RAN4#113 has approved a WF as usual with some agreements and identified open issues [1].

In this contribution, we provide our analysis and views on some open issues related to REFSENS.

# 2 Discussion

FR1 OOK-based receiver

For the receiver of OOK-based LP-WUS signals, RAN4#113 has made the following agreements [1]:

Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information

Based on our simulation evaluation under the agreed setup, we obtain the following performance curves indicating MDR-SNR relationship of an OOK-based receiver, where 1% MDR corresponds to -3.3 dB SNR, as illustrated in Fig. 1. For IM, we believe the starting point 2.5dB is a reasonable value for deriving REFSENS, and for LR noise figure, some relaxation, e.g., 6dB, should be allowed compared with MR (NF = 9), hence 15dB could be used for REFSENS derivation.

Fig. 1, Simulated MDR vs. SNR of OOK-based receiver

With such considerations above, Table-1 below summarizes IM, NF and SNR for OOK-based receiver, where the total IM+NF+SNR is 14.2dB.

Table – 1 IM, NF and SNR values for OOK-based receiver

| FR1 OOK-based receiver | CATT |
| --- | --- |
| IM(dB) | 2.5 |
| NF(dB) (assume MR=9dB) | 15 |
| SNR(dB) | -3.3 |
| IM+NF+SNR (dB) | 14.2 |

Proposal 1: For OOK-based receiver, RAN4 to define its REFSENS by assuming IM as 2.5dB, NF as 15dB and SNR as -3.3dB.

FR1 OFDM-based receiver

For the receiver of OFDM-based signals, RAN4#113 recommended to leave it completely to the next meeting [1]:

Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR Agreements: - Further discuss next meeting

For the implementation margin, 2.5dB could be used as a starting point as well. And for noise figure, a tightened value compared with OOK-based receiver, e.g., 3dB tightened as 12dB, is a feasible value for deriving REFSENS. And for SNR, we observed that OFDM-based receiver outperforms OOK-based receiver by around 4dB, i.e., -7dB. However, SNR may need to be validated with more inputs from RAN1 when the signal design is completed. Table-2 below summarizes preliminary IM, NF and SNR for OFDM-based receiver, where the total IM+NF+SNR is 7.5dB.

Table – 2 Preliminary IM, NF and SNR values for OFDM-based receiver

| FR1 OFDM-based receiver | CATT |
| --- | --- |
| IM(dB) | 2.5 |
| NF(dB) (assume MR=9dB) | 12 |
| SNR(dB) | -7 |
| IM+NF+SNR (dB) | 7.5 |

Proposal 2: For OFDM-based receiver, RAN4 to define its REFSENS by preliminarily assuming IM as 2.5dB, NF as 15dB and SNR as -3.3dB, and SNR is to be validated after RAN1 completes LP-WUS signal design.

1 or 2 sets of REFSENS requirements

Regarding the decision to introduce different REFSENS requirements for the two types of receivers, RAN4 adopted a strategy where it first discusses REFSENS separately for each receiver type, and then decides whether to define a single set or two sets of REFSENS based on the outcomes of the discussion [1]:

Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values.

It is not surprising that a tightened noise figure and a better SNR of OFDM-based receiver are expected compared with that for OOK-based receiver, which makes a big difference between the final REFSENS values, hence, two sets of REFSENS may be introduced after validating SNR for OFDM-based receivers.

Proposal 3: RAN4 to introduce two sets of REFSENS for OOK-based and OFDM-based receivers respectively after the SNR has been validated for the OFDM-based receiver.

Another issue is how to address cases with and without overlaid OFDM sequences when specifying REFSENS. According to the WID, both scenarios are targeted in this work item [2], so both should be taken into account. However, for REFSENS, only the worst-case scenario between the cases with and without overlaid OFDM sequences needs to be considered. This can be determined once RAN1 finalizes the overlaid OFDM sequence design.

Proposal 4: RAN4 to consider only the worst-case scenario between the cases with and without overlaid OFDM sequences when specifying REFSENS.

# 3 Conclusion

In this contribution, we have the following proposals for REFSENS for LP-WUS:

Proposal 1: For OOK-based receiver, RAN4 to define its REFSENS by assuming IM as 2.5dB, NF as 15dB and SNR as -3.3dB.

Proposal 2: For OFDM-based receiver, RAN4 to define its REFSENS by preliminarily assuming IM as 2.5dB, NF as 15dB and SNR as -3.3dB, and SNR is to be validated after RAN1 completes LP-WUS signal design.

Proposal 3: RAN4 to introduce two sets of REFSENS for OOK-based and OFDM-based receivers respectively after the SNR has been validated for the OFDM-based receiver.

Proposal 4: RAN4 to consider only the worst-case scenario between the cases with and without overlaid OFDM sequences when specifying REFSENS.

# Reference

[1] R4-2420528, “WF on UE RF requirements for LP-WUS”, vivo

[2] RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DoCoMo