3GPP TSG RAN WG4 Meeting #114		R4-2501076

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on power domain enhancements for single carrier

Agenda Item:	7.1.1.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For power domain enhancements for single carrier, RAN4#113 has made some progress and identified the following open issues [1]:

Issue 1-1-2: Approaches of calculating the new inner RB region - WF - FFS description of deriving the RB allocation region inside the UE CBW which could use inner RB MPR with information of the extended UE CBW - Candidate wordings, other options are not precluded - The intersection of the Inner region relative to the extended UE CBW and the Outer region relative to the configured UE CBW is where the inner MPR can be expected from UE - The intersection of the Inner MPR region of the extended UE CBW with the original MPR region based on the UE CBW - For enhanced MPR requirements, at least in the case where BW is extended by 0.5 * UE CBW on one or both sides of UE CBW, an RB allocation (LCRB) within the UE CBW is considered as an extended inner RB allocation if an RB allocation-extCBW (of extended CBW) fully covering the same frequency location is also determined as an inner RB allocation (according to existing 38.101-1 MPR formula with corresponding NRB of the extended CBW applied). - Further improve the wording (better with illustrated figures) to derive a generic solution - How UE get the extended RB number as well as the RB shift is one possible way, other methods are not precluded - FFS how to reflect the new inner region in the spec Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW - WF - Extension on each side of UE CBW could be different - FFS on extended RB numbers/channel bandwidth, which are up to X on each side of UE CBW, depends on the adjacent spectrum by operator in all possible scenarios - FFS whether extended RB number/channel bandwidth should be limited to max 50MHz at each side, which means the extended CBW is relevant to the configured UE CBW - Fixed extended RB number/channel bandwidth to X is not precluded. - Extension is not used for the specific UE UL transmission but can be used by other UEs UL transmissions. It is generally understood that going beyond X on each side does not further reduce the MPR. - FFS on X value #### Issue 1-2-1: Where to use extended IBE in the larger BS CBW - WF - IBE should be used between edges of UE CBW and extended UE CBW - FFS the application of IBE in guard band of UE CBW and/or extended UE CBW Issue 1-2-2: How to derive the extended IBE - WF - FFS the method to derive the extended IBE, which should cover both cases for partial RB allocation and full RB allocation in the UE CBW #### Issue 1-2-3: Boundary to apply ACLR and SEM - WF - ACLR and SEM should be applicable from the edge of extended UE CBW. - FFS the following alternatives - Alt 1: The OOBE requirements are based on UE CBW - Alt 2: The OOBE requirements are based on extended UE CBW

In this contribution, we provide our views on these open issues.

# 2 Discussion

For the applicable regions for MPR enhancement after extending UE channel bandwidth, Fig. 1(a) and Fig. 1(b) illustrate the regions (including inner, outer and edge regions) based on the original UE CBW and extended UE CBW respectively. And Fig. 1(c) shows regions from both CBWs under the same scale where the original UE CBW regions are of a certain RB shift. Therefore, we propose to describe the applicable regions for MPR enhancement as “The enhanced MPR applies to the intersection of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW, with both the extended UE channel bandwidth and the number of shifted RBs being signaled by the network.”.

- Regions based on original UE CBW

- Regions based on extended UE CBW

- Regions from shifted (a) and (b)

Fig. 1, Regions of extended UE CBW Vs Shifted regions of original UE CBW

Proposal 1: RAN4 to describe the applicable regions for MPR enhancement as:

The enhanced MPR applies to the intersection of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW, with both the extended UE channel bandwidth and the number of shifted RBs being signaled by the network.

For the extension itself, as we proposed in our previous paper [2], there are several restrictions:

- The extended UE channel bandwidth should not exceed BS channel bandwidth

- The minimum UE channel bandwidth to allow the extension is 10MHz

- Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth

Proposal 2: Extension should fulfil all the following conditions:

- The extended UE channel bandwidth should not exceed BS channel bandwidth

- The minimum UE channel bandwidth to allow the extension is 10MHz

- Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth

Regarding the applicability of IBE for guard-bands of original UE channel bandwidth and extended UE channel bandwidth, it should be specified in a consistent way. Currently, no IBE requirement is applied in a guard-band of UE channel bandwidth, therefore, since the channel bandwidth is virtually extended, then the guard-band of the original UE CBW should fulfil IBE requirements, but not for the guard-band of the extended UE CBW.

Proposal 3: IBE requirement is applied to the guard band of the original UE CBW, but not to the extended UE CBW.

For IBE between the edge of the UE CBW and the edge of the extended UE CBW, it could be constant and should be an extension of the IBE for the signal within the UE CBW with the same equation below:

Where:

- NRB should be corresponding to the original UE CBW

- should be calculated according to the actual RB allocation

- ∆RB should be the offset value between the edge of actual UE transmission bandwidth configuration and actual RB allocation.

Proposal 4: IBE between the edge of the UE CBW and the edge of the extended UE CBW should be constant and an extension of the IBE for the signal within the UE CBW.

Similar consideration would apply to OOBE, which means that the OOBE requirements should be based on UE CBW.

Proposal 5: OOBE requirements should be based on UE CBW.

# 3 Conclusion

In this contribution, we have the following proposals for power domain enhancements for single carrier:

Proposal 1: RAN4 to describe the applicable regions for MPR enhancement as:

The enhanced MPR applies to the intersection of the Inner region relative to the extended UE CBW and the shifted outer region relative to the original configured UE CBW, with both the extended UE channel bandwidth and the number of shifted RBs being signaled by the network.

Proposal 2: Extension should fulfil all the following conditions:

- The extended UE channel bandwidth should not exceed BS channel bandwidth

- The minimum UE channel bandwidth to allow the extension is 10MHz

- Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth

Proposal 3: IBE requirement is applied to the guard band of the original UE CBW, but not to the extended UE CBW.

Proposal 4: IBE between the edge of the UE CBW and the edge of the extended UE CBW should be constant and an extension of the IBE for the signal within the UE CBW.

Proposal 5: OOBE requirements should be based on UE CBW.

# Reference

[1] R4-2420527, “WF on Power domain enhancement”, CATT

[2] R4-2417882, “On power domain enhancements for single carrier”, CATT