# 3GPP TSG-RAN WG4 Meeting #114                                 R4-2501085

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.27.2.3

Source: 	CMCC

Title: 			 Discussion on conditional intra-CU LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #105 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meetin, there was initial discussion on measurements related enhancements for LTM, and a WF was agreed [3]. This contribution provides discussion on conditional intra-CU LTM.

- Discussion

Issue 1-1-1: The applicable scenarios for conditional LTM RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.. < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

According to the discussion in last meeting, whether CLTM requirements apply to candidate cells on deactivated SCC is related with the discussion on Rel-18 LTM. In Rel-18 LTM, LTM candidate cells on deactivated SCC is supported in RAN2, but RAN4 related requirements, e.g. interruption, are not specified due to limited timeline. It is necessary to define requirements to fix this mismatch between RAN2 and RAN4. And after the LTM requirements are applied to candidate cells on deactivated SCC, the Rel-19 CLTM requirements can also be defined.

Proposal 1: it is proposed to define measurement requirements for L1 measurement on deactivated SCC.

Issue 1-2-1: Early UL Synchronization RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.. For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

According to RAN2 agreements, UE based TA measurement mechanism is supported for conditional intra-CU LTM. Autonomous TA adjustment for LTM is introduced in Rel-18 LTM. However, no dedicated requirements for UE based TA adjustment are specified in RAN4 in R18 due to limited timeline. In order to guarantee the performance, it is necessary to define related RAN4 requirements, e.g.timing requirements, for UE based TA adjustment.

Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment.

Issue 1-3-2: The starting point of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. - Proposal 2: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. - Proposal 3: Similar as event-triggered L1 reporting. The CondEvent is met over the air. - Proposal 4: Wait for more RAN2 agreements.

In high level, this issue is related with how to define cell switch delay requirements for conditional LTM and subsequent conditional LTM, i.e. whether to define seperate delay requirements or a unified requirements to cover both cases, since the main difference between the delay requirements of the two cases is that the starting point is different. For legacy Conditional PSCell Addition/Change (CPA/C), Subsequent Conditional PSCell Addition/Change (SCPA/C), the requirements are defined seperately, since CPA/C and SCPA/C is introduced in different release. While for now, conditional LTM and subsequent conditional LTM are considered in the same release. It is possible to define a unified requirements to cover both cases. According to the discussion in previous meetings, it seems that most companies prefer to define seperate requirements, we are also fine with this.

Our following discussion is based on the assumption that cell switch delay requirements for conditional LTM and subsequent conditional LTM are specified seperately.

Rel-18 LTM cell switch delay can be used as baseline to discuss delay for conditional LTM. Rel-18 LTM cell switch delay DLTM is specified as the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.  DLTM = Tcmd + TLTM-interrupt. Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. TLTM-interrupt is interruption time, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms. In detail, TLTM-RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. TRS-proc is the time for SSB processing. TLTM-IU is the interruption uncertainty during LTM cell switch which is related with whether RACH-based LTM cell switch or RACH-less LTM cell switch is in use.

For conditional LTM, there is no cell switch command, which means that it is not necessary to include Tcmd which equal to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement. Similar like CHO, CPAC, or SCPAC, UE will receive a RRC message implying conditional LTM. The starting point of conditional LTM is the time when UE receives RRC command triggering conditional LTM, similar as CHO, CPAC. The corresponding delay requirements include the component of TRRC_delay, which is the RRC processing delay. The ending point is the time the UE transmits the first UL message on the target cell.

- Proposal 3: for conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM.

Issue 1-3-4: The components of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. - Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. - Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. - Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements. - Proposal 7: Wait for RAN2 progress.

In detail, the cell switch delay for conditionl LTM is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. TRRC is the RRC procedure delay. TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure is measurements time. TLTM-interrupt is the interruption time as specified in Rel-18. TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- Proposal 4: for conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TRRC_delay is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

Issue 1-4-2: The starting point of Subsequent conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. - Proposal 2: Wait for RAN2 further progress.

For subsequent conditional LTM, if we follow the approach for subsequent CPAC, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch. The ending point is the time the UE transmits the first UL message on the target cell.

- Proposal 5: for subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch.

Issue 1-4-4: The components of subsequent conditional LTM delay < Way forward > FFS the following proposal: - Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. - TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. - Tmeasure is measurements time. - TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms. - TLTM-interrupt is the interruption time as specified in Rel-18.

The cell switch delay for subsequent conditionl LTM is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt. TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM. Tmeasure is measurements time. TLTM-interrupt is the interruption time as specified in Rel-18. TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- Proposal 6: for subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Conclusion

This contribution provides discussion on conditional LTM. The proposals are:

Proposal 1: it is proposed to define measurement requirements for L1 measurement on deactivated SCC.

Proposal 2: since RAN2 is agreed that UE based TA measurement mechanism is supported for conditional intra-CU LTM, it is proposed to define related RAN4 requirements for UE based TA adjustment.

- Proposal 3: for conditional LTM, the starting point is the time when UE receives RRC command triggering conditional LTM.

- Proposal 4: for conditional LTM, cell switch delay is proposed as DLTM_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TRRC_delay is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 5: for subsequent conditional LTM, the starting point is the time when UE transmits RRCReconfigurationcomplete message for the previous cell switch.

- Proposal 6: for subsequent conditional LTM, cell switch delay is proposed as DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurement time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

- R4-2414060, WF on R19 Further NR mobility enhancement, Apple

- RP-233970, Revised WID on Further NR Mobility Enhancements

- R2-2404102, Report of 3GPP TSG RAN WG2 meeting #125bis

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4