# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501088

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.23.3.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides further discussion on UE-to-UE CLI handling.

- Discussion

#### Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

For measurement methods for CLI handling, in last meeting, it was agreed that RAN4 shall specify L1 CLI measurement requirements at least for Method #1 and Method #2, additional methods can be considered based on further RAN1 agreements.

In last RAN1 meeting, RAN1 further agreed to support Method #3: UE measures RSSI within UL subband. Since RAN1 has supported Method #3, it is necessary for RAN4 to specify L1 CLI measurement requirements for Method #3.

RAN1 #119 Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. RAN1 #118bis Agreements For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband). RAN1 #118 Agreements For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting

Proposal 1: it is proposed that RAN4 specify requirements for Method#3: UE measures RSSI within UL subband.

Issue 1-1-4: Measurement resources - Proposals - Proposal 1 (CMCC, ZTE, MTK, Samsung): - RAN4 firstly define requirements based on wideband reporting, and further discuss sub-band reporting pending on RAN1 progress. - Proposal 2 (Nokia): - RAN4 should define new requirements for the sub-band CLI-RSSI reporting supported by RAN1. Sub-band CLI-RSSI reporting is applicable for both measurement resource type #1 and type #2. - Proposal 3 (CATT, MTK): - RAN4 to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are applicable for L1 CLI-RSSI measurements, when one CLI-RSSI resource occupies two discontinuous DL subbands (measurement resource type#2) - Proposal 4 (vivo): - For the wide-band reporting when measurement results from different discontinuous DL subbands need be combined (measurement resource type#2), how to combine the measurement results could be up to UE implementation. - Proposal 5 (LGE): - RAN4 need to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. - Proposal 6 (HW): - RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource or reporting configuration. - Proposal 7 (vivo): - RAN4 needs to discuss the applicability of requirements when the number of RBs used for measurement is less than 48. - Proposal 8 (MTK): - For measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband, existing requirements are applicable provided that the minimum number of resources are satisfied. - Proposal 9 (Samsung): For measurement resource type, RAN4 shall cover semi-static, aperiodic and periodic resource types following the configuration in CSI-ResourceConfig

According to RAN1 agreements, as duplicated as following, the resourceType for L1-CLI-RSRI and L1-SRS-RSRP measurement can be set to periodic, semi-persistent or aperiodic. RAN4 need to define requirements for to cover these resource types.

Proposal 2: it is proposed that RAN4 define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for resource types of periodic, semi-persistent or aperiodic.

Agreement in RAN1#118 meeting For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - FFS: Subband CLI-RSSI reporting - Agreement in RAN1#118bis meeting For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - Note: No new usage needs to be defined for SRS resource sets Conclusion in RAN1#119 meeting There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19. For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

As agreed in RAN1, there are two measurement resource type: CLI-RSSI measurement resource is configured within a DL subband or the CLI-RSSI measurement resource is configured across two DL subbands. As for reporting, RAN1 agreed that wideband reporting is supported, and no consensus on sub-band CLI-RSSI reporting. From RAN4 requirements point of view, we can firstly focus on wideband. Whether and how to consider sub-band reporting can be further discussed pending on RAN1 progress.

Proposal 3: it is proposed that RAN4 firstly define requirements for wideband reporting.

For SRS SNR, according to the agreed simulation assumption [5], 1, 2, 3 dB are simulated. According to the simulation results summary [6], it can be seen that the performance difference among different SRS SNR is minimum, which is within 0.5dB. With higher side condition, the applied scenario is limited. To guarantee the application of this feature, it is proposed that the side condition is 1dB.

Proposal 4: for L1 CLI measurement, it is proposed that the side condition (SRS Es/Iot) is 1dB.

In RAN4 #112bis meeting, RAN4 sent LS to RAN1 to ask RAN1 to provide clarification on the target scenarios for UE-to-UE L1 CLI measurement [3]. The RAN1 reply LS was sent to RAN4 in last meeting [4], the details are duplicated as following.

RAN1 reply LS on target scenarios for UE-to-UE L1 CLI measurement (R1-2410890) - 1 Overall description When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement. - 2 Actions To RAN4 ACTION: RAN1 respectfully asks RAN4 to take the above reply into account in their future work.

According to RAN1 reply LS, L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both intra-cell scenario and inter-cell scenario. We notice that RAN1 reply LS also mentioned that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, considering no specification impact for SRS configuration information exchange between gNBs. However, similar situation exsits for Rel-16 L3 CLI measurement that even though how SRS configuration information are exchanged between gNBs are not specified in Rel-16, but RAN4 requirements for Rel-16 CLI measurement are specified to cover inter-cell scenario with the assumption that SRS configuration information exchange between gNBs is left to network implementation. Similar approach can be used for RAN4 requirements definition for Rel-19 L1 CLI measurement. What’s more, From deployment point of view, only consider intra-cell is too limited. Based on above consideration, it is proposed to define RAN4 requirements for L1 CLI for both intra-cell scenario and inter-cell scenario.

Proposal 5: for timing offset, it is proposed to define L1 UE-to-UE CLI measurement requirements with cell phase error, same as Rel-16 L3 CLI measurement.

According to the agreed simulation assumption [5], both case 1 and case 2 are simulated. For Case 1, the residual timing error is 4.67us for FR1 and 3.67us for FR2, same as the Rel-16. Case 2 remove the cell phase error, in detail, the residual timing error is 1.67us for FR1 and 0.67us for FR2. We summarize simulation results provided by companies in last meeting to compare the measurement between case 1 and case 2, the detail can be found as following Table 1. According to Table 1 of simulation results summary, it can be seen that for the smaller SCS, e.g. SCS of 15KHz and 30KHz, the performance difference between case 1 and case 2 is within 1dB. While with SCS is increasing, the performance difference between 2 cases are increasing. For SCS of 120KHz, the difference could be 5dB.

Table 1 performance comparison between case 1 and case 2 based on companies’ simulation results,

(SNR=1dB, SRS symbol number = 1)

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | CATT (R4-2417696) |  |  |  | HW (R4-2419148) |  |  |  | Samsung (R4-2419620) |  |  |  | Ericsson (R4-2419442) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.98 | 0.41 | -0.61 | 0.71 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -0.77 | 0.2 | -0.41 | 0.54 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -0.71 | 0.11 | -0.35 | 0.48 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -0.83 | 0.13 | -0.5 | 0.48 | -0.98 | 0.07 | -0.54 | 0.42 | 1.2 | 1.2 | 0.7 |  | -0.14 | 0.74 | 0.03 | 0.76 |
|  |  |  | 2 | -0.7 | -0.01 | -0.36 | 0.33 |  |  |  |  | 1.1 | 1.1 | 0.6 |  |  |  |  |  |
|  |  |  | 3 | -0.63 | -0.06 | -0.29 | 0.26 | -0.84 | -0.23 | -0.39 | 0.15 | 1 | 1 | 0.5 |  |  |  |  |  |
|  | 30 | 24 | 1 | -1.58 | -0.07 | -0.81 | 0.57 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -1.38 | -0.26 | -0.61 | 0.34 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.27 | -0.37 | -0.55 | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -1.46 | -0.35 | -0.67 | 0.3 | -1.66 | -0.5 | -0.78 | 0.19 | 2.1 | 2.1 | 1 |  | -0.49 | 0.67 | -0.06 | 0.74 |
|  |  |  | 2 | -1.29 | -0.52 | -0.54 | 0.16 |  |  |  |  | 1.9 | 1.9 | 0.8 |  |  |  |  |  |
|  |  |  | 3 | -1.23 | -0.59 | -0.48 | 0.1 | -1.54 | -0.84 | -0.64 | -0.08 | 1.7 | 1.7 | 0.8 |  |  |  |  |  |
|  | 60 | 24 | 1 | -3.12 | -1.27 | -1.28 | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.82 | -1.53 | -1.03 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -2.66 | -1.62 | -0.93 | -0.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.9 | -1.58 | -1.09 | -0.09 | -3.28 | -1.76 | -1.25 | -0.16 | 3.9 | 3.9 | 1.5 |  | -1.32 | 0.58 | -0.28 | 0.71 |
|  |  |  | 2 | -2.68 | -1.77 | -0.96 | -0.23 |  |  |  |  | 3.6 | 3.6 | 1.4 |  |  |  |  |  |
|  |  |  | 3 | -2.61 | -1.85 | -0.89 | -0.3 | -3.05 | -2.18 | -1.13 | -0.5 | 3.4 | 3.4 | 1.3 |  |  |  |  |  |
| FR2 | 60 | 24 | 1 | -2.36 | -0.66 | -0.71 | 0.63 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.07 | -0.93 | -0.5 | 0.43 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.98 | -1.04 | -0.44 | 0.36 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.21 | -1.04 | -0.6 | 0.38 | -2.59 | -1.22 | -0.7 | 0.3 | 3.1 | 3.1 | 0.9 |  | -0.93 | 0.62 | -0.02 | 0.74 |
|  |  |  | 2 | -2.04 | -1.2 | -0.47 | 0.23 |  |  |  |  | 2.7 | 2.7 | 0.7 |  |  |  |  |  |
|  |  |  | 3 | -1.97 | -1.27 | -0.39 | 0.18 | -2.33 | -1.56 | -0.56 | 0.02 | 2.6 | 2.6 | 0.6 |  |  |  |  |  |
|  | 120 | 24 | 1 | -5.58 | -3.12 | -1.07 | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -5.22 | -3.48 | -0.86 | 0.16 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -5.06 | -3.62 | -0.76 | 0.07 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -5.37 | -3.56 | -0.93 | 0.09 | -6.01 | -3.6 | -1.05 | -0.05 | 6.8 | 6.8 | 1.3 |  | -2.71 | 0.52 | -0.2 | 0.73 |
|  |  |  | 2 | -5.07 | -3.82 | -0.77 | -0.07 |  |  |  |  | 6.2 | 6.2 | 1.1 |  |  |  |  |  |
|  |  |  | 3 | -4.96 | -3.92 | -0.73 | -0.15 | -5.56 | -4.18 | -0.92 | -0.3 | 6 | 6 | 1 |  |  |  |  |  |

As for how to define L1-SRS-RSRP measurement requirements, one possible way is to define requirements for case 1 (inter-cell scenario) and case 2 (intra-cell scenario) seperately. UE can determine which set of requirements is in use based on the received timing difference. Similar situation exits for CSI-SINR measurement accuracy. Following is the duplication of CSI-SINR inter frequency absolute accuracy. As highlighted in blue, for different timing offset between the reference measurement timing and the target CSI-RS, the upper bound of applied side condition is different, which means for CSI-SINR measurement accuracy, UE apply different requirements based on different received timing difference. Similar approach can be used to define L1-SRS-RSRP measurement accuracy requirements.

Observation 1: for Rel-16 CSI-SINR, two sets of measurement accuracy requirements are defined, And UE determine which set of requirements is in use based on the received timing difference.

Table 10.1.14.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR1 (TS38.133)

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

The other way to define L1-SRS-RSRP measurement requirements is to define one set requirements to cover both inter-cell scenario and intra-cell scenario. Following two tables are the existing L3 SRS-RSRP measurement accuracy, which are defined with 3 samples and side condition is SNR larger than 1.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 10.1.22.1.1-2: SRS-RSRP absolute accuracy in FR2

| Accuracy |  |  |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  | Extreme condition |  | SRS Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  |  |  | Minimum Io |  |  | Maximum Io |
| dB |  |  |  | dB | dBm / SCSSRS Note 2 |  | dBm/BWChannel | dBm/BWChannel |
| SCSSRS (kHz) |  | SCSSRS (kHz) |  |  | SCSSRS = | SCSSRS = |  |  |
| 60 | 120 | 60 | 120 |  | 60kHz | 120kHz |  |  |
| 6 | 8.5 | 9 | 11.5 | ≥1 | Same value as SRS_RP in Table B.2.7-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 9 | 11.5 | 11 | 13.5 | ≥1 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |  |  |

Compare existing L3-SRS-RSRP measurement accuracy requirements and Table 1 of simulation results summary for L1-SRS-RSRP, it can be seen that, the L1-SRS-RSRP measurement performance with 1 sample can be covered by existing L3-SRS-RSRP measurement requirements. For example, according to above Table 10.1.22.1.1-2, the L3-SRS-RSRP absolute accuracy for SCS of 120KHz with 48 PRBs is +-9dB, while according to Table 1 of L1-SRS-RSRP simulation results summary, for SCS of 120KHz, SRS BW of 48PRB, and 1 sample, the worst measurement performance of L1-SRS-RSRP among companies is 6.8dB, even consider RF margin additionally, existing L3-SRS-RSRP measurement accuracy can also be applied to  L1-SRS-RSRP. In summary, for the case with cell phase error, under the condition of same number of SRS BW and same side condition, L1-SRS-RSRP measurement accuracy with 1 sample can be covered by L3-SRS-RSRP measurement accuracy with 3 samples.

Observation 2: for L1-SRS-RSRP, the measurement performance with 1 sample and case 1 (with cell phase error) can be covered by existing L3-SRS-RSRP measurement accuracy requirements with 3 samples.

Based on above consideration, it is proposed to define one set requirements to cover both intra-cell scenario and inter-cell scenario, and exsiting L3-SRS-RSRP accuracy requirements can be applied to L1-SRS-RSRP.

Proposal 6: It is proposed to define one set requirements to cover both the case with cell phase error (intra-cell scenario) and the case without cell phase error (inter-cell scenario). And exsiting L3-SRS-RSRP accuracy requirements can be applied to L1-SRS-RSRP.

#### Issue 1-1-6: Measurement period - Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing.

For measurement period, according to existing L1-RSRP measurement, both 1 sample and 3 sample are considered pending on the higher layer parameter timeRestrictionForChannelMeasurement. The sample number is 1 if timeRestrictionForChannelMeasurement is configured, and the sample number is 3 otherwise. Same approach can be used for L1-SRS-RSRP measurement.

Proposal 7: for L1-SRS-RSRP, measurement period is specified based on 1 and 3 samples pending on NW configuration, e.g.  timeRestrictionForChannelMeasurement.

Issue 1-1-7: Measurement reporting - Proposals - Proposal 1 (CATT, Nokia, CTC, vivo, HW, ZTE, E///, MTK): - At least L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting. - Proposal 2 (LGE, CMCC, vivo, HW, ZTE, MTK, Samsung): - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

As duplicated as following, RAN1 discussion mainly focus on aperiodic reporting. RAN4 discussion can focus on aperiodic reporting, and other reporting modes can be further discussed pending on RAN1 propgress.

Agreements in RAN1#118bis Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - Note: No new usage needs to be defined for SRS resource sets For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter. Agreements in RAN1#119 For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Proposal 8: for L1 CLI measurement, it is proposed that RAN4 firstly focus on aperiodic reporting, other reporting modes can be further discussed pending on RAN1 propgress.

As for the measurement accuracy, according to the simulation results summary [6], it can be seen that the performance difference between 1 sample and 3 sample is minimum, which is within 0.5dB. And further considering the RF carlibration margin, the difference is negligible. Based on above analysis, it is proposed that same accuracy requirements are applied to the case with 1 sample and the case with 3 sample.

Observation 3: for L1-SRS-RSRP, the measurement performance difference between 1 sample and 3 sample is minimum, which is within 0.5dB.

Proposal 9: for L1-SRS-RSRP, it is proposed that same accuracy requirements are applied for both the case with 1 sample and the case with 3 sample.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: it is proposed that RAN4 specify requirements for Method#3: UE measures RSSI within UL subband.

Proposal 2: it is proposed that RAN4 define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for resource types of periodic, semi-persistent or aperiodic.

Proposal 3: it is proposed that RAN4 firstly define requirements for wideband reporting.

Proposal 4: for L1 CLI measurement, it is proposed that the side condition (SRS Es/Iot) is 1dB.

Proposal 5: for timing offset, it is proposed to define L1 UE-to-UE CLI measurement requirements with cell phase error, same as Rel-16 L3 CLI measurement.

Observation 1: for Rel-16 CSI-SINR, two sets of measurement accuracy requirements are defined, And UE determine which set of requirements is in use based on the received timing difference.

Observation 2: for L1-SRS-RSRP, the measurement performance with 1 sample and case 1 (with cell phase error) can be covered by existing L3-SRS-RSRP measurement accuracy requirements with 3 samples.

Proposal 6: It is proposed to define one set requirements to cover both the case with cell phase error (intra-cell scenario) and the case without cell phase error (inter-cell scenario). And exsiting L3-SRS-RSRP accuracy requirements can be applied to L1-SRS-RSRP.

Proposal 7: for L1-SRS-RSRP, measurement period is specified based on 1 and 3 samples pending on NW configuration, e.g.  timeRestrictionForChannelMeasurement.

Proposal 8: for L1 CLI measurement, it is proposed that RAN4 firstly focus on aperiodic reporting, other reporting modes can be further discussed pending on RAN1 propgress.

Observation 3: for L1-SRS-RSRP, the measurement performance difference between 1 sample and 3 sample is minimum, which is within 0.5dB.

Proposal 9: for L1-SRS-RSRP, it is proposed that same accuracy requirements are applied for both the case with 1 sample and the case with 3 sample.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2420166, Summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson