# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501092

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.3

Source: 	CMCC

Title: 			 Discussion on RRM requirements for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM requirements for beam management.

- Discussion

Issue 3-1: Relative RSRP Accuracy - Proposals - Option 1: The definition of relative RSRP accuracy shall be: The relative RSRP accuracy for AI/ML-based beam prediction = the ground truth of L1-RSRP of the predicted beam index i - the ground truth of the L1-RSRP of the Top-1 ideal beam index j - Option 2: The definition of relative RSRP accuracy shall be: predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams. - Option 3: The definition of relative RSRP accuracy shall be: the RSRP difference between the predicted L1-RSRP of different beams. - Option 4: others

In general, it is preferred to follow the definition of legacy L1-RSRP measurement accuracy. According to legacy L1-RSRP measurement, both absolute accuracy and relative accuracy are specified. The absolute accuracy is the difference between the measured L1-RSRP and the ideal L1-RSRP. Relative accuracy is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell on which UE performs L1-RSRP measurements.

In RAN4#112bis meeting, it was agreed that the absolute RSRP accuracy for AI/ML based beam prediction is the difference between predicted L1-RSRP of beam index i and groundtruth of L1-RSRP of beam index i, which is similar as the definition of legacy L1-RSRP measurement accuracy. As for the relative accuracy, some companies propose not necessary to have relative accuracy requirements in addition to absolute accuracy requirements for AI/ML, while some companies mentioned that relative accuracy is related to monitoring and RAN1 has similar discussion. We do not have strong view. Our main consideration is that if it is agreed to define relative RSRP accuracy for AI/ML, it is proposed to follow the legacy way. In detail, the relative RSRP accuracy is the difference between the predicted L1-RSRP of beam index i and the predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value.

Proposal 1: the relative RSRP accuracy is proposed as:

- Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value.

Agreements in RAN4#112bis RSRP Absolute accuracy: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

As for the part with square brackets in the agreement, we do not think it is necessary. In our view, the accuracy requirements should apply to all predicted beams, no need to limit to a subset of predicted beams, which are selected predefined side conditions. There are two reasons to suggest to apply the accuracy requirements to all predicted beams. On one hand, all the predicted/reported beams are benificial for network, for example, can be used for load balancing. On the other hand, the final reported beam index and/or related L1-RSRP are based on ranking of all the predicted beams. If the accuracy of some predicted beams are not guaranteed, it will have impact on the final decision of the reported beams.

Proposal 2: it is proposed that the RSRP absolute accuracy requirements apply to all predicted beams, not limit to the beam index in top-K beams.

Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

The key issue is whether the UE knows with which Rx beam to receive the predicted Tx beam or not. Our initial understading is that UE RX beam is known. For the the beam in set B, it is known since UE has measured it. For the beam not in setB, it can also be known since the UE Rx beam gain has to be taken into account when making the prediction of gNB TX beam. However, according to the discussion, it is related with UE implementation. Some companies mentioned that UE will take the Rx spherical coverage into account but the UE Rx beam index is not input into the prediction model. In this case, UE RX beam is unknown. One proposal is to assume the Rx beam is unknown since RAN4 define the minimum requirements. However, in our view, it is not a good way to preculde the advanced model. Taking current situation into account, introduce UE capability is a possible way to move forward.

Proposal 3: For the beam in set B, it is known. For the beam not in set B, it is prefered to assume UE Rx beam is known, but we are also OK to have UE capability to move forward.

UE Rx beam knowledge has impact on TCI state activation requirements, since the requirements are related to whether the TCI state is known or unknown. If the predicted beam is reported within a certain time period before the TCI state switch command, with the assumption that UE Rx beam knowledge is known for the predicted TX beams, the TCI state can be considered as known, and the related TCI state activation delay can be reduced a lot compared with that of the unknown TCI state.

Issue 3-4: Metrics/KPIs for beam prediction For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Add new metric: - Top-K/N (%): the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams". FFS how N is derived. - Option 5: Add a requirement based on relative difference between predictions under different conditions/inputs (for example varying signal levels by N dBs but maintain same relative difference). - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: both RSRP accuracy and beam prediction accuracy are used as KPIs for beam prediction - RSRP absolute accuracy applies to all predicted beams

According to RAN1 agreements, as diplicated as following, Top 1 or Top K beam prediction accuracy is supported. From our point of view, except beam prediction accuracy, RSRP accuracy can also be considered as metrics for requirements/tests, since the two metrics verify the inference performance from different perspective. Beam prediction accuracy (absolute or relative) is similar like existing L1-RSRP measurement accuracy. For example, the absolute prediction accuracy is the difference between predict L1-RSRP and ideal L1-RSRP.  Beam prediction accuracy is used to reflect the percentage that the predict beam is the “genie-aided” beam. Having good RSRP accuracy does not mean that the beam prediction accuracy is good, vice versa.

Agreement@ RAN1 #118b meeting For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

Proposal 4: it is proposed that both the RSRP accuracy and beam prediction accuracy are the metrics/KPIs for beam prediction.

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

In last meeting, it was agreed that the prediction delay includes measurement for prediction, inference delay, and FFS other components. Initially, we do not think measurement need to be considered since it is the legacy measurement. Howver, companies argue that inference caanot be performed without measurement, and from requirements point of view, the start of prediction delay is the time when UE is aked to do prediction. With this explanation, we are fine to include measurement in the prediction delay. But we still think for the component of measurement, legacy measurement delay can be reused, or at least no longer delay is expected.

Proposal 5: for measurement for prediction, legacy measurement delay can be reused, or at least no longer delay is expected.

- Conclusion

This contribution provides discussion on RRM requirements for beam management. The proposals are:

Proposal 1: the relative RSRP accuracy is proposed as:

- Relative RSRP accuracy = predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value.

Proposal 2: it is proposed that the RSRP absolute accuracy requirements apply to all predicted beams, not limit to the beam index in top-K beams.

Proposal 3: For the beam in set B, it is known. For the beam not in set B, it is prefered to assume UE Rx beam is known, but we are also OK to have UE capability to move forward.

Proposal 4: it is proposed that both the RSRP accuracy and beam prediction accuracy are the metrics/KPIs for beam prediction.

Proposal 5: for measurement for prediction, legacy measurement delay can be reused, or at least no longer delay is expected.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface