# 3GPP TSG-RAN WG4 Meeting #114                            R4-2501102

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	5.23.1

Source: 	CMCC

Title: 			 (NR_Mob_enh2-Core) Discussion on RRM maintenance for Rel-18 LTM

Document for:	Discussion

- Introduction

In last meeting, there was discussion on the maintenance for Rel-18 LTM, and a WF was approved [1]. This contribution provides discussion on the open issues.

- Discussion

## 2.1 RAN1 LS

RAN1 sent LS to RAN4 on whether/how to handle the collision between an SSB and a valid RO [2]. The details are duplicated as following.

RAN1 LS on collision between SSB and RA occasion for LTM (R4-2500008) 1. Overall Description: On R18 LTM, for unpaired spectrum, RAN1 identified possible collision between valid RA occasion (RO) and SSB associated with different cells in time domain, which is due to that RO validation is performed per cell. The collision could happen when the different cells are in the same frequency band or in different frequency band. It is unclear to RAN1 on whether/how to handle the collision between an SSB and a valid RO associated with different cells. 2. Actions: To RAN4: RAN1 respectfully asks RAN4 on whether/how to handle the collision between an SSB and a valid RO as described above and to provide answer for it.

Since RO validation for a cell does not consider SSBs from different cells, it is possible that there is collision between SSB and PRACH associated with different cells.

One way is to leave this to UE implementation, i.e. whether to transmit the PRACH or receive SSB is up to UE implementation. Another way is to clarify UE behaviour, make it clear either UE will transmit the PRACH or UE will receive SSB when there is collision. We slightly prefer the latter way.

This issue is similar as the collision between UL transmission and DL measurement. Taking scheduling availability of UE performing measurements in TDD bands on FR1 as an example, the details are duplicated as following. It can be seen that UE is not expected to transmit PUCCH/PUSCH/SRS if they are colliding with SSB, while PRACH is not covered. Our interpretention is that PRACH is prioritized if there is collision between PRACH and SSB. And PRACH is for UL synchronization, which is very important.

Based on above consideration, for the collision between SSB and RO, it is proposed that UE transmit PRACH.

Scheduling availability of UE performing measurements in TDD bands on FR1 (9.2.5.3.1, TS38.133) When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1. When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

Proposal 1: for the collision between SSB and RO, it is proposed to prioritize PRACH tramsmission.

## 2.2 UE feature

| Index | Feature group | Components |
| --- | --- | --- |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. |
| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps |

For 39-3-3, the open issue is that companies has different understanding about the “total cells of serving cells and neighboring cells”.

Option 1: ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

Option 2: ‘total cells of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. cells with SSB resources configured by CSI-MeasConfig are also counted.

We prefer option 1. FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18 is introduced for LTM, only the LTM candidate cell(s) are considered.

Proposal 2: for FG 39-3-3, it is proposed that ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

For 39-3-5, the open issue is that companies has different views about “number of SSB resources UE can measure for L1-RSRP per frequency layer”.

Option 1: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

Option 2: ‘number of SSB resources UE can measure for L1-RSRP per frequency layer’ includes both LTM candidate cell(s) and SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

Similar view as that for FG39-3-3, our preference is option 1, since this is for LTM, only the SSB resources configured for LTM candidate cell(s) are considered.

Proposal 3: for FG 39-3-5, it is proposed that “number of SSB resources UE can measure for L1-RSRP per frequency layer”includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not

- Conclusion

This contribution provides discussion on issue about collision between SSB and RA occasion for LTM. The proposals are:

Proposal 1: for the collision between SSB and RO, it is proposed to prioritize PRACH tramsmission.

Proposal 2: for FG 39-3-3, it is proposed that ‘total cells of serving cells and neighbouring cells’ includes only the LTM candidate cell(s), any of which can be one current serving cell or not.

Proposal 3: for FG 39-3-5, it is proposed that “number of SSB resources UE can measure for L1-RSRP per frequency layer”includes only the SSB resources configured for LTM candidate cell(s), any of which can be one current serving cell or not.

- Reference

- R4-2420234, WF on Further NR mobility enhancements, MediaTek Inc.

- R4-2500008 (R1-2410911), LS on collision between SSB and RA occasion for LTM

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R4-2413960, WF on Further NR mobility enhancements, MediaTek Inc.

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek Inc.

[3] R4-2321621, WF on NR mobility enhancements (part 1), MediaTek Inc.

[4] R4-2403385, WF on NR mobility enhancements (part 1), MediaTek Inc.