3GPP TSG-RAN WG4 Meeting #114	R4-2501109

Athens, GR, 17th – 21st February, 2025

Source:	China Telecom

Title:	RF requirements for PC2 RedCap in FDD bands

Agenda Item:	7.7.1

Document for:	Discussion

- Introduction

In RAN#106, the revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1 was approved [1] by capturing the objective of FDD bands supporting:

- Specify the PC2 RF requirements for RedCap UE on both TDD and FDD bands, based on 1Tx architecture with 26 dBm PA. - The PC2 RF requirements include: UE maximum output power, Tx power tolerance, MPR, A-MPR and RSD requirements if needed. - For TDD bands, PC2 RF requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. - Note: Not differentiate the applicable form factors for TDD bands in the specification. - For FDD bands, specify RF requirements for PC2 RedCap UE - Band specific requirements are specified for the example bands including n3 and n1 when band agnostic way is not applicable (Example bands are selected by considering different sensitivity degradation causes) - Note: Wearable device supporting HPUE is not in the scope for FDD bands in this work item Note 1: Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE. Note 2: PC2 for Rel-17 Redcap and Rel-18 eRedcap are considered for this WI.

This contribution discusses the corresponding specification impact for PC2 RedCap in FDD bands as highlighted in the objectives.

- Discussion

The latest WF [2] was approved with reaching the following agreements regarding PC2 RedCap in FDD bands.

Issue 1-1-1: A-MPR for FD-FDD RedCap PC2 UE Online agreement: - A-MPR for Redcap PC2 UE needs to be specified based on single Tx architecture by re-using the non-RedCap PC2 where applicable Issue 1-2-1: RSD for FD-FDD RedCap PC2 2Rx UE Online agreement: - RSD for non-RedCap 2Rx PC2 UE can be re-used for RedCap PC2 2Rx UE for the bands, on which RSD requirements for non-RedCap 2Rx PC2 are defined based on 1Tx architecture. Issue 1-2-2: RSD for FD-FDD RedCap PC2 1Rx UE Online agreement: - RSD for RedCap PC2 1Rx UE needs to be specified based on single Tx architecture

Thus, based on the above mentioned agreements and WI objectives, we think the specification impact for FDD PC2 RedCap UE including: UE maximum output power, RSD for 1Rx and 2Rx and A-MPR(depend on specific case).

UE maximum output power

The change to the specification only need to remove the restriction of ‘for TDD bands’ as endorsed in the draft CR [3]

Proposal 1: Change to UE maximum output power needs to remove the restriction of ‘for TDD bands’ as endorsed in the draft CR [3]

RSD for 1Rx and 2Rx

For a PC2 RedCap UE equipped with 2 Rx antenna ports, the REFSENS degradation requirement is specified in Table 7.3.2-1c, which are reused from non-RedCap PC2 UE not supporting Tx Diversity

For a PC2 RedCap UE equipped with 1 Rx antenna ports, from specification definition point, we think theΔR1R method that applies to PC3 RedCap UE can be reused. The REFSENS requirement for PC2 RedCap UE with 1Rx is modified by the amount of ΔR1R based on PC2 RedCap UE with 2Rx. In total, the specific value could be derived by

REFSENSPC2 RedCap 1Rx=REFSENSPC3 2Rx + DegardationPC2 1Tx+ ΔR1R

Proposal 2: For a PC2 RedCap UE with 2Rx, the REFSENS degradation requirement is specified in Table 7.3.2-1c

Proposal 3: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC3 2Rx + DegardationPC2 1Tx+ ΔR1R

MPR/A-MPR for RedCap PC2

In general, the MPR requirement for PC2 is relaxed for UE supporting dual Tx comparing to baseline requirement (UE supporting 1Tx or 2Tx with additional capability like full power mode), as shown by the table 6.2.2-2 and 6.2D.2-1. That is to say only 2Tx UE may need additional relax for MPR requirement. Thus, it could be inferred that MPR/A-MPR for 1Tx PC2 RedCap can be reused for PC2 normal UE requirement, which means no need to define MPR/A-MPR for RedCap PC2 UE.

Proposal 4: No need to define MPR/A-MPR for RedCap PC2 UE.

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |
| NOTE 1: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost NOTE 2: For a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostQPSKRel18] is set to 1, the reference power is increased by ΔPPowerBoost |  |  |  |  |

- Conclusions

In this contribution, we discuss the specification impact for PC2 RedCap in FDD bands and have the following proposals:

Proposal 1: Change to UE maximum output power needs to remove the restriction of ‘for TDD bands’ as endorsed in the draft CR [3]

Proposal 2: For a PC2 RedCap UE with 2Rx, the REFSENS degradation requirement is specified in Table 7.3.2-1c

Proposal 3: For a PC2 RedCap UE with 1Rx, the REFSENS requirement is specified by the formula:

REFSENSPC2 RedCap 1Rx=REFSENSPC3 2Rx + DegardationPC2 1Tx+ ΔR1R

Proposal 4: No need to define MPR/A-MPR for RedCap PC2 UE.

- References

RP-243267, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom, MediaTek, RAN#106, Dec., 2024

R4-2314748, WF on NR power class 2 RedCap UE in FR1, China Telecom, RAN4#108, Aug., 2023

R4-2420357, CR to 38.101-1: PC2 Redcap support for TDD bands, Ericsson, RAN4#113, Nov., 2024