3GPP TSG-RAN WG4 Meeting #114 	   R4-2501116

# Athens, Greece, 17th – 21st , Feburary 2025

Agenda Item:	7.27.2.3

Source: 	Ericsson

Title:	On general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September , the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For RAN4 112 bis meeting, many open issues are listed in the WF[2] In this contribution, we will address several aspects which are related to RRM requirements for conditional LTM.

# Discussion

## Scope of the RRM requirement for conditional LTM

Certain agreements have been reached during the RAN4#112bis meeting regarding the Rel-19 CLTM scope. One remaining issue is regarding the subsequent CLTM and how to specify RAN 4 RRM requirement for the subsequent CLTM.

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#112bis < Agreement > - Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion. RAN4 # 113 < Agreement > ad hoc o FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. o Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

Currently, there is no specified L1 measurement requirement for CSI reporting on deactivated SCC. Instead, deactivated SCCs adhere to an explicitly configured measurement cycle for L3 measurements based on TS 38.321 clause 5.9.

The MAC entity shall for each configured SCell:​ 1> if the SCell is deactivated:​ 2> …​ 2> not report CSI for the SCell;​ 2> not transmit on RACH on the SCell;

The Release-18 LTM candidate cell configuration, based on TS 38.331, allows the network to configure a deactivated SCC as an SpCell within the CellGroupConfig IE, and then afterwards as an LTM candidate cell in the ltm-CandidateConfig Following this configuration, the UE is required to perform L1 measurements for the LTM candidate evaluation after receiving the RRC reconfiguration for the LTM candidate cell configuration.

From a configuration perspective, we do not observe any differences between the Release-19 CLTM and the Release-18 LTM; the same configuration process is applicable to CLTM.

From the perspective of UE measurement behaviour, it is unlikely for the UE to deviate from the configuration and omit evaluating a specifically configured candidate cell, as the UE performs measurements on the configured frequency carrier.

While we acknowledge that current L1 measurement requirements for deactivated SCCs are lacking, once the UE receives the LTM candidate configuration, it should no longer treat the frequency carrier as a deactivated SCC, given that the frequency is designated as an SpCell in the LTM candidate configuration.

- Observation 1: Network may configure deactivated SCC as a spCell and as a LTM candidate for evaluation based on Rel-18 LTM procedure.

- Observation 2: Currently there is no L1 measurement requirement for deactivated SCC to be applied after the LTM candidate configuration even though this frequency is being configured as spCell.

- Proposal 1: The deactivated SCC is applicable for the scenario where Pcell switch to LTM candidate cell that is a serving Scell (deactivated SCC) from both procedure and requirement point of view and shall be supported for Rel-19 CLTM.

Issue 1-1-2: Measurement requirements on conditional LTM < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting.

Based on RAN2 # 128 meeting, several agreements were reached from RAN2 in regarding to support the L3 measurement-based condition LTM.

We don’t see any distinguish between FR group for the support of L3 measurements from the RAN2 agreements and we question why RAN4 only supports part of the whole scenario.

Agreements on CLTM: - The triggering condition of conditional LTM can be based on L3 measurement. - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- Proposal 2: The L3 measurement based conditional LTM shall be supported and RAN4 shall specify requirements accordingly both in FR1 and FR2.

## Conditional LTM early synchronization and TCI state activation

Base on RAN4#113 meeting agreements for DL sync, the early TCI state indication and PDCCH order RACH for early UL sync will be supported as Rel-18 legacy LTM and the CLTM requirements will take the consideration of the early sync and early TCI indication.

Issue 1-2-1: Early UL Synchronization RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM.. For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

Besides the PDCCH ordered early TA acquisition, the Rel-18 LTM already supports UE based TA measurement, the issue remains for Rel-19 CLTM is whether to define requirements considering the UE based TA estimation.

From network point of view, the early UL sync shall happen before the conditional LTM measurement trigger cell switch, the UE will go through RACH-less cell switch procedure. This will benefit the mobility cell switch delay hence improve the mobility performance. To have a valid TA value is crucial for the UE to maintain UL sync earlier than the CLTM execution being trigger.

The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition procedure is triggered by PDCCH order realized through UE-based TA measurement as configured by RRC.

In network triggered case, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU. The serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch.

In the UE based case, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command, if it does not include any valid TA value.

The network may also send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

Difference between the Rel-18 LTM and Rel-19 CLTM is there will be no cell switch command. The time when network should order early TA acquisition will totally depend on network, if there is certain UE report being sent to the network this can be very good indication for the network. It is the network responsibility to have the early TA acquisition before the CLTM execution being triggered.

From procedure point of view, the early UL sync of LTM and CLTM are the same. We also understand certain concern on the measurement accuracy of the TA value due to the measurement time of the TA value is up to UE implementation. Based on current RAN2 agreements, when both networks ordered TA acquisition and UE has estimated the TA value, the UE shall follow the value based on network ordered TA acquisition. Since how UE measures the TA value is up to UE implementation, we don’t see the motivation to set requirements on this matter.

- Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

## Conditional LTM delay

Issue 1-4-1: General aspects on Subsequent Conditional LTM delay RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.. < Way forward > FFS the following proposals: - Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. - Proposal 1a: TRRC will not be considered in subsequent conditional LTM delay. - Proposal 1b: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM. - Proposal 2: Wait for further progress in other working groups and postpone detailed discussions on whether and how to define RRM requirements for subsequent conditional LTM.

The subsequent CLTM is one of another aspect that needs to be further discussed. Based on RAN4#113 meeting agreement, both subsequent CLTM and 1st time CLTM ending point of the delay is the same. The ending point defined as the time when UE transmits the first UL transmission on the target cell.

The only difference to have a generic CLTM delay requirement including both 1st time and subsequent CLTM is the definition of the starting point.

We think the subsequent CLTM will not have the RRC delay in comparing with the 1st time CLTM since the RRC processing delay only exist for the 1st time decoding the RRC message for the CLTM full configuration.

Also to mention the starting point can be different in compare the subsequent CLTM and the 1st time CLTM. The subsequent CLTM starting point can be the time when UE transmits the RRCReconfigurationcomplete for the 1st time CLTM which follows similar logic as the subsequent CPC from Rel-18 legacy.

The Rel-18 LTM interruption is the component Tinterrupt.

- Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

- Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution +   +TEarly_process.

- Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: The deactivated SCC is applicable for the scenario where Pcell switch to LTM candidate cell that is a serving Scell (deactivated SCC) from both procedure and requirement point of view and shall be supported for Rel-19 CLTM.

	Proposal 2: The L3 measurement based conditional LTM shall be supported and RAN4 shall specify requirements accordingly both in FR1 and FR2.

	Proposal 3: RAN4 do not define RRM requirements for the UE based TA measurement/estimation.

	Proposal 4: RAN4 specify the 1st time CLTM and subsequent CLTM delay separately .

	Proposal 5: The 1st time CLTM delay can be defined as  DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution +   +TEarly_process.

	Proposal 6: The subsequent CLTM delay can be defined as: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.

- R4-2420102 “WF on RRM requirements for NR_Mob_Ph4_Part2”, China Telecom, November, 2024