3GPP TSG-RAN WG4 Meeting#114		R4-2501122

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.8.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on UE RF requirements for LB-LB CA switching

Document for:	Discussion

Introduction

In RAN-P#106 meeting, a new WID [1] was approved. In this contribution, we want to provide some initial views on UE RF requirements for LB-LB CA switching.

- 4.1 Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

Discussion

UE RF requirements

- Switching delay

According to the new WID, we could take CA_n5A-n29A as an example to analyse in this WI. In RAN4, we need to specify the switching delay and time mask for this band combination. In addition, based on the motivation [2] for this WI, on one side, CA_n5A-n29A, these two DL bands will not work at the same time and it is necessary to perform time-sharing switching based on the TTI period. On the other side, with regard to additional components and volume in the form factor, CA_n5A-n29A aggregating via a single antenna. Hence, in our understanding, switching delay depends on UE architecture implementation.

UE reference architecuture1

Figure1: UE reference architecture1 for CA_n5A-n29A.

Based on UE reference architecture1, there are antenna and RF switch common for n5 band and n29 SDL band. Hence, the switching delay of antenna tuning elements switching is approximately 2us and the switching delay of RF switch is 5us. Hence, in our understanding, 5us or the smallest switching delay 35us for existing spec could be the switching delay.

UE references architecture2

Figure2: UE reference architecture2 for CA_n5A-n29A.

Based on UE reference architecture1, there are antenna and RF switch and mixer common for n5 band and n29 SDL band. Hence, the switching delay of antenna tuning elements switching is approximately 2us and the switching delay of RF switch is 5us, the switching delay of PLL switching is 140us. Hence, in our understanding, 140us could be the switching delay.

Different architecture has different switching delay for CA_n5A-n29A carrier switching. Based on the above analysis, switching delay for LB-LB CA carrier switching could be {5us, 35us, 140us} as a starting point.

Proposal 1: The switching delay for LB-LB CA carrier switching could be {5us, 35us, 140us} as a starting point.

- Time mask

According to the switching procedure based on figure3 [3], RAN1 needs to define a semi-static switching pattern based on RRC configuration. In our understanding, definition of time mask could depend on the semi-static switching pattern from RAN1.

Figure3: possible low-low CA switching solution

Proposal 2: Definition of Time mask could depend on the semi-static switching pattern.

Conclusions

In this paper, we provide our views on UE RF requirements for LB-LB CA, we have made the following proposals.

Proposal 1: The switching delay for LB-LB CA carrier switching could be {5us, 35us, 140us} as a starting point.

Proposal 2: Definition of Time mask could depend on the semi-static switching pattern.

- Reference

- RP-243317, NR_LBCA_Sw WID, Apple. TSG-RAN WG 106 meeting

- RP-242532, NR_LBCA_Sw_motivation, AT&T. TSG-RAN WG 106 meeting