# 3GPP TSG-RAN WG4 Meeting #114	R4-2501129

Athens, GR, 17th – 21st February, 2025

Title: 	On CSI-RS based L1 measurements

Source: 	OPPO

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility[1].

Issue 3-1-1: Whether to consider SSB-based L3 measurement only? < Agreement > - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. - Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether sStep 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] - Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement <Agreement> - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).] - Issue 3-8: supported measurement type <Agreement> - CSI-RS based L1 measurement without gap is supported. - FFS on CSI-RS based L1 measurement with gap

In this contribution, we provide our views on CSI-RS based L1 measurements.

Discussion

ssue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet <Candidate solutions> - Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo) - Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson) - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi) - Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE) <Recommended WF> - Continue discussion.

We think the known or unknown definition is mainly for cell detection, it is known then UE can skip the L3 cell detection and acquiring rough timing with rough beam. Either SSB-based or CSI-RS based L1 measurement is to get the fine time/frequency tracking with fine beam but not to evaluate the cell as known or unknown. About whether to skip step 2 for FR2-1, we think the difference is whether UE can early know the TCI state of CSI-RS L1 beam in step 3.   Depending on whether step 2 is skipped, the requirements in step 3 may be different.

We support to remove L1 measurement for FR2-1 as side condition in step 2 for both FR1 and FR2-1. and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability

Proposal 1: For measurement procedure, support to remove step 2 for FR1 and FR2-1. Define the worst delay requirement in step 3 assuming CSI-RS based L1 measurement is needed for fine timing/frequency tracking.

Issue 3-1-3: Whether to use detectable or measurable? <Candidate solutions> - Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT) - Option 2: Use detectable. (Apple, ZTE) - Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia) <Recommended WF> - Continue discussion. RAN4 can first try to align understanding on what “detectable” and “measurable” mean.

About the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, we prefer to follow CSI-RS based L3 measurement. And due to the cell detection also relies on SSB, the CSI-RS from the target cell configured for L1 measurement remains “measurable”, which is more suitable than “detectable”.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains “measurable”.

Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell <Candidate solutions> - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Saumsung) <Recommended WF> - Continue discussion.

About the side condition, we prefer to reuse the legacy SINR=-3dB. It is not reasonable to relax the condition for CSI-RS.

Proposal 3: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

<Candidate solutions> - Option 1: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells. (CATT) - Option 2: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS. (Apple, vivo) - Option 3: Start from periodic CSI RS measurements, and wait for wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support. (Nokia) - Option 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent. (Samsung) - Option 5: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS (ZTE) <Recommended WF> - Continue discussion.

We prefer to follow option 3 that RAN4 starts from periodic CSI RS measurements, and wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support.

Proposal 4: We prefer to follow option 3 that RAN4 starts from periodic CSI RS measurements, and wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support.

- Issue 3-10: CSI-RS based L1 measurement w.r.t active BWP <Candidate solutions> - Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT) - Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO) - Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE) - Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo) - Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP. <Recommended WF> - Continue discussion.

About the measurement types, we also agree to support intra-frequency without gap and inter-frequency with gap. If companies are ok to deprioritize CSI-RS based L1-RSRP measurement outside active BWP, we are also supportive. But it will impact the definition of intra-frequency measurement which cannot base on the center frequency of CSI-RS resource. Option 2 and 4 are ok to us. We further suggest to only define the requirements for CSI-RS based L1-RSRP measurement within active BWP and without gap.

Proposal 5: Support intra-frequency measurement and inter-frequency measurement, and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. Otherwise, an optional UE capability for supporting the case that CSI-RSs are outside active BWP shall be introduced.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: For measurement procedure, support to remove step 2 for FR1 and FR2-1. Define the worst delay requirement in step 3 assuming CSI-RS based L1 measurement is needed for fine timing/frequency tracking.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains “measurable”.

Proposal 3: Reuse the legacy side condition of SINR=-3dB for CSI-RS based L1 measurement for neighbor cell.

Proposal 4: We prefer to follow option 3 that RAN4 starts from periodic CSI RS measurements, and wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support.

Proposal 5: Support intra-frequency measurement and inter-frequency measurement, and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. Otherwise, an optional UE capability for supporting the case that CSI-RSs are outside active BWP shall be introduced.

Reference

- R4-2420101, WF on NR mobility enhancements Phase 4_part1, Apple