# 3GPP TSG-RAN WG4 Meeting #114	R4-2501135

# Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.26.4.2

Source:	OPPO

Title:	Discussion on core requirements for LP-WUS/WUR

Document for:	Approval

- Introduction

Previous meeting discussed and reached some agreements regarding LP-WUS/WUR in [1] [2]. This contribution will provide our further considerations on the open issues captured in [3].

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Background: Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. RAN4 to further discuss case #2: RRM measurement case index; MR serving cell measurement; MR neighbouring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. Case #4 is covered by issue for higher priority frequency layer: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

During previous meeting, 4 different RRM offloading / relaxation cases were proposed while only case #1 and case #3 were agreed. The remaining issue is whether to additional consider case #2 and case #4. Comparing the case #2 and case #3, we think case #3 is more reasonable. Firstly, if the UE already turns on the MR for serving cell measurement, it could also perform neighboring cell measurement with MR without much extra power consumption. Secondly, working in relaxed cases rather than fully offloading case #1 means that the serving cell quality is not good enough and is likely to further deteriorate, e.g. does not meet the criterion S any longer. To make sure the UE could reselect to a suitable neighboring cell in time, even with equal or lower priority, neighboring cell measurement is necessary. Therefore, we propose to deprioritize case #2. The main purpose of case #4 is to cover higher priority frequency layers, which should be always measured even if the current serving cell has good channel quality in the legacy procedure. The case is one special case between case #1 and case #3 and is trying to open the measurement on high priority layers for fully offloading case. We understand it is not necessary since the neighbor cell measurement can be triggered by setting proper threshold of case #3 to cover the possible mobility issue. Considering the reasons above and workload, we prefer to deprioritize case #4 and focus on the requirements for the already agreed cases.

Proposal 1: Deprioritize case #2 and case #4.

Issue 1-1-3: Measurement requirements to be specified for LP-WUR - Proposals - P1: Do not define requirements for OFDM-based LR serving cell measurement based on LP-SS; RAN4 defines measurement requirements for the following: (xiaomi CMCC ZTE Apple oppo) - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state; - Same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS with and without overlaid OFDM sequence at Idle/Inactive state. (Apple) - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - P2: The measurement requirements of OFDM based LP-WUR based on LP-SS with or without overlaid OFDM sequence(s) will be the same as that of OOK based LP-WUR based on LP-SS. (Huawei vivo)

Agreement in RAN1 #118bis meeting Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Different measurement types could be supported by LP-WUR and whether to define the corresponding RRM requirements for each type needs to be discussed. There should be common understanding to define requirements for OOK based LP-WUR measurement based on LP-SS and OFDM based LP-WUR measurement based on existing SSS, but the controversial part is whether to consider requirements for OFDM based LP-WUR measurement based on LP-SS.

From RAN1 perspective, there will not have specific LP-SS design for OFDM based LP-WUR. If we are going to define the requirements, the evaluations on OFDM based LP-WUR measurement on LP-SS are needed. According to the options in issue 1-1-3 above, the proponents suggest using the same requirements as that of OOK based LP-WUR based on LP-SS. But based on RAN1 agreements, whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable, which means the overlaid sequence may not be standardized and can be left to NW implementation. If NW transmit the unstandardized overlaid sequence, it is not sure whether the UE can meet the requirements. So we prefer to focus on the uncontroversial cases, i.e. OOK based LP-WUR serving cell measurement based on LP-SS and OFDM based LP-WUR serving cell measurement based on existing SSS.

Proposal 2: RAN4 to firstly focus on the measurement requirements for the following two cases:

- OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) - Proposals - P1: LR status is ON (xiaomi) - P2: LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON. (LG) - P3-1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. (Apple) - P3-2: When a LP-based threshold is configured, at legacy case LP-WUR needs be “ON” at least some duration early before comparing with a LP-based threshold. (vivo Huawei Ericsson) - No need define any LR measurement requirements when a UE is at legacy case (vivo Huawei)

RAN2#127 For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. RAN2#127bis - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

In previous meeting, the LP-WUR status at legacy case was proposed to be clarified. Based on RAN2 agreements, LR measurement can be considered for the entry condition for LP-WUS monitoring and fully offloading, and whether to consider LR measurement for entry condition of RRM relaxation is still FFS. If LR based threshold is configured for the evaluation of entry condition, technically LP-WUR needs to be “ON” at least some duration early before comparing with a LP-based threshold. But we don’t think the requirements for this duration is needed since the time for turning on LR is up to UE implementation. And since for legacy case, the measurement is relying on MR and UE is not required to keep the LR always on, we think we do not need to define the measurement requirements for LP-WUR in this scenario.

Proposal 3: No need to specify LR status or any LR measurement requirements when a UE is at legacy case.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

In last meeting, the periodicity(interval) for SSB based LP-WUR measurement delay requirements was discussed and it was agreed to define requirements based on LO periodicity, but there are still two open parts to be decided. The first one is whether to define upper bound when LP-SS is not configured. If the upper bound is defined, the requirements would be defined as min(N*LO periodicity, Tupper bound). And it means when DRX cycle is large, UE is required to perform measurement based on an interval smaller than DRX cycle. We think it is not reasonable or necessary, and there were not any such requirements in legacy. The second one is whether to use LP-SS periodicity when it is larger than LO periodicity. We think it is not needed to define specific requirements for this case and it is better to apply same requirements by using LO periodicity. On the other hand, since we are defining SSB based measurement requirements, it is not preferred to use LP-SS periodicity which may cause confusion. It is possible that only OFDM based receiver is implemented to measure SSB and UE may not differentiate the cases when LP-SS is configured or not configured, then the result is that UE will still follow the requirements only based on LO periodicity.

Proposal 4: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 5: For SSB based LP-WUR measurement requirements, when LP-SS is configured, apply the requirements based on LO periodicity regardless of LP-SS periodicity.

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3) - Proposals - P1: Do not need a dedicated evaluation delay requirement. Use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/ Fully Offloading /MR RRM relaxation (CATT CMCC CT vivo Samsung) - P2: (Apple) - For WUS paging monitoring/MR RRM relaxation: use the MR measurement period and/or LR measurement period to check the criteria; - For MR fully offloading; use the MR measurement period and/or LR measurement period for measurement filtering and a dedicated evaluation delay requirement - P3: Define evaluation requirements for the exit condition based on single measurement period; Define evaluation requirements for the entry condition based on evaluation period which includes multiple measurement period; Each measurement should be based on filtering of at least 2 samples. (Huawei) - P4: (MTK) - RAN4 to define same requirements to evaluate the exit condition for each: 1) exiting from Case#1 to legacy UE and 2) exiting from Case#3 to legacy UE based on LR. - RAN4 to use the same evaluation requirements for entry condition when switching from Case#1 to Case#3 and from Case#3 to Case#1, where - entry conditions from Case#1 to Case#3 can be based on LR only. - entry conditions from Case#3 to Case#1 can be based on LR or LR+MR. - RAN4 to consider LR measurements only to define same requirements to evaluate the entry condition for each: 1) switching from Case#1 to Case#3 and 2) switching from Case#3 to Case#1; RAN4 to define requirements to evaluate the entry condition from legacy UE to Case#1 or Case#3 blased on MR measurements only. - Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements. - P5: (LG) - RAN4 to define evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement. Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for Case 1 and Case 3 - For evaluation requirements for LP-WUR for LP-WUS monitoring/fully offloading (case 1), discuss the following options, - Option 1: Define evaluation requirements used for corresponding entry/exit conditions evaluation using the corresponding LP-WUR measurement period. - Option 2: Define evaluation requirements used for corresponding entry/exit conditions evaluation using a duration longer than corresponding LP-WUR measurement period, FFS on detail. - Option 3: Define evaluation requirements used for corresponding entry conditions based on option 2, and exit conditions based on option 1.

In last meeting, the evaluation requirements for entry/exit condition for case 1 and case 3 were discussed. We understand there are two main aspects to be decided. The first one is the differentiation of different cases, e.g., separate evaluation requirements for case 1 and case 3, and the second one is whether to use a longer duration than measurement period for evaluation requirements of entry/exit conditions.

For the first aspect, we think the main difference is whether LR or MR based threshold should be used, which is discussing and should be decided in RAN2. Based on current agreements in RAN2, the configurable thresholds are summarized below. From RAN4 perspective, we think we can define evaluation requirements for LR based criteria and MR based criteria respectively and not to mention the different cases. As for which requirements to be used, it depends on which threshold is configured and can be left to RAN2 to decide.

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | MR threshold and FFS for LR threshold | FFS |

For the second aspect, the main concern is ping-pong effect. We think a longer duration than measurement period for both entry and exit conditions can be used to avoid it. But if all companies agree, we can also accept longer duration for entry condition and single measurement period for exit condition.

Proposal 6: RAN4 to define evaluation requirements for LR based criteria and MR based criteria respectively. which requirements to be used for each case depends on the threshold configured and can be left to RAN2 to decide.

Proposal 7: Define evaluation requirements used for corresponding entry/exit conditions evaluation using a duration longer than corresponding LR/MR measurement period.

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo) DRX cycle length [s]; Scaling factor. 0.32; TBD. 0.64; TBD. 1.28; TBD. 2.56; TBD. Issue 1-3-2: On Neighbour cell and serving cell relaxation factor Agreement: Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

In last meeting, it was agreed that the MR RRM relaxation factor is larger than or equal to 4 and the same relaxation factor applies to serving cell and neighbor cell, but the detailed scaling factor is still FFS. Firstly, we understand the scaling factor should be the fixed value defined in the spec rather than to be configured by the NW, since there are no criteria for NW to determine the relaxation factor. Then for the fixed scaling factor, we prefer to use single value and 8 or 16 can be considered as candidate. But if all companies agree, we can compromise to define multiple values depending on DRX cycle length, i.e., option 3.

Proposal 8: The scaling factor for MR RRM relaxation is defined as single fixed value and can be selected from 8 or 16.

Issue 1-2-4: On interruption related requirements for LP-WUR - Proposals - P1: Wait for RAN1 further progress or based on RAN1’s conclusion. (CATT CMCC oppo Huawei) - P2: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple) - P3: Suggest to use the term “MR activation” time to replace “interruption” time; Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, ramping duration is based on RAN1 conclusion, detailed conditions are FFS. (vivo) - “MR activation” duration = Ramping duration + (2 + 2) samples * SSB_ periodicity - “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * SSB_ periodicity + TSI-NR + 2*Ttarget_cell_SMTC_period - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung) - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g. 1~2 SSB sufficient - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

Agreement in RAN1 #118 meeting For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement in RAN1 #118bis meeting Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

RAN1#119 meeting: Proposal 1-4r1 For the 3 candidate values for the wake-up delay capability report, support {[downselect between 40ms, 80ms], 500ms, and 900ms}. - Note: It is assumed that SSB periodicity is 20msec For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

In our understanding, the discussed interruption time is the wake-up delay discussed in RAN1, which is also applicable for RRC idle/inactive modes. As shown above, RAN1 is still discussing the approach for UE capability report which will impact RAN4 discussion. So we suggest wait further RAN1 progress on this issue.

Proposal 9: The interruption time/wake-up delay between LP-WUS reception and MR ready for paging monitoring could be defined based on further progress in RAN1.

Issue 1-4-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi vivo LG ZTE Apple MTK) - P2: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress. Paging reception requirement impact can be discussed after further input from RAN2 and RAN1 (Samsung) - P3: Postponed until more progress in other groups (Ericsson Huawei) - P4: RAN4 to discuss interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. (Qualcomm) - Postpone the discussion on this issue (vivo)

According to the objectives description in the WID, the measurement is performed by MR at connected mode and no new RAN4 RRM requirements are needed.

Proposal 10: No RRM objectives is expected for connected mode in this WI.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Deprioritize case #2 and case #4.

Proposal 2: RAN4 to firstly focus on the measurement requirements for the following two cases:

- OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Proposal 3: No need to specify LR status or any LR measurement requirements when a UE is at legacy case.

Proposal 4: No need to define upper bound when LP-SS is not configured for SSB based LP-WUR measurement requirements.

Proposal 5: For SSB based LP-WUR measurement requirements, when LP-SS is configured, apply the requirements based on LO periodicity regardless of LP-SS periodicity.

Proposal 6: RAN4 to define evaluation requirements for LR based criteria and MR based criteria respectively. which requirements to be used for each case depends on the threshold configured and can be left to RAN2 to decide.

Proposal 7: Define evaluation requirements used for corresponding entry/exit conditions evaluation using a duration longer than corresponding LR/MR measurement period.

Proposal 8: The scaling factor for MR RRM relaxation is defined as single fixed value and can be selected from 8 or 16.

Proposal 9: The interruption time/wake-up delay between LP-WUS reception and MR ready for paging monitoring could be defined based on further progress in RAN1.

Proposal 10: No RRM objectives is expected for connected mode in this WI.

References

- R4-2420117, Ad-hoc minutes for NR_LPWUS, vivo, RAN4#113

- R4-2420100, WF on RRM requirements for NR_LPWUS, vivo, RAN4#113

- R4-2418284, Topic summary for [113][226] NR_LPWUS, vivo, RAN4#113