3GPP TSG-RAN WG4 Meeting #114		R4-2501148

Athens, Greece, 17th – 21st February 2025

Agenda item:	7.17.2.2

Source:	Intel Corporation

Title:	Discussion on delay reduction by optimizing CSSF outside gap

Document for:	Discussion

# Introduction

The Rel-19 NR RRM enhancements work item was approved in March 2024 and an updated WID was approved in RANP #105 in [1]. The work item descriptions of objectives of the item are shown in the below box.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last WG meeting, a WF was approved in [2] to capture all the discussions and agreements.

In this paper we provide our analysis on delay reduction by optimizing CSSF outside gap.

# Discussion

## CSSF outside gap in CA/DC operations

Measuring one serving CC per band

One of the motivations to have this general enhancement for all FR2-1 UE CSSF outside gap is that the intra-band carriers in FR2 share the most similar characteristics as targets to SSB based L3 measurements. There is a consensus in RAN4 since Rel-15 that for FR2 target cells of a certain band, the UE chooses target cells within a single frequency layer for neighbour cell measurements. This is specified in TS 38.133 clause 9.2.3.2.

For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Based on this consensus, for FR2 measurements, there is no need to scale the measurement delay by number of carriers within the same band for each serving SCC. The UE measures on one carrier of a band in FR2 and all other carriers in the band is considered as known and measured. The measurement results can be shared among all the intra-band carriers. Note that there are indeed preconditions for this enhancement to apply to the existing requirements for FR2-1 SSB based L3 measurement delay.

In Rel-19 RAN4 specifications, we need to define the rules and principles for the measurement when the UE supports single carrier measurement within one band, and when the network allows the UE to do so.

  - The UE needs only to measure on one of the carriers among all that are configured as CA from the network to the UE in a single band in FR2.

  - UE chooses the carrier that it measures on among all carriers in the same band.

  - The UE chooses the PCC with priority over all others, and the UE then chooses PSCC otherwise.

  - When there is no PCC nor PSCC in the target band, the UE chooses an SCC where the target SSB is configured in a Measurement Object to the UE.

  - Otherwise, it is up to UE’s implementation and network is not clear about which one the UE chooses.

  - When the network indicates to the UE within a FR2 band, that the UE is required to measure on the indicated carrier, the UE follows the network indication (the indication from network can be per-UE RRC indication).

Proposal 1: The UE needs only to measure on one of the carriers among all that are configured as CA from the network to the UE in a single band in FR2.

Proposal 2: The UE chooses the PCC with priority over all others, and the UE then chooses PSCC otherwise. When there is no PCC nor PSCC in the target band, the UE chooses an SCC where the target SSB is configured in a Measurement Object to the UE. Otherwise, it is up to UE’s implementation and network is not clear about which one the UE chooses.

Proposal 3: When the network indicates to the UE within a FR2 band, that the UE is required to measure on the indicated carrier, the UE follows the network indication (the indication from network can be per-UE RRC indication).

Proposal 4: Single carrier measurement does not apply to FR1.

Additional searcher for CSSF outside gap

Another direction of enhancements to CSSF outside gap within the scope of this item is to allow UE 3 searchers assumption. This means that the UE with three searchers support 3 parallel measurements, as compared to the legacy assumption which is 2, that facilitate a much faster measurement in general and it is beneficial to the system mobility/measurement performance. Different from single CC measurements, this assumption applies to both FR1 and FR2 measurements.

When measurement gap is not configured to the UE, the UE uses the additional searcher on measurements without gap to speed up the measurement reporting of any interested carrier for the system.

In the last WG meeting, an initial agreement was

Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

In the Rel-19 specifications, we define the additional searcher enhancements so that the UE can measure outside measurement gap on a target carrier either from network indication or from the UE’s choice.

  - The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

  - The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

  - The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

  - The CSSF outside gap definition for the additional searcher measurement requirements is 1 when there is only one target carrier, while it is N when there is N carriers configured/decided by UE among the targets.

  - This requirement applies only when there is no measurement gap configured in Rel-19.

  - This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

Proposal 5: The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

Proposal 6: The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

Proposal 7: The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

Proposal 8: This requirement applies only when there is no measurement gap configured in Rel-19.

Proposal 9: This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

# Conclusion

In this paper we provide our analysis on delay reduction by optimizing CSSF outside gap.

Proposal 1: The UE needs only to measure on one of the carriers among all that are configured as CA from the network to the UE in a single band in FR2.

Proposal 2: The UE chooses the PCC with priority over all others, and the UE then chooses PSCC otherwise. When there is no PCC nor PSCC in the target band, the UE chooses an SCC where the target SSB is configured in a Measurement Object to the UE. Otherwise, it is up to UE’s implementation and network is not clear about which one the UE chooses.

Proposal 3: When the network indicates to the UE within a FR2 band, that the UE is required to measure on the indicated carrier, the UE follows the network indication (the indication from network can be per-UE RRC indication).

Proposal 4: Single carrier measurement does not apply to FR1.

Proposal 5: The UE follows measurement behaviours according to the legacy measurement and report configurations with legacy searcher assumptions. UE PCC/PSCC/SCC measurements are not enhanced, and the existing requirements remain unchanged.

Proposal 6: The UE is required to use the additional searcher (an extra set of baseband resources for RRM measurement) according to network configuration which is dedicated to the additional searcher feature.

Proposal 7: The target of the UE measurement using the additional searcher is either according to the network indication or to the UE’s choice.

Proposal 8: This requirement applies only when there is no measurement gap configured in Rel-19.

Proposal 9: This requirement applies to all scenarios including FR1 CA/DC, FR2 CA/DC and FR1 + FR2 cases.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple