3GPP TSG RAN WG4 Meeting #114		 R4-2501150

Athens, GR, 17th – 21nd February, 2025

Agenda Item:	7.6.2

Source: 	MediaTek Inc.

Title: 	Discussion on remaining signalling aspects for DL fragmented carriers study

Document for:	Approval

1. Introduction

It had been discussed in previous RAN4 meeting that considering the spectrum hold by North America operators are usually non-contiguous blocks as stated in the reference [1][2][3]. In the RAN4#112bis, UE architecture assumption and initial evaluation based on existing test configurations were agreed and captured in the WF [4]. In this contribution, we continue discussion and provide our proposals on the open issues.

2. Discussion

2.1 Continue discussion on signalling aspects

In the previous RAN4 meetings, it had been agreed the method for reducing number of UE Rx chains is to adopt full-shared Rx chain architecture (one Rx RF chain). As for signalling aspects, fallback behaviour and semi-static switching, they were still open for further discussion. It was agreed in RAN4#113 that companies are encouraged to provide analysis with pros and cons on preferred signalling solutions.

When UE is attached to a serving cell, UE needs to report its category and supported band combos per network enquiry. UE may further report its capability including RF parameters (include band combos), feature group indicators and UE-NR-Capabilities. For the UE supporting DL NCCA with one Rx RF chain capability, there may be corresponding UE capability to be introduced and reported in the above procedure.

If UE supports one Rx RF chain, the UE would be able to simultaneously support higher order CA combos. Considering UE may also be capable to apply the new capability on more than one band, it needs to be considered how the new capability is properly reported. There are some alternatives can also be considered for UE report the new capability as discussed in [8] and copied below:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

Alt. 2: Band combination capability update with the supported number of CCs

Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

To minimize RAN2 signalling design effort, it would be nature to consider Alt 1. Furthermore, we still need to consider bellow different signalling approaches:

Alt 1-1: The capability is reported per CC per band per BC

Alt 1-2: The capability is reported per band per BC

Other approaches can be further discussed in RAN4

Observation 1: The one Rx RF chain capability is better to be reported as per band per BC. Signalling design also need to consider how to report the capability when the number of non-contiguous carriers is three or more carriers.

When we discuss the signalling consideration of the new capability, it would also need to think together with how to determine the mechanism to enable or disable one Rx RF chain configuration as the agreed WF[7] stated:

Issue 2-5: Triggering condition to enable ”One Rx RF chain”

RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled

- Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled

  - Other determining factors are not precluded to be studied

- Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers that may degrade the receiving signal quality of wanted carriers.

A new measurement mechanism would be needed to justify the in-gap blocker power level and compare with the wanted signal power levels.

In one Rx RF chain mode, gNB may trigger and configure UE to measure the power levels of total received signals, including out-of-channel, PCC, SCC and in-gap blocker/interference and noise with measurement procedure using existing RSSI measurement method. The network can configure the interference measurement bandwidth and the virtual RB location to be measured, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. It is also possible to apply the measurement together with wanted CC so that measurement gap may not be needed.

In the separated Rx RF chain mode, measurement gap based measurement would be required, while in the onn Rx RF chain mode, the UE may be able to perform gap-less measurement on both wanted CCs and in-gap range since the supported bandwidth of one UE is at least up to 100MHz to cover both wanted CCs and in-gap range.

Figure 1. Spectrum plot of DL fragmented carriers and in-gap interference

Observation 2: The nominal NR UE can support one CBW up to 100MHz so it would be possible to to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 3: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can compare the ratio of in-gap interference level and wanted CCs level to judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be saved with this approach

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Observation 4: There are different signalling methods can be considered. One aspect is UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) to inform the network that network can schedule CA configurations to UE accordingly. Another aspect is UE report all measured information including in-gap interference level to let the network judge whether one Rx RF chain can be enabled and corresponding CA configurations.

Proposal 1: The existing RSSI measurement method can be used for the in-gap interference measurement

In last RAN4 meeting, issue 2-7 was also left for further discussion and copied below:

Issue 2-7: Others

RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s

In our view, the fundamental CA configuration is single band with two downlink non-contiguous carriers in the study item. So there must be either one of the two-fragmented carriers to be a PCELL. For the extension application of this capability, it may be possible to be applied for an intra-band three non-contiguous carrier aggregation with two adjacent carriers of them are possible to enable one Rx RF chain capability. So the signalling may also need to consider how to indicate the capability for three or more carriers scenarios.

Proposal 2: For single band operation, there must be either one of the two-fragmented carriers to be a PCELL. For higher order configuration, the DL fragmented carriers with one Rx RF chain can be including one PCell or pure SCells. The signalling design for one Rx RF chain capability shall also consider the higher order configuration usage as well.

The discussion on fallback configuration was also not yet clarified. Here we provide our views on the fallback operation from single band point of view and view of higher order configuration though the higher order discussion may slightly beyond the SI scope.

For single band operation, where there are only two CCs which is baseline of the study, when network or UE judge the in-gap interference is too large that one Rx RF chain is not applicable, fallback configuration needs to be applied. The fallback of the DL NCCA configuration shall be a single CC case as illustrated in the Figure 2 below. During the discussion in previous meeting one company was worried about what would the UE behaviour be when the CC1 is not PCC. In our understanding, current fallback may keep PCC unchanged. As for the role change, it is already supported by current RRC specification which is not a part of fallback operation. This is also same for a single band with more than two CCs that may be considered as extension of the feature.

Figure 2. Illustration of single band fallback for one Rx RF chain

For two band (band A and B) operation when there are two DL non-contiguous CCs in band A and single DL CC in band B as illustrated in Figure 3 bellow. Our assumption is fallback options are up to network scheduling. When network or UE judge the in-gap interference is too large that one Rx RF chain is not applicable, there may be several possible cases:

Case 1. Network may ask UE to release CC3. For this case, current RRC signalling is already capable so no new signalling is needed for the fallback case. The band A will continue suffer performance degradation from large in-gap interference if the network does not schedule change on band A.

Case 2. Network may ask UE to release CC2 which is same with that of single band fallback operation. No new signalling is need for this case either.

Case 3. UE may apply less Rx path for the band A. eg. UE only apply 2Rx in band A if band A requires mandatory 4Rx. This could be the case if network does not response to UE’s measurement report. This is not expected by UE but this could happen since it is up to network how to treat the UE measurement report. We may need to address somewhere in the spec that no UE requirement is expected for the case.

Figure 3. Illustration of two-band fallback for one Rx RF chain per band

Proposal 3: It is up to network how to apply the fallback. However there shall be no UE requirement for the case 3 if UE reports unable to continue one Rx RF chain mode but network does not response to this.

Proposal 4: We propose to capture above signalling approach in the last part of the contribution into TR38.755

3. Conclusion

For triggering condition to enable ”One Rx RF chain” mode:

Observation 1: The one Rx RF chain capability is better to be reported as per band per BC. Signalling design also need to consider how to report the capability when the number of non-contiguous carriers is three or more carriers.

Observation 2: The nominal NR UE can support one CBW up to 100MHz so it would be possible to to measure noise level of both wanted CCs and in-gap range if total BW is within the 100MHz under one Rx RF chain mode.

Observation 3: One possible method to measure in-gap interference level is to referring existing RSSI measurement procedure with virtual RB ranking in the in-gap region or refer to measuring neighbour cell procedure. The measurement would be applicable in both one Rx RF chain mode and separated Rx RF chain mode.

Observation 4: There are different signalling methods can be considered. One aspect is UE can judge whether the one Rx RF chain can be enabled and just send one-bit (True/False) to inform the network that network can schedule CA configurations to UE accordingly. Another aspect is UE report all measured information including in-gap interference level to let the network judge whether one Rx RF chain can be enabled and corresponding CA configurations.

Proposal 1: The existing RSSI measurement method can be used for the in-gap interference measurement

For applicability on PCell or SCell:

Proposal 2: For single band operation, there must be either one of the two-fragmented carriers to be a PCELL. For higher order configuration, the DL fragmented carriers with one Rx RF chain can be including one PCell or pure SCells. The signalling design for one Rx RF chain capability shall also consider the higher order configuration usage as well.

Discussion on fallback behaviour:

Proposal 3: It is up to network how to apply the fallback. However there shall be no UE requirement for the case 3 if UE reports unable to continue one Rx RF chain mode but network does not response to this.

Proposal 4: We propose to capture above signalling approach in the following contents of the contribution into TR38.755

TP for TR38.755

5. Study of methods for reducing the number of UE Rx chains

<Start of TP>

On the signalling aspect for the one Rx RF chain mode

When UE is attached to a serving cell, UE needs to report its category and supported band combos per network enquiry. UE may further report its capability including RF parameters (include band combos), feature group indicators and UE-NR-Capabilities. For the UE supporting DL NCCA with one Rx RF chain capability, there may be corresponding UE capability to be introduced and reported in the above procedure.

If UE supports one Rx RF chain, the UE would be able to simultaneously support higher order CA combos. Considering UE may also be capable to apply the new capability on more than one band, it needs to be considered how the new capability is properly reported. There are some alternatives can also be considered for UE report the new capability as discussed in [8] and copied below:

Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

Alt. 2: Band combination capability update with the supported number of CCs

Alt. 3: New band combination definition e.g. CA_F_n25(2A) + new set of signaling

To minimize RAN2 signalling design effort, it would be nature to consider Alt 1. Furthermore, we still need to consider bellow different signalling approaches:

Alt 1-1: The capability is reported per CC per band per BC

Alt 1-2: The capability is reported per band per BC

Other approaches can be further discussed

When we discuss the signalling consideration of the new capability, it would also need to think together with how to determine the mechanism to enable or disable one Rx RF chain configuration as the agreed WF[7] stated:

Triggering condition to enable ”One Rx RF chain”

RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled

- Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled

  - Other determining factors are not precluded to be studied

- Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2

When the UE enables one Rx RF chain to receive two non-contiguous carriers under the network, it was assumed another adjacent channel downlink carrier, from collocated second operator, in the gap may also exists at the same time. From companies simulation, the in-gap blocker level may possible be 10 or 20 dB higher than the wanted carriers that may degrade the receiving signal quality of wanted carriers.

A new measurement mechanism would be needed to justify the in-gap blocker power level and compare with the wanted signal power levels.

In one Rx RF chain mode, gNB may trigger and configure UE to measure the power levels of total received signals, including out-of-channel, PCC, SCC and in-gap blocker/interference and noise with measurement procedure using existing RSSI measurement method. The network can configure the interference measurement bandwidth and the virtual RB location to be measured, wherein the RB location to be measured in the in-gap region can be referring to PCC or one of the CCs. It is also possible to apply the measurement together with wanted CC so that measurement gap may not be needed.

In the separated Rx RF chain mode, measurement gap based measurement would be required, while in the onn Rx RF chain mode, the UE may be able to perform gap-less measurement on both wanted CCs and in-gap range since the supported bandwidth of one UE is at least up to 100MHz to cover both wanted CCs and in-gap range.

Figure 5-1. Spectrum plot of DL fragmented carriers and in-gap interference

After UE measures all the power levels, there are different signalling methods to proceed the measurement results:

- UE judge whether one Rx RF chain is applicable: Consider different UE may have different interference rejection capability and to save dynamic signalling overhead, UE can compare the ratio of in-gap interference level and wanted CCs level to judge whether the one Rx RF chain can be enabled and only report capable/not-capable and necessary CCs information to network. The signalling overhead can be saved with this approach

- Network judge whether one Rx RF chain is applicable: An alternative is that UE reports all the measured information, including in-gap interference level, wanted signal power levels and UE capable ISR (interference-to-signal ratio), to network for the network to judge whether one Rx RF chain can be enabled.

Discussion on fallback aspects

For single band operation, where there are only two CCs which is baseline of the study, when network or UE judge the in-gap interference is too large that one Rx RF chain is not applicable, fallback configuration needs to be applied. The fallback of the DL NCCA configuration shall be a single CC case as illustrated in the Figure 2 below. During the discussion in previous meeting one company was worried about what would the UE behaviour be when the CC1 is not PCC. In our understanding, current fallback may keep PCC unchanged. As for the role change, it is already supported by current RRC specification which is not a part of fallback operation. This is also same for a single band with more than two CCs that may be considered as extension of the feature.

Figure 5-2. Illustration of single band fallback for one Rx RF chain

For two band (band A and B) operation when there are two DL non-contiguous CCs in band A and single DL CC in band B as illustrated in Figure 3 bellow. Our assumption is fallback options are up to network scheduling. When network or UE judge the in-gap interference is too large that one Rx RF chain is not applicable, there may be several possible cases:

Case 1. Network may ask UE to release CC3. For this case, current RRC signalling is already capable so no new signalling is needed for the fallback case. The band A will continue suffer performance degradation from large in-gap interference if the network does not schedule change on band A.

Case 2. Network may ask UE to release CC2 which is same with that of single band fallback operation. No new signalling is need for this case either.

Case 3. UE may apply less Rx path for the band A. eg. UE only apply 2Rx in band A if band A requires mandatory 4Rx. This could be the case if network does not response to UE’s measurement report. This is not expected by UE but this could happen since it is up to network how to treat the UE measurement report. We may need to address somewhere in the spec that no UE requirement is expected for the case.

Figure 5-3. Illustration of two-band fallback for one Rx RF chain per band

In general It is up to network how to schedule UE fallback configuration. However there shall be no UE requirement for the case 3 if UE reports unable to continue one Rx RF chain mode but network does not response to it.

<End of TP>

4. Reference for the contribution:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2417201 WF on fragmented DL carriers study, RAN4#112bis

- R4-2415510 On methods for reducing the number of UE Rx chains for fragmented carriers, Apple, RAN4#112bis

- R4-2415804 On methods for reducing the number of UE Rx chain, Huawei, RAN4#112bis

- R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

- R4-2415510 On methods for reducing the number of UE Rx chains for fragmented carriers, RAN4#112bis, Appple