# 3GPP TSG-RAN WG4 Meeting #114	R4-2501157

# Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	7.26.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were discussed in RAN4 #113 [1], and a WF was agreed [2]. According to the WF, the link level simulation assumption for ACS/ASCS evaluation has been agreed upon, but some prominent issues for REFSENS, which includes reference architecture, NF, and IM, still need to be further discussed.

In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- Type of LP-WU receiver

One particular issue that was discussed in RAN4#113 is whether to specify one type or two type of receivers.

- Discuss and decide how to define two LP-WUR types (capability).

- Several options can be considered:

  - 1) OOK-based and OFDM-based, can support only one or both

  - 2) high-NF-based and low-NF-based, can support only one?

  - 3)other?

In our view, whether we would need to have separate LP-WUR type for OOK and OFDM is up to RAN1 design, where an issue is whether the OFDM symbol would carry any information. From the RAN4 aspect, the focus shall be whether the same requirements can be applied to all LP-WUR implementations.

Proposal 1: RAN4 discussion on the LP-WUR type shall focus on whether the same minimal requirements can be applied to different receiver implementations or not.

The number of requirements for OFDM and ED-based LP-WUR has been discussed over the past few meetings without a conclusion. In general, both types of WUS signal and the corresponding receivers need to support sufficient coverage of LP-WUS (as PUSCH of Msg 3). Since RAN4 only defines minimum requirements, and the requirements themselves are usually implementation agnostic, it should be sufficient to define only one set of requirements covering all receivers.

Observation 1 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Proposal 2: There is no need to distinguish OFDM based and OOK based receiver from the minimal requirement aspect.

Another dimension of device type was brought into discussion in RAN4#113, which is based on the NF. In our understanding, the intention here is to allow low power implementation in the LP-WUR. However, this is not a proper way from the minimal requirement aspect. RAN4 requirements always allow different implementations with different NFs if they meet the target performance. In other words, the high NF implementation can be accommodated as long as the NF is not too high to meet the target coverage of the LP-WUS. In the section below, we provided some baseline NF which allow very low power consumption implementation (without LNA), and we suggest using those NF as baseline to define the REFSENS requirement.

Proposal 3:  As RAN4 minimal requirements allow all different implementations with both high and low NF (as long as the receiver can meet the coverage target), there is no need to define different LP-WUR type based on NF.

Last but not least, we would like to highlight the importance of striving to define a single set of core requirements for LP-WUR. Multiple LP-WUR types can lead to device and market fragmentation, which increases the cost of supporting this feature from both UE and network side, which will eventually negatively impact on the adoption of this technology in the field. Therefore, RAN4 shall strive to specify a single set of requirements for all LP-WUR implementations and avoid increasing the number of device type.

Proposal 4: RAN4 shall strive to specify a single set of requirements for all LP-WUR implementations and avoid increasing the number of device types.

- REFSENS for LP-WUR

Per the outcome of the study item phase in TR38.869, the NF of zero-IF based ED receiver is in the range of 12-16 dB, which is higher than the NF of OFDM receiver that was typically assumed for UE in FR1 (in the range of 9-12 dB). Therefore, it can be expected that the REFSENS of the ED-based receiver will be worse than the OFDM-based receiver, and RAN4 may consider focusing on the ED-based receiver to derive the minimum requirement.

Proposal 5	 RAN4 adopts zero-IF ED-based receiver to derive a single set of minimum requirements on REFSENS for zero-IF ED and OFDM receivers.

## Side condition for REFSENS

Although multiple contributions have been submitted in previous RAN4 meetings on the target SNR for LP-WUR, there is a lack of detail on how the sequence has been detected; thus, it is hard to compare the results. For the sake of clarity, we will explain how our detection algorithm works in our simulation. In our simulation, we assume a preamble is prior to the data information, and this is to allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity. The LP-WUR first needs to look for the preamble and only continues to detect the data if the preamble is detected. Fig. 1(a) shows the general signal structure used in our simulation, which contains sequences of length K and L bits, representing the preamble and data, respectively. The data part indicates the UE's subsequent action after LP-WUS detection and/or identity of (group) UE. Fig. 1(b) illustrates a general block diagram of the digital baseband processing used to detect the LP-WUS in this contribution.

- (b)

Fig. 1 (a) LP-WUS structure, consisting of two fields, preamble used for synchronization and possibly cell identity purposes, and data part to indicate the potential subsequent action or UE (group) identity. (b) Digital baseband processing.

Per our understanding, the usage of the preamble can significantly reduce the FAR, and it will further impact the MDR when we design the detection threshold for the sequence. Meanwhile, the receiver could also use LP-SS to compensate for the frequency drift in the oscillator further. Although the preamble and LP-SS design are still under RAN1 discussion, it would be beneficial if RAN4 concludes whether any preamble or LP-SS would be considered in the RF testing before determining the actual SNR threshold.

Proposal 6	Pending on RAN1 decision, RAN4 shall decide if the preamble or LP-SS would be configured in the LP-WUS RF testing.

## REFSENS requirement

In this section, we present our proposed threshold SNR, NF, and REFSENS values based on the zero-IF architecture of the ED-based receiver.

Based on our simulation results, the target SNR based on our sequence and receiver design above can reach about -7 dB for 1% MDR and -3 dB for 1% BER under AWGN channel (see appendix), there are about 4dB processing gain (1% MDR vs. 1% BER) based on our design. Considering that the different receiver implementations may present different processing gains, an SNR value between -7 and -2 dB can be considered.

Observation 2	 Based on our sequence and receiver design, the target SNR is about -7 dB for 1% MDR and -3 dB for 1% BER.

Proposal 7	RAN4 adopts a target SNR value in the range between -7 dB to -2 dB for LP-WUS.

The NF of a zero IF receiver is estimated as below, an antenna switching is included to take into account the antenna sharing mechanism. It can be seen in total 10 dB NF is sufficient to accommodate the receiver performance. In addition, a relatively high NF for LNA is assumed with consideration to lower the power consumption on it.

| Component | Gain (dB) | NF (dB) |
| --- | --- | --- |
| Matching network /Antenna switching/RF BPF | -4 | 3 |
| LNA | 15 | 5 |
| Mixer | -1 | 10 |
| BB amplifier | 30 | 3 |
| BB LPF | -1 | 2 |
| 4-bit ADC | -1 | 4 |
| Total | 48 (with LNA) /23 (without LNA) | 9.15 (with LNA) /14.42 (without LNA) |

For further power consumption reduction, LNA may consider to be reduced, where this can lead to an increased NF to 15 dB. To maximize the power saving gain on the LP-WUR, it is proposed that RAN4 consider taking 15 dB NF as starting point to derive the REFSENS.

Proposal 8	RAN4 adopts the NF value of 15 dB as baseline to derive the REFSENS.

The implementation margin (IM) of 2.5 dB has been used to derive the REFSENS for UE in both LTE and NR. In our understanding, this IM value should at least cover both RF and BB margins, and 2.5 dB can be used as a starting point to derive the REFSENS requirement for LP-WUR.

Proposal 9	RAN4 adopts 2.5 dB IM as a starting point to derive the REFSENS.

With all the parameters above, the REFSENS of LP-WUS for 11 RB with 30 kHz can be calculated as below:

REFSENS=-174 dBm+10*log3.96MHz+15 NF+-2SNR+2.5 dB (IM)=-92.5 dBm

Observation 3	 With the proposed SNR, NF, and IM, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -92.5 dBm.

With such an estimation of REFSENS, RAN4 can further check if the coverage target can be fulfilled.

Proposal 10 	RAN4 consider the REFSENS value of -92.5 dBm as starting point and further checks if the coverage target can be fulfilled once some baseline REFSENS value is agreed.

- ACS and ASCS Requirement

## Requirement for ACS and ASCS

From the network co-existence aspect, the LP-WUS is expected to co-exist with other signals and suffer the same level of interference as a normal NR receiver. Thus, setting the same interference signal level for the LP-WUR as MR would be reasonable for the ACS requirement.

Observation 4	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Proposal 11	Define the same interference signal level as MR for LP-WUR for ACS requirements.

Meanwhile, it is understood that due to the lower power consumption in the LP-WUR, the capability of LP-WUR to tolerate interference would be worse than the main receiver. Therefore, it is possible to specify a higher wanted signal level in the core requirements to balance the power consumption and the interference mitigation for the LP-WUR.

Proposal 12: a higher wanted signal can be specified for LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

For ACSC requirement, based on our simulation results (see appendix), the number of guards RBs does not significantly affect ACSC performance. Therefore, it is proposed that 0 RB be adopted for ACSC test cases.

Proposal 13 	RAN4 adopts 0 guard RB for ACSC test case.

- Testability of LP-WUS

From the testability aspect, different from the legacy REFSENS or ACS test for the MR, there will be no ACK/NACK for the LP-WUS receiver, and therefore, how the test equipment (TE) shall be able to count the missed detection rate is unclear.

It is clear that if a full cycle of wake up and connection needs to be performed (wake-uppaging random access  connected mode then go back to Idle), the test time will be excessive.

Observation 5: the test time maybe excessive if a full cycle of wake up and connection needs to be performed.

As an alternative, 3GPP may also investigate whether succeeding transmissions (e.g., PRACH) on the main radio can be detected by the TE once the UE has been woken up. For the sake of test time, it would be preferred that no connection mode needs to be created. However, it would be a question of how the UE LP-WUR would behave if there is no response from the TE on the MR after a few attempts.

Observation 6	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

One possible solution is to implement a dedicated test mode to allow the UE to record whenever it detects an intended wake-up sequence and report this value to TE once the test is done.

Observation 7	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

But in general, we think the detail of test method can be left to RAN5 to decide, while RAN4 can focus on the issue of what needs to be tested.

Proposal 14 the detail of test method can be left to RAN5, while RAN4 focus on what needs to be tested.

On this aspect, it is possible for a UE to support the LP-WUS under both Idle mode and connected mode. For the RF requirement, it is sufficient to test one of the modes in our view as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both modes.

Proposal 15  For RF requirement, it is sufficient to only verify the UE performance under one state (e.g., either Idle or connected) as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both Idle and Connected modes.

As Idle mode is the most important use case, it is proposed to take the test under Idle mode as a baseline for further discussion.

Proposal 16  Test under IDLE mode can be taken as a baseline since this is likely to be the most important use cases for Rel-19 LP-WUS.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Observation 2	 Based on our sequence and receiver design, the target SNR is about -7 dB for 1% MDR and -3 dB for 1% BER.

Observation 3	 With the proposed SNR, NF, and IM, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -92.5 dBm.

Observation 4	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Observation 5: the test time maybe excessive if a full cycle of wake up and connection needs to be performed.

Observation 6	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

Observation 7	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

Proposal 1 	RAN4 discussion on the LP-WUR type shall focus on whether the same minimal requirements can be applied to different receiver implementations or not.

Proposal 2 	There is no need to distinguish OFDM based and OOK based receiver from the minimal requirement aspect.

Proposal 3  	As RAN4 minimal requirements allow all different implementations with both high and low NF (as long as the receiver can meet the coverage target), there is no need to define different LP-WUR type based on NF.

Proposal 4	 RAN4 shall strive to specify a single set of requirements for all LP-WUR implementations and avoid increasing the number of device types.

Proposal 5	 RAN4 adopts zero-IF ED-based receiver to derive a single set of minimum requirements on REFSENS for zero-IF ED and OFDM receivers.

Proposal 6	Pending on RAN1 decision, RAN4 shall decide if the preamble or LP-SS would be configured in the LP-WUS RF testing.

Proposal 7	RAN4 adopts a target SNR value in the range between -7 dB to -2 dB for LP-WUS.

Proposal 8	RAN4 adopts the NF value of 15 dB as baseline to derive the REFSENS.

Proposal 9	RAN4 adopts 2.5 dB IM as a starting point to derive the REFSENS.

Proposal 10 	RAN4 consider the REFSENS value of -92.5 dBm as starting point and further checks if the coverage target can be fulfilled once some baseline REFSENS value is agreed.

Proposal 11	Define the same interference signal level as MR for LP-WUR for ACS requirements.

Proposal 12	a higher wanted signal can be specified for LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Proposal 13 	RAN4 adopts 0 guard RB for ACSC test case.

Proposal 14 the detail of test method can be left to RAN5, while RAN4 focus on what needs to be tested.

Proposal 15  	For RF requirement, it is sufficient to only verify the UE performance under one state (e.g., either Idle or connected) as we only verify the detection performance of the LP-WUR rather than the function and protocols, and the LP-WUS and the receiver is expected to be the same for both Idle and Connected modes.

Proposal 16  	Test under IDLE mode can be taken as a baseline since this is likely to be the most important use cases for Rel-19 LP-WUS.

- References

R4-2415235		Topic summary for [112bis][131] NR_LPWUS_UERF	Moderator(Vivo)

R4-2417112		WF on UE RF requirements for LP-WUS	Vivo

TR38.869		Study on low-power Wake-up Signal and Receiver for NR

- Appendix LLS

In this section, we present initial simulation results for ASCS analysis with OOK-4 waveform. Our simulations are based on the simulation assumption for link level simulation (LLS) agreed during RAN4#111, shown below:

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Chip rate | M=4 |
| WUS duration | 126 bits ~ 1mSec |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0 |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 |
| WUS RB | - Fixed 11RB ~ 3.96MHz |
| Position within channel | - Center of the channel |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth |
| Filter | - 5th Order Butterworth matching fixed 3.96MHz RF bandwidth |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same average PSD with WUS signal |
| Sampling rate | 7.68MHz |
| ADC bit width | Ideal ADC |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Power boosting | No power boosting |
| Channel Model | TDL-C 300 and AWGN |
| Performance metric | 1% MDR with false alarm rate below 1% |

In our simulation, LP-WUS design has the following characteristics:

- Kasami sequences of equal length of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 1 msec.

- Overlaid OFDM sequence with length 16 consisting of an m-sequence of length 15 and a random single bit. The overlaid OFDM sequence is generated based on option 1-1 agreed in RAN1, i.e., before DFT/LS block.

- The LP-WUS occupies 16 OFDM-symbols, 11RBs (1 RB GB at each side of the LP-WUS bandwidth) with subcarrier spacing of 30 kHz.

We have used ideal ADC but it is expected that the ADC performance would not degrade our MDR performance significantly. The order of lowpass Butterworth is set to 5 and we assume no frequency and timing error.

We study the results, BER and MDR, for AWGN and TDL-C 300 channel for two guard band settings of 0 RB and 1 RB on each side of the LP-WUS bandwidth. The achieved false-alarm rate for the studied scenarios is below 0.1%, thanks to LP-WUS structure, sequence design, and DBB processing.

Fig. A1  Bit error rate (BER) for AWGN (left) and TDLC (right) channel models.

Fig. A2    Miss detection rate (MDR) for AWGN (left) and TDLC (right) channel models.

Based on this initial simulation results, it can be observed that the number of guard RB does not affect the MDR performance significantly. Additionally, the processing gain from the sequence design can compensate for performance loss of the LP-WUR due to its ultra-low power design, allowing to reach the target coverage level even at relatively high BER.