# 3GPP TSG-RAN WG4 Meeting #114                                                             R4-2501174

# Athens, Greece, 17th – 21st  February, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for RedCap NTN enhancement

Agenda item:	7.29.4.1

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN phase 3 [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. As this is the first meeting for RAN4 RRM to work on this WI, we will provide our initial views on potential impacts on RRM requirements.

Discussion

The objectives of Rel-19 NTN phase 3 enhancement are described in [1] including:

2.1 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective. The objective will be divided into several aspects:

  - 2.1 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

RAN4 shall define the RRM requirements based on the latest revised WID as below:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In above WID description, we know that RAN4 shall combine with R17/R18 RedCap WI and R17/R18 NR NTN WI to discuss this objective. Thus, we shall study the features introduced in RedCap which will impact the definition of requirements.

2.1.1 RRM requirements for (e)RedCap UE with FR1-NTN

In the last meeting, we have discussed the potential requirements one by one and three general cases shall be considered:

- Unique requirements for RedCap

- Unique requirements for NR NTN

- Shared requirements for RedCap and NR NTN

2.1.1.1 Potential impact on RRM requirements for RedCap UEs (1Rx and 2Rx)

First of all, we would like to analyze the potential RRM requirements with the reduced UE receiving branches. And the details can be seen as the following sections.

2.1.1.1.1 Shared requirements for RedCap and NTN

2.1.1.1.1.1 RRC Idle/Inactive mobility

Based on the RRM requirements for legacy normal UE, the potential R19 RedCap UEs in NR NTN mobility requirements in IDLE/INACTIVE may be:

Table 2 Potential cell re-selection requirements

| Scenario |  | Requirements | Has impact or not |
| --- | --- | --- | --- |
| TN-NTN | NR-NR | Inter-frequency cell re-selection | Yes |
|  | LTE-NR | Inter-RAT cell re-selection | RAN2 is discussing |
| NTN-TN | NR-NR | Inter-frequency cell re-selection and Inter-RAT cell re-selection | Yes |
|  | NR-LTE |  | Yes |
| NTN-NTN (earth-fixed) |  | Intra-frequency cell re-selection | Yes |
|  |  | Inter-frequency cell re-selection | Yes |
| NTN-NTN (earth-moving) |  | Intra-frequency cell re-selection | Yes |
|  |  | Inter-frequency cell re-selection | Yes |

Based on WID description “UEs with NR NTN operating in FR1-NTN bands”. We deem that the above scenarios shall be considered. Combined with discussion in RedCap WI, we shall specify the requirements on 1Rx and 2Rx.

- Measurements of  TN-NTN and NTN-TN for 1Rx and 2Rx RedCap UE

| TN-NTN | NR-NR | Inter-frequency cell re-selection | Yes |
| --- | --- | --- | --- |
|  | LTE-NR | Inter-RAT cell re-selection | RAN2 is discussing |
| NTN-TN | NR-NR | Inter-frequency cell re-selection and Inter-RAT cell re-selection | Yes |
|  | NR-LTE |  | Yes |

In R18 NR NTN WI, we define the cell re-selection requirements between TN and NTN including inter-RAT and inter-frequency measurements requirements. Thus, RAN4 shall define related requirements on TN-NTN (NR-NR) and NTN-TN(NR-NR/NR-LTE) for 1Rx and 2Rx RedCap UE. As for TN-NTN (LTE-NR), RAN2 is under discussion and there has been no RAN4 RRM impact so far.

Observation 1: RAN2 is discussing TN to NTN (LTE to NR) cell re-selection and there has been not RAN4 RRM impact so far.

Observation 2: RAN4 has already defined the inter-RAT and inter-frequency measurement requirements between TN and NTN in R18 NTN.

For measurement requirements in inter-frequency and inter-RAT, we mainly define the Tdetect/measure/evaluate, and the legacy measurement requirements can be seen as below:

TN to NTN (NR to NR): Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for TN DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1; FR2-1Note1; FR2-2 Note2. 0.32; 1; 8; 12; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 5; 8; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 6; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 5; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length.. Table 4.2C.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for NTN DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter [s] (number of DRX cycles); Tmeasure,NR_Inter [s] (number of DRX cycles); Tevaluate,NR_Inter [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x 1.5 (36 x N1 x 1.5); 1.28 x N1 x 1.5 (4 x N1 x 1.5); 5.12 x N1 x 1.5 (16 x N1 x 1.5). 0.64; 17.92x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.4-2: Tdetect,NR_Inter_enh, Tmeasure,NR_Inter_enh and Tevaluate,NR_Inter_enh for NTN DRX cycle length [s]; Tdetect,NR_Inter_enh [s] (number of DRX cycles); Tmeasure,NR_Inter_enh [s] (number of DRX cycles); Tevaluate,NR_Inter_enh [s] (number of DRX cycles). 0.32; [3.2 x M2 (10 x M2)] Note 1; [0.32 x M3 ([1] x M3)] Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; [6.4 (10)]; [0.64 (1)]; 1.92 (3). 1.28; [10.24 (8)]; 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2. NTN to TN (NR to NR): Same as TN to NTN. NTN to TN (NR to LTE): Table 4.2.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN DRX cycle length [s]; Tdetect,EUTRAN [s] (number of DRX cycles); Tmeasure,EUTRAN [s] (number of DRX cycles); Tevaluate,EUTRAN [s] (number of DRX cycles). 0.32; 11.52 (36); 1.28 (4); 5.12 (16). 0.64; 17.92 (28); 1.28 (2); 5.12 (8). 1.28; 32(25); 1.28 (1); 6.4 (5). 2.56; 58.88 (23); 2.56 (1); 7.68 (3).

From my perspective, the cell re-selection measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs.

Proposal 1: The measurement requirements for inter-frequency and inter-RAT measurement requirements in legacy NTN shall be reused for 2Rx and 1Rx RedCap UEs when considering TN to  NTN (NR-NR) and NTN to TN (NR-NR).

Proposal 2: For inter-RAT measurement, the orginal accuracy margin shall be maintained for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

- Measurements of NTN-NTN for 1Rx and 2Rx RedCap UE

| NTN-NTN (earth-fixed) | Intra-frequency cell re-selection | Yes |
| --- | --- | --- |
|  | Inter-frequency cell re-selection | Yes |
| NTN-NTN (earth-moving) | Intra-frequency cell re-selection | Yes |
|  | Inter-frequency cell re-selection | Yes |

In R17, we mainly introduced the time-based and location-based condition for NTN to NTN cell re-selection. In R18, we just reuse the legacy NTN requirements and only introduced movingReferenceLocation and location-based margin from 50m to 80m based on RAN2 agreements because of earth-moving scenario. Thus, the legacy requirements shall be reused for 1Rx and 2Rx RedCap UEs.

Proposal 3: The cell re-selection intra-frequency and inter-frequency measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs when considering NTN to NTN cell re-selection.

2.1.1.1.1.2 Measurement Procedure

Based on R17/R18 NR NTN discussion for normal UE, the measurement related requirements can be seen as below:

| Measurement | Intra-frequency measurement | Intra-frequency cell identification and measurement period |
| --- | --- | --- |
|  | Inter-frequency measurement | Inter-frequency cell identification and measurement period |
| Scenario | Multiple SMTC | Based on time and frequency difference |

For defining measurement requirements, we mainly focus on the intra-frequency and inter-frequency measurement especially for cell identification and measurement period. However, RAN4 shall clarify the basic scenario for redcap UE in NTN before studying the related requirements.

Multiple SMTCs

In legacy, RAN2 introduced multiple SMTC based on time and frequency difference. Although we are studying the redcap UE in NTN scenario, the multiple SMTCs shall also be reused since in my opinion, this is related to the NTN scenario not the types of UE. For legacy NTN, 2 SMTCs are mandatory supported and 4 SMTCs are optionally supported based on the UE capability. However, considering the complexity limitation of RedCap UEs, we deem that more SMTCs maybe are not proper for them. Thus, to my understanding, 2 SMTCs are enough for RedCap UEs.

Observation 3: RAN2 introduced the multiple SMTCs based on time and frequency difference in NTN in R17.

Proposal 4: Multiple SMTCs (2 SMTCs) shall be reused for RedCap UEs in NTN scenario.

Cell identification

In this section, we mainly discuss the inter-frequency measurement without measurement gaps as an example.

PSS/SSS detection and time index detection

UE performs cell identification including the time period of PSS/SSS detection and time index detection. The requirements are defined as below:

Legacy normal UE： Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1.. Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. RedCap UE: Table 9.2B.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 2Rx RedCap UE DRX cycle; TPSS/SSS_sync_intra_RedCap. No DRX; max(600ms, ceil(5 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-3: Time period for PSS/SSS detection, (Frequency range FR1) for 1Rx RedCap UE DRX cycle; TPSS/SSS_sync_intra_RedCap. No DRX; max(600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(600ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-4: Time period for time index detection (FR1) for 2Rx RedCap UE DRX cycle; TSSB_time_index_intra_RedCap. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2B.5.1-5: Time period for time index detection (FR1) for 1Rx RedCap UE DRX cycle; TSSB_time_index_intra_RedCap. No DRX; max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap. DRX cycle≤ 320ms; max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap. DRX cycle>320ms; Ceil(6 x Kp) x DRX cycle x CSSFintra_RedCap. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. NTN scenario： Table 9.2C.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(1.5 x 5 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2C.5.1-2: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Based on the table in above, we find that the legacy NR NTN requirements and the legacy  requirements for 2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period of PSS/SSS detection and time index detection are relaxed and highlight can be seen in the table.

From my perspective, the time period of PSS/SSS detection and time index detection can be reused for 2Rx RedCap UE in NTN. For 1Rx RedCap UE in NTN, the legacy time period for 1Rx RedCap UE can be baseline.

Observation 4: The legacy NR NTN requirements and the legacy requirements for  2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period are relaxed.

Proposal 5: RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements.

Measurement period (intra-frequency measurement without measurement gaps)

Based on R17/R18 discussion, we also discussed the legacy TN/ legacy RedCap UE/legacy NR NTN measurement requirements, and we find that the requirements are same in TS 38.133. Thus, to my understanding, the RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

Observation 5：The measurement period requirements for legacy TN/ legacy RedCap UE/legacy NR NTN are same in TS 38.133.

Proposal 6: The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

2.1.1.1.2 Unique requirements for NTN

In legacy, RAN4 has already studied several unique requirements for NTN based on its special characteristics. In the last meeting, we also agreed several potential RRM requirements shall be studied for RedCap UEs in NTN scenario. And the following potential requirements agreed in last meeting can be seen as below:

| Satellite switch with re-synchronization | Hard / Soft switching delay |
| --- | --- |
| Pathloss reference signal switching delay | MAC-CE based pathloss reference signal switch delay |

Also, the following aspects in the below table shall firstly study the feasibility and then study the related RRM requirements:

FFS: - R17/18 Conditional Handover - RACH-less Handover - Minimization of Derive Tests (MDT) in RRC Idle state and RRC Inactive state

Next, we will also study each requirements one by one.

2.1.2.1.2.1 Satellite switch with re-synchronization

For satellite switch, this is NTN specific requirement that we have hard satellite switch and soft satellite switch.

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The requirements in this clause are applicable to SA NR SAN. The requirements in this clause are applicable to both hard and soft switch over in quasi-earth fixed scenario from NR SAN FR1 cell to NR SAN FR1 cell.

For hard satellite switch, when the UE switches from the current serving satellite to a new satellite, it first disconnects from the original satellite before establishing a connection with the new one.

For soft satellite switch, it allows the UE to maintain connections with multiple satellites simultaneously. During the handover process, the UE first establishes a communication link with the target satellite and only disconnects from the original satellite after ensuring the new connection is stable.

- Hard satellite switching delay

For hard satellite switch delay, the main delay can be seen as below:

Figure1  Hard satellite switch delay

The interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

First of all, we find that the interruption time in hard satellite switch delay is nearly same as the RedCap UEs handover requirements. We can see the each component of interruption time in legacy hard satellite switch delay defined in existing spec:

Legacy Satellite hard switching interruption time: the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - T∆ is same as the one defined in section 6.1C.2.2.2.1. - Tprocessing is time for UE processing. Tprocessing can be up to 10 ms. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH, according to NW configuration and scheduling, or PRACH if TA timer is not running and there is no PUCCH SR - Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

We know that the Tsearch will be impacted by reduced UE receiving branches. From my perspective, if the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2dB, then Tsearch =1*Tfirst_ssb ms if UE is capable of  2Rx antennas; Tsearch=2*Tfirst_ssb ms if UE is only required to support 1Rx antenna.

Observation 6: The hard satellite switching delay will be impacted by reduced UE receiving branches.

Proposal 7: Considering hard satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

- Soft satellite switching delay

For soft satellite switch delay, the main delay can be seen as below:

Figure 2 Soft satellite switch delay

The soft satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

We know that the soft satellite switch delay includes two parts: Sync to target satellite and the interruption. For the first part, it includes full or partial Tsearch + T∆ + Tmargin or t-service-t-serviceStart. For Tsearch + T∆ + Tmargin, if the delay of Tsearch + T∆ + Tmargin  is larger than t-service-t-serviceStart, the interruption will also contain partial Tsearch + T∆ + Tmargin  and the soft satellite switch delay will be much longer than the case which the delay of Tsearch + T∆ + Tmargin  is smaller than t-service-t-serviceStart. Tsearch is also involved in soft satellite switch delay. To my understanding, the same method in hard satellite switch shall be used.

Proposal 8: Considering soft satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

2.1.1.1.2.2 Pathloss reference signal switching delay

For pathloss reference signal switch, this is the RS designated by the network for the UE to update its channel pathloss. For example, when a UE moves from the edge of current cell to a new cell, the power of signal and quality will change.To ensure the UE can adapt the new channel condition, the network will send the MAC-CE activation message to notify UE that the RS shall be switched. The UE is required to complete the switch within a specific window after receiving this message and commence the pathloss estimation using the new reference signal. However, the specific window is actually the pathloss reference signal switching delay. If the target pathloss reference signal is known, upon receiving PDSCH carrying MAC-CE activation in slot n, UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot n +  THARQ+3Nslotsubframe,µ +  NM∗  5∗Ttarget_PL−RS + 2msNR slot lengtℎ.  The UE shall be able to apply old pathloss reference signals until the slot n + THARQ+ 3Nslotsubframe,µ.

However, we think there is no impact on RedCap UEs in NTN scenario since each component is not impacted by reduced UE receiving branches.

Proposal 9: RAN4 shall reuse the legacy passloss reference signal switching delay in NTN for 1Rx and 2Rx RedCap UEs in NTN scenario.

2.1.1.1.2.3 Minimization of Derive Tests (MDT) in RRC Idle state and RRC Inactive state

For minimization of derive tests, RAN4 mainly focus on the logged MDT which is different legacy derive tests. The general procedure can be seen as below:

Figure  MDT procedure

And for this part there are three parts shall be considered:

- Logging measurements in RRC_IDLE state, reporting the logged measurement;

- Logging of RRC connection establishment failure, reporting the logged failure;

- Logging of radio link failure and handover failure, reporting the logged failure.

For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

To my  understanding, RAN4 shall define the related timing accuracy requirements. However, the requirements will not be impacted by the reduced UE receiving branches since the accuracy of the relative time stamping is the drift of the time stamping. Whatever it’s the 1Rx UE or 2Rx UE, if the drift of time stamping for each measurement data collection is not more than the seconds in legacy, the requirements can be applied. Since the differences between 1Rx UE and 2Rx UE in terms of timestamp recording accuracy are primarily determined by their internal hardware configurations, not directly by the number of antennas.

However, we see majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Observation 7: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 8: Majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Proposal 10: RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

2.1.1.1.3 Unique requirements for RedCap

In the last meeting, we just discussed the requirements only for RedCap which can be seen as below:

- Configured Grant based Small Data Transmission (CG-SDT) - Random Access based Small Data Transmission (RA-SDT)

For unique requirements for RedCap, we will analyze its feasibility for RedCap UEs in NTN scenario and then decide to whether define the related RRM requirements or not.

2.1.1.1.3.1 Configured Grant based Small Data Transmission (CG-SDT)

For CG-SDT, it likes the LTE PUR which does not need the PRACH process in order to reduce the overhead of the uplink resources and reduce the power consumption especially for the small data transmission.

Based on the existing specification, the UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. Thus, the most important thing is the timing synchronization. RAN4 has already defined the requirements to perform the TA validation based on the RSRP change. When UE is configured with the CG-SDT parameters, UE will store the original RSRP value which is the measurement reference RSRP1. Then UE will select SSB which is above the cg-SDT-RSRP-ThresholdSSB or has the same SSB index as the SSB selected for initial transmission for CG-SDT. Based on the selected SSB, UE will measure the current RSRP value of the downlink pathloss reference and than the change between the stored RSRP and the RSRP for current DL pathloss reference will be calculated. If the change is within the threshold, the TA is valid; otherwise, UE will drop the CG-SDT. The current RedCap UEs’ requirement is as below:

Table 5.5.3-1 Valid measurement for FR1 for UE not configurd with eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

We see that the SDT maybe also apply for the NTN scenario which is not the corner case. However, if we plan to define the requirements in NTN, GSO and NGSO shall be distinguished.

For NTN GSO scenario, if UE receives the CG-SDT configuration, UE shall store the latest RSRP value, then UE will select the proper SSB to measure the current DL RSRP, the RSRP change will be smaller since the position of GSO satellite is stable from UE perspective and UE would not have the large mobility. The legacy RedCap UEs’ RSRP measurement can be the baseline.

For NTN NGSO scenario, the main difference compared to GSO is the velocity of the satellite. In theory, the RSRP change will be very large for NGSO and the TA will be invalid based on RSRP change which will exceed the RSRP change threshold.  However, based on RAN2’s understanding we find the threshold of RSRP change can be up to 34dB:

We think this threshold is relaxed enough and also there are 5bits spared. Thus, we support the CG-SDT shall be studied in NGSO scenario. The  valid measurement for FR1 in NGSO shall be the same as in GSO.

The next point shall be also considered in NTN scenario which is the SDT timer, the TA validation shall be verified before the timer expiration.

Above all, the final valid measurement for FR1 can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different:

The 1 Rx RedCap UE for determining whether to perform SDT procedure defined in clause 5.27 [7] applies: - sdt-RSRP-Threshold-r17 as the signaled value of sdt-RSRP-Threshold-r17 [2] + 1 dB. sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated

We shall also reuse the margin as legacy.

Observation 9: The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission.

Observation 10: For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different.

Observation 11: Based on RAN2’s understanding,  the threshold of RSRP change can be up to 34dB.

Proposal 11: CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)) |
| RSRP2 | (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2 |
| Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |

Proposal 12: Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs.

2.1.1.1.3.2 Random access based Small Data Transmission (RA-SDT)

For random access based small data transmission, we find the legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs. Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs. Thus, if the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. And the accuracy margin for 1Rx can be reused.

Observation 12: The legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs

Proposal 13: If the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. The extra 1dB accuracy margin for 1Rx can be reused.

2.1.1.2 Potential impact on RRM requirements for RedCap UE (HD-FDD UE)

The second feature is half-duplex FDD operation. Compared to FD-FDD UE, HD-FDD operates by using separate frequency bands for uplink and downlink communication, but with a UE only being able to either transmit or receive at any given time – not both simultaneously. In RedCap WI, they discussed the several requirements for HD-FDD UE and define lots of applicable conditions not the new requirements. We can see the handover requirements as below:

Handover: Legacy RedCap HD-UE： For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

We find that the legacy handover requirements can be met for HD-FDD UE, however, the measurement applicable conditions may be defined for HD-FDD operation. Thus, when we considering defining the requirements for RedCap HD-FDD UE in NTN scenario, the legacy requirements can be the baseline and the applicable condition can refer to the legacy conditions.

Proposal 14: The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

2.1.1.3 Other features

For this section, RAN4 is still discussing whether to consider or not on:

- eDRX for LEO

- Relaxation measurement in Idle/Inactive state

- Concurrent gap

We will study the above features one by one in following contents.

2.1.1.3.1 eDRX for LEO

In the last meeting, for eDRX configuration RAN4 reached the following agreements:

- < Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

For LEO quasi earth-fixed scenario,  we obtain the information that the LEO quasi earth-fixed serving time can be up to 10min based on the lowest evaluation angle LEO 30°. Some companies provide the other values like 5min,7min etc. Thus, the serving time may be from 5min to 10min. From my perspective, the eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 10.24s (Tdetect=235.52s). However, for inter-frequency NR cells, the number of inter-frequency carriers also needed to be considered, so some margin of measurement time shall be needed. If the eDRX value is defined as 10.24s and the network configures one carrier for inter-frequency measurement, the 10.24s is suitable. If eDRX value is defined as 10.24s and the network configures more than one carrier for inter-frequency measurement, 10.24s is not suitable since the Tdetect_inter_redcap could not be enough to cover more than one carrier measurement if considering the smallest serving time 5min.

Observation 13: The serving time may be from 5min to 10min for  LEO quasi earth-fixed cell.

Observation 14: If consider more than one carrier for inter-frequency measurement, eDRX equals 10.24s could not be enough to cover more than one carrier measurement if considering the smallest serving time 5min.

Proposal 15: The eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 5.12s.

2.1.1.3.2 Relaxation measurement in Idle/Inactive state

In the last meeting, RAN4 discussed the relaxation measurement for RedCap UE in FR1 NTN scenario and the following agreements can be seen:

< Agreement >: - For supported types of relaxations for (e)RedCap UE with FR1-NTN, - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection. - RAN4 to further discuss the thresholds and specific relaxation factor.

In the last meeting, we just mentioned that low mobility criteria and stationary criteria could not be distinguished by UE since the change of RSRP is not obvious in NTN scenario because of near-far-effect, the following figure can be seen:

Figure NTN near-far-effect

Thus, we deem that the legacy relaxed criteria could be reused, that is, low mobility condition and not-at-cell edge condition could be reused (R16 criteria). For relaxation factor, we think if the legacy R16 criteria is used, the legacy scaling factor shall be considered, which K3=3 is enough for us.

Observation 14: Low mobility criteria and stationary criteria could not be distinguished by UE since the change of RSRP is not obvious in NTN scenario because of near-far-effect

Proposal 16: For relaxed criteria, low mobility condition and not-at-cell edge condition could be reused (R16 criteria).

Proposal 17: For relaxed scaling factor, the legacy scaling factor shall be reused (K3=3).

Conclusion

In this paper we provided our views on RRM requirements on R19 NR RedCap NTN .

Observation 1: RAN2 is discussing TN to NTN (LTE to NR) cell re-selection and there has been not RAN4 RRM impact so far.

Observation 2: RAN4 has already defined the inter-RAT and inter-frequency measurement requirements between TN and NTN in R18 NTN.

Proposal 1: The measurement requirements for inter-frequency and inter-RAT measurement requirements in legacy NTN shall be reused for 2Rx and 1Rx RedCap UEs when considering TN to  NTN (NR-NR) and NTN to TN (NR-NR).

Proposal 2: For inter-RAT measurement, the orginal accuracy margin shall be maintained for 1Rx RedCap UE in NR NTN if the legacy requirements are reused.

Proposal 3: The cell re-selection intra-frequency and inter-frequency measurement requirements in legacy NTN can be reused for 2Rx and 1Rx RedCap UEs when considering NTN to NTN cell re-selection.

Observation 3: RAN2 introduced the multiple SMTCs based on time and frequency difference in NTN in R17.

Proposal 4: Multiple SMTCs (2 SMTCs) shall be reused for RedCap UEs in NTN scenario.

Observation 4: The legacy NR NTN requirements and the legacy requirements for  2Rx RedCap UE are the same as legacy requirements. For 1Rx RedCap UE, the time period are relaxed.

Proposal 5: RAN4 shall define time period of PSS/SSS detection and time index detection. The requirements for 1Rx and 2Rx RedCap UE in NTN scenario shall reuse the legacy RedCap requirements.

Observation 5：The measurement period requirements for legacy TN/ legacy RedCap UE/legacy NR NTN are same in TS 38.133.

Proposal 6: The RedCap UEs with 1Rx and 2Rx in NTN shall own the same measurement period requirements as legacy RedCap UE.

Observation 6: The hard satellite switching delay will be impacted by reduced UE receiving branches.

Proposal 7: Considering hard satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

Proposal 8: Considering soft satellite switching delay, Tsearch will not be changed which is 1*Tfirst_ssb ms for 2Rx RedCap UEs. For 1Rx RedCap UEs, Tsearch will be 2*Tfirst_ssb ms.

Proposal 9: RAN4 shall reuse the legacy passloss reference signal switching delay in NTN for 1Rx and 2Rx RedCap UEs in NTN scenario.

Observation 7: For the logged MDT requirements and RRC connection establishment failure logging and reporting, the requirements consist of two parts: measurement requirements and relative time stamp. Different part has different requirements, among them the relative time stamp has different time duration which is values in seconds.

Observation 8: Majority companies do not have strong view to define the related requirements for MDT since the RedCap UEs have low complexity and low cost. Few companies deem that the cell size is large enough for NTN and MDT requirements have been defined in legacy NTN. Thus, MDT shall be defined. However, several companies think that normal UEs can also perform MDT and it is not necessary to study MDT for RedCap UEs. We do not have the strong view on this.

Proposal 10: RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

Observation 9: The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards the serving cell prior to transmission.

Observation 10: For 1Rx and 2Rx RedCap UEs, the requirements are same only the accuracy margin is different.

Observation 11: Based on RAN2’s understanding,  the threshold of RSRP change can be up to 34dB.

Proposal 11: CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be:

Measurement

FR1

RSRP1

(T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX))

RSRP2

(T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2

Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.

Proposal 12: Compared to 2Rx UE, extra 1dB margin shall be needed for 1Rx RedCap UEs.

Observation 12: The legacy SDT requirements is the same as the legacy RA requirements for 1Rx and 2Rx UEs

Proposal 13: If the random access for 2Rx and 1Rx RedCap requirements are defined, the RA based SDT requirements shall be reused. The extra 1dB accuracy margin for 1Rx can be reused.

Proposal 14: The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario.

Observation 13: The serving time may be from 5min to 10min for  LEO quasi earth-fixed cell.

Observation 14: If consider more than one carrier for inter-frequency measurement, eDRX equals 10.24s could not be enough to cover more than one carrier measurement if considering the smallest serving time 5min.

Proposal 15: The eRDX could be configured for LEO quasi earth-fixed cell and the value can be up to 5.12s (Tdetect=235.52s).

Observation 14: Low mobility criteria and stationary criteria could not be distinguished by UE since the change of RSRP is not obvious in NTN scenario because of near-far-effect

Proposal 16: For relaxed criteria, low mobility condition and not-at-cell edge condition could be reused (R16 criteria).

Proposal 17: For relaxed scaling factor, the legacy scaling factor shall be reused (K3=3).

- References

[1] RP-241667，Rel-19 WID NR NTN phase 3.

[2] TS38.133

[3] TS38.304

[4] TS38.331