3GPP TSG-RAN WG4 Meeting#114		R4-2501196

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.6.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN4#113 meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

- Main session online agreements #### Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. - Ad hoc meeting agreements #### Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 #### Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode - Companies are encouraged to bring analysis with their own assumptions following the table format - A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1 Table 1. Link budget of Self-interference for 1st configuration on band n3 Tx leakage; Wanted Carrier (PCC); Wanted Carrier (SCC); Noise floor; ADC noise (11 bit): -68.0dBFs (6.02N+1.76). Tx leakage mapping to 0dBFs (PAPR is not considered); Tx leakage mapping to -12dBFS (PAPR is considered). PA out; 27; -; -; -; -; -. Duplexer isolation; 50; -; -; -; -; -. Rx Antenna input; -; -97; -92.3; -; -; -. FE loss; -; 4; 4; -; -; -. Duplexer output; -23; -101; -96.3; -; -; -. LPF attenuation; 13.4; 1; 1; -; -; -. LPF output (dBm); -36.4; -102; -97.3; -; -; -. Noise floor for basic REFSENS; -; -; -; -101; -; -. NF increase due to AGC adjustment; -; -; -; 10; -; -. Noise floor; -; -; -; -91; -; -. In-band noise (Derived based on MSD and SNR = -1dB); -; -; -96.3; -; -; -. ADC output (dBm); -; -; -; -; -104.4; -92.4. SNR after ADC (dB); -; -; -; -; -11.2(PCC); -13.4(PCC). -7.6(SCC); -9.4(SCC). #### Issue 3-4: Discussion on in-gap requirements RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier #### Issue 2-4: Signaling related WF: Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. #### Issue 3-1: Discussion on evaluation priority of example bands WF: It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase. - Open issues (for information) Open issues are to be discussed in the meetings later on. Other issues are not precluded to be further discussed #### Issue 2-7: Others RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s #### Issue 3-2: Discussion on maximum power level of in-gap interference WF: For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS #### Issue 3-5: Discussion on DL fragmented carriers in TDD bands WF:Background: To consider transition time if there’s one LO for the TDD band and study the feasibility Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation: - Option 1: One LO will be enough for both UL and DL. - Option 2: Separate LOs will be needed for UL and DL.

Discussion

Triggering condition to enable “One RF Rx chain”

According to the last meeting agreement, RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled. Further study whether the absolute power level of in-gap interferer and/or the PSD difference in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled. In our understanding, in-gap interferer just comes from other operators. The other factors, such as IMD5 and IMD7, image of LO and TX leakage all need to be considered. Hence, we need to consider total aggregated power of interferences in the gap between CC1 and CC2. In addition, how to measure the total aggregated power of all interferences in the gap between CC1 and CC2. We can depend on gNB triggering the UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Proposal 1: The total aggregated power of all interferences in the gap between CC1 and CC2 needs to be studied.

Proposal 2: GNB triggers UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Discussion on 2Rx applicability

According to the current spec, there is no clear specification about how many Rx should be implemented for DL NCCA. In our understanding, there is no need to limit the UE need to support 4Rx when UE is configured DL NCCA in the mandatory when configured in fragmented carrier operation. It is up to UE implementation. However, in order to simplify the workload in SI, RAN4 doesn’t need to discuss 2Rx. We can assume UE can support 4Rx when UE is configured DL NCCA in the mandatory 4Rx when configured in fragmented carrier operation, RAN4 doesn’t need to discuss 2Rx.

Proposal 3: Whether 4Rx or 2Rx is applicable to DL NCCA up to UE implementation. In SI stage, UE can support 4Rx when UE is configured DL NCCA in the mandatory 4RX when configured in fragmented carrier operation.

Discussion of impacts on DL performance for UE enabling fragmented carrier mode

PSD difference

Regarding performance evaluation based on existing ACS requirements, due to the limited time, we didn’t do simulation and testing evaluation. For ACS DL NCCA requirements, which are defined with the uplink configuration in accordance with Table 7.3A.2.2-1.For this uplink configuration, the UE shall meet the requirements for each sub-block are specified in clauses 7.5 for one component carrier. According to the Table 7.5-3 for case1 and Table 7.5-4 for case2, it is observed that the power of interferer is higher than 31.5dB compared to wanted signals. In our understanding, power spectral density (PSD) difference,  as we assumed here, is confined by the ADC dynamic range, since there will be nothing helped from the filter when we use a wide filter covering both carrier components. It is a challenge to control the AGC table, which will cause performance degradation. Depending on the waveforms of the in-gap interference, the peak to average ratio has to be reserved. If we use partially shared architecture, we can suppress the in-gap interference by 30dB. However, now we need to use fully shared architecture, we can bear the in-gap interference by 31.5-30=1.5dB. Considering the margin, our ADC resolution will reserve two bits of accuracy, it can bear an additional 12dB. And we need to consider the margin from 3dB to 6dB. Hence, 1.5dB+12dB-6dB=7.5dB. In addition, based on the above analysis, we need to keep some margin to consider the interference caused by IMD5 and IMD7, except the interference caused by adjacent channel operators. Generally speaking, in our understanding, we can use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point. If the operators cannot guarantee or control the power level of the interference by 6dB, we can judge by UE measurement and reporting.

Proposal 4: Use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point.

Signalling related

The SID [2] mentioned that the ability to semi-statically switch hardware resources means that UE is needed to switch between fully shared RX chain and separate RX chains for DL NCCA( two intra-band non-contiguous carriers). However, according to the last meeting agreements, when the one RF Rx chain cannot be enabled, UE can fall back to single carrier, which means that the UE doesn’t need to switch between fully shared RX chain and separate RX chains for DL NCCA. It is simplified for switching. In case of UE ability to switch, UE needs to report the supported CA configuration to the network by RRC signalling. An alternative way is to report the supported CA configuration to the network by separate RX chains with UE capability by default. When the network needs more DL throughput, the gNB triggers UE to measure the in-gap interference, if the UE has a fully shared RX chain, UE measures the in-gap interferences. If the UE measures that in-gap interference meet the threshold which is defined in the spec, the UE can report the new supported CA configuration to the network. If the UE measures the in-gap interference doesn’t meet the threshold, UE report the existing CA configuration by separate RX chains with UE capability.

Proposal 5: UE can report the new CA configuration to the network with the fully shared Rx chain together if UE measures the in-gap interference meets the threshold.

Impacts on DL RX requirements

In the last meeting, some companies discussed the impacts on DL Rx requirements. In our understanding, the purpose of reducing the number of RX chains is to enable more number of CCs. SNR dynamic range of fully shared architecture compared to partially shared architecture can be decreased. However, there should be no performance degradation. Otherwise, it can’t be called DL NCCA. Which means that the coverage range can be reduced, such as REFESENS and Max input level, but performance is retained. At the same time, the interferer for ACS and IBB/NBB requirements can be reduced but throughput can be retained. If the interference is too high, we can switch from fully shared architecture to partially shared architecture or switch from 2CC to single CC.

Proposal 6: Tolerable interference reduced may be considered.

Discussion on DL fragmented carriers in TDD bands

In the last meeting, there was an open issue to consider transition time if there’s one LO for the TDD band and study the feasibility. We need to discuss one LO or separate Los for both UL and DL for TDD. In our view, the total aggregation bandwidth for uplink and downlink are usually asymmetric. Generally speaking, the total downlink bandwidth is larger than the total uplink bandwidth. Hence, we support option2, separate LOs will be needed for UL and DL.

Proposal 7: We support option2, separate LOs will be needed for UL and DL in TDD bands.

Conclusions

In this paper, we provide our views on methods for reducing the number of UE RX chains, we have made the following proposals.

Proposal 1: The total aggregated power of all interferences in the gap between CC1 and CC2 needs to be studied.

Proposal 2: GNB triggers UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Proposal 3: Whether 4Rx or 2Rx is applicable to DL NCCA up to UE implementation. In SI stage, UE can support 4Rx when UE is configured DL NCCA in the mandatory 4RX when configured in fragmented carrier operation.

Proposal 4: Use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point.

Proposal 5: UE can report the new CA configuration to the network with the fully shared Rx chain together if UE measures the in-gap interference meets the threshold.

Proposal 6: Tolerable interference reduced may be considered.

Proposal 7: We support option2, separate LOs will be needed for UL and DL in TDD bands.

- Reference

- R4-2420519,WF on fragmented DL carriers study, Mediatek Inc. TSG-RAN WG4 113 meeting