3GPP TSG-RAN WG4 Meeting # 114	R4-2501212

Athens, GR, 17th – 21st February, 2025

Agenda item:	5.11.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on requirements maintenance for Rel-18 eFeRRM SCell activation

Document for:	Discussion

# Introduction

The core part and performance part of Rel-18 WI of even further RRM enhancement was completed with remaining issues captured in WF [1][2]. In this contribution, we further provide our views on remaining issues.

# Discussion

Regarding the applicable scenarios and requirements multiple SCells activation, the progress are summarized as follows:

R4-2410259 RAN4#111 Issue 1-1: Applicability of multiple SCell activation with L3 reporting on FR1 and FR2 band Agreement: - For FR1, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no contiguous active serving cell or there is no contiguous known SCell(s) to the unknown to-be-activated SCell on the FR1 band. - This condition will be added section 8.3.18. - For FR2, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no active serving cell or there is no known SCell(s) on the same band. - This condition will be added section 8.3.18. Issue 1-2: Multiple SCell activation requirement with L3 reporting based on issue 1-1 - Discuss directly in the CR. R4-2414020 RAN4#112 Issue 1-1-1: Requirement for the case when NOT all the unknown to-be-activated SCells have L3 report on FR1 band (some have L3 report but others not) Agreement: - Not define RAN4 requirement for the case where not all the unknown to-be-activated SCells are reported in the L3 reporting and cell detection is still needed on some of the unknown SCells on the same FR1 band. Issue 1-1-5: FR1 SCell activation enhancement considering the measurement period Agreement: - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

Based on the progress so far, the applicable scenario for multiple SCell activations are further reduced. For FR1, all to-be-activated SCells are unknown and have L3 report; For FR2, UE reports L3 report for at least one unknown SCell in the same FR2 band and there is no parallel to-be-activated FR1 SCell which needs Cell detection. Thus, for FR2, the requirements are the same as single FR2 SCell activation.

Observation 1: Based on the clarification on applicable scenario so far:

- For FR1, all to-be-activated SCells are unknown and have L3 report;

- For FR2, UE reports L3 report for at least one unknown SCell in the same FR2 band and there is no parallel to-be-activated FR1 SCell which needs Cell detection

Observation 2: For FR2, the requirements are the same as single FR2 SCell activation.

For FR1, since it is agreed in last meeting that RAN4 shall also consider the legacy principle that AGC is needed when the measurement period is above 2400ms, the requirements for FR1 multiple SCell activation shall be updated. Take legacy multiple SCell activation requirements as reference, which is shown as follows. Considering the agreed applicable scenario, the highlight cases are valid for this feature. It could be observed that when there is at least one to-be-activated SCell within the same band whose measurement period is larger than 2400ms, TFirstSSB_MAX_multiple_scells is needed for AGC or searching. For other cases, only fine time tracking is needed.

Tactivation_time_multiple_scells is the target SCell activation delay in millisecond in multiple SCell activation scenario. If the SCell is known and belongs to FR1 and the measurement period of the SCell being activated is equal to or smaller than [2400ms], Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 known Scell with the measurement period larger than [2400ms] but does not have any parallel to-be-activated SCell which is FR1 unknown SCell. - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown Scell - otherwise, TFirstSSB_MAX_multiple_scells + 5ms. If the SCell is known and belongs to FR1 and the measurement period of the SCell being activated is larger than [2400ms], Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs + 5ms, if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown Scell - otherwise, TFirstSSB_MAX_multiple_scells + Trs + 5ms

Observation 3: For FR1, when there is at least one to-be-activated SCell within the same band whose measurement period is larger than 2400ms, TFirstSSB_MAX_multiple_scells is needed for AGC or searching

Based on the analysis above, regarding how to update the requirements, it is complicated as there is dependency among different procedures. We use following figure to elaborate the time order.

Before UE can perform fine timing, following two procedure needs to be completed:

- Complete AGC

- L3 report and gNB configure TCI based on L3 report.

Thus, there should be a max (FirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC + 3ms + THARQ) before fine timing.

Besides, there is another parallel procedure which is to activate SP CSI-RS. The CSI-RS can only be activated after L3 report since gNB needs to configured TCI of the SP CSI-RS. Thus, for SCell activation, following two procedures needs to be completed:

- SP CSI-RS activation

- Fine timing

Thus, for the SCell activation requirement, we need another max (4ms + TL3 report + Tuncertainty_SP + 3ms+ THARQ, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms)

Fig.1 FR1 multiple SCell activation with L3 report.

Thus, the requirements are proposed to be formulated as follows:

For FR1 target SCell, if on the same band UE has at least one to-be-activated SCell with measurment period larger than 2400 ms, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + TL3 report + Tuncertainty_SP + THARQ + 3ms, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC + THARQ + 3ms) + TFineTiming + 2ms), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + TL3 report + Tuncertainty_RRC + TRRC_delay, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC+ THARQ + 3ms) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

Otherwise, Tactivation_time_multiple_scells is:

-	10ms + THARQ + TL3 report   + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if the semi-persistent CSI-RS is used for CSI reporting

-	7ms + TL3 report   + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if the periodic CSI-RS is used for CSI reporting

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17.

Proposal 1: The requirements for multiple SCell activation with L3 reporting to be updated as follows:

For FR1 target SCell, if on the same band UE has at least one to-be-activated SCell with measurment period larger than 2400 ms, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + TL3 report + Tuncertainty_SP + THARQ + 3ms, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC + THARQ + 3ms) + TFineTiming + 2ms), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + TL3 report + Tuncertainty_RRC + TRRC_delay, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC+ THARQ + 3ms) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

Otherwise, Tactivation_time_multiple_scells is:

-	10ms + THARQ + TL3 report   + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if the semi-persistent CSI-RS is used for CSI reporting

-	7ms + TL3 report   + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if the periodic CSI-RS is used for CSI reporting

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17.

The above proposed changes can also be found in our companied CR in this meeting.

# Conclusions

Observation 1: Based on the clarification on applicable scenario so far:

- For FR1, all to-be-activated SCells are unknown and have L3 report;

- For FR2, UE reports L3 report for at least one unknown SCell in the same FR2 band and there is no parallel to-be-activated FR1 SCell which needs Cell detection

Observation 2: For FR2, the requirements are the same as single FR2 SCell activation.

Observation 3: For FR1, when there is at least one to-be-activated SCell within the same band whose measurement period is larger than 2400ms, TFirstSSB_MAX_multiple_scells is needed for AGC or searching

Proposal 1: The requirements for multiple SCell activation with L3 reporting to be updated as follows:

For FR1 target SCell, if on the same band UE has at least one to-be-activated SCell with measurment period larger than 2400 ms, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + TL3 report + Tuncertainty_SP + THARQ + 3ms, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC + THARQ + 3ms) + TFineTiming + 2ms), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + TL3 report + Tuncertainty_RRC + TRRC_delay, max (TFirstSSB_MAX_multiple_scells, 4ms + TL3 report + Tuncertainty_MAC+ THARQ + 3ms) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

Otherwise, Tactivation_time_multiple_scells is:

-	10ms + THARQ + TL3 report   + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if the semi-persistent CSI-RS is used for CSI reporting

-	7ms + TL3 report   + max (THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if the periodic CSI-RS is used for CSI reporting

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17.

# References

[1] R4-2410259 WF for [111][205] NR_RRM_enh3_part1

[2] R4-2414020 WF for NR_RRM_enh3