3GPP TSG-RAN WG4 Meeting # 114	R4-2501221

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.25.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adaptation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN 4 discussed the RRM impacts of SSB adaption in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

In Oct RAN4 meeting, RAN4 send a LS [2] to RAN1 for clarification on time/frequency/spatial domain relation between for SSB adaptation. RAN1 discussed the question asked in the LS and provide answers as follows:

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Apart from the LS reply, RAN1 also achieved following agreements in last meeting:

Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Observation 1: Based on RAN1 progress so far, following scenario are supported:

- SSB adaption in SCell

- Time domain: SSB burst periodicity can be adapted

- Frequency domain: same frequency location before and after SSB adaption

- QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same

Proposal 1: RAN4 take following assumption for further discussion, which can be further updated accordingly with further RAN1 progress.

- SSB adaption in SCell

- Time domain: SSB burst periodicity can be adapted

- Frequency domain: same frequency location before and after SSB adaption

- QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same

## SSB adaptation for L1 measurement

For L1 measurement, RAN4 achieved following agreements in last meeting:

Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues.

One FFS point is which L1 requirements are subject to the requirements are transition. Based on legacy requirements, the transition requirements are only defined for BFD/CBD. For L1-RSRP and L1-SINR, there is no such requirements. For L1-RSRP and L1-SINR, there is some difference from BFD/CBD. The evaluation window for filtering is assumed for BFD/CBD measurement, which means instant determination is may not be expected. For L1-RSRP/L1-SINR, one valuable scenario is that SSB periodicity is adapted to a shorter one to have a timely beam reporting. Thus, for L1-RSRP/L1-SINR, we suggest to consider following two approach:

- UE to combine the SSB before and after adaption

- UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation.

Proposal 2: Only define transition requirements of SSB adaption for BFD/CBD. For SSB adaption for L1-RSRP/L1-SINR, RAN4 to considering following options:

- Option 1: UE to combine the SSB before and after adaption

- Option 2: UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation.

## SSB adaptation for L3 measurement

For SSB adaptation for L3 measurement, RAN4 send a LS to RAN2 to ask for the potential impacts on SMTC [3]. Based on the discussion in last meeting, RAN4’s common understanding is that the L3 measurement is still based on SMTC, which means UE is still expected to detect/measure Cells within SMTC. Regarding whether there is any impact on SMTC (i.e. new SMTC or SMTC adaptation as mentioned by companies) is up to RAN2 discussion. For instance, if gNB can guarantee that SSB can always be found in the configured SMTC, there is no any impacts on L3 measurement behaviour/requirements. If RAN2 decides to introduce SMTC adaptation, then RAN4 may needs to further discuss the potential transition requirements for SMTC adaptation.

Proposal 3: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition).

## SSB adaptation for SCell activation

Based on latest progress on SSB adaption scenario, it is almost the same as OD-SSB which is only for SCell, and at least SSB periodicity adaption is supported.

Observation 2: Based on latest progress on SSB adaption scenario, OD-SSB and SSB adaption share a lot of common characteristics (e.g. Only for SCell, and at least SSB periodicity adaption is supported)

For OD-SSB, one import scenario is OD-SSB with smaller SSB periodicity is triggered simultaneously with SCell activation to speed up the SCell activation procedure. Then for SSB adaption, the same scenario shall also be supported. For instance, the SSB periodicity is changed (e.g. from 40 ms to 20 ms) before SCell activation or at the same time as SCell activation, and it is straightforward that UE just uses the new SSB periodicity for SCell activation as agreed for OD-SSB. Because for this case, it is not clear how to configure the SMTC in servingCellConfig.

However, how to trigger the SSB adaptation is still under discussion in RAN1. It is not clear whether is triggered by MAC-CE as OD-SSB, or RRC signalling. Besides, similar as the discussion in OD-SSB, there is no clear motivation for SSB adaption during SCell activation.

Proposal 4: Similar as OD-SSB agreements, UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation.

- Detailed triggering on SSB adaptation depends on further RAN1 discussion.

- FFS whether to consider SSB adaption happens during SCell activation, which can be aligned with the progress in OD-SSB.

## Others

Based on the discussion in last meeting, companies also proposed to evaluate the impacts on TCI state switching. From our understanding, SSB is only used for timing tracking and L1-RSRP for an unknown TCI state. In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption. Thus, we think there is no RRM impacts on TCI state switching for SSB adaption.

Observation 3: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption.

# Conclusions

Observation 1: Based on RAN1 progress so far, following scenario are supported:

- SSB adaption in SCell

- Time domain: SSB burst periodicity can be adapted

- Frequency domain: same frequency location before and after SSB adaption

- QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same

Proposal 1: RAN4 take following assumption for further discussion, which can be further updated accordingly with further RAN1 progress.

- SSB adaption in SCell

- Time domain: SSB burst periodicity can be adapted

- Frequency domain: same frequency location before and after SSB adaption

- QCL: Same SSB index are QCLed when actually transmitted SSBs within a burst before and after SSB are same

Proposal 2: Only define transition requirements of SSB adaption for BFD/CBD. For SSB adaption for L1-RSRP/L1-SINR, RAN4 to considering following options:

- Option 1: UE to combine the SSB before and after adaption

- Option 2: UE to restart measurement using the SSB periodicity after adaptation from the time of adaptation.

Proposal 3: For SSB adaptation in L3 measurement, RAN4 assumes that UE is still expected to perform measurement based on SMTC. RAN4 to wait for RAN2 LS reply to decide whether there is RAN4 requirements impacts (e.g. whether to define requirements at transition).

Observation 2: Based on latest progress on SSB adaption scenario, OD-SSB and SSB adaption share a lot common characteristics (e.g. Only for SCell, and at least SSB periodicity adaption is supported)

Proposal 4: Similar as OD-SSB agreements, UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation.

- Detailed triggering on SSB adaptation depends on further RAN1 discussion.

- FFS whether to consider SSB adaption happens during SCell activation, which can be aligned with the progress in OD-SSB.

Observation 3: In realistic scenario, it is less likely that gNB trigger the TCI state switching simultaneously with SSB adaption.

# References

[1] R4-2420242 WF on RRM requirements for Netw_Energy_NR_enh_Part2

[2] R4-2416913 LS on SSB relation in OD-SSB and SSB adaptation on Scell

[2] R4-2416911 LS on SSB adaptation

[3] R4-2415663 Topic summary for [112bis] [216] Netw_Energy_NR_enh_Part2