3GPP TSG-RAN WG4 Meeting # 114	R4-2501230

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	7.27.2.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on Event triggered L1 measurement reporting.

Discussion

In legacy requirements for L3 measurement, it is specified that the event triggered measurement reporting delay shall be less than the L3-RSRP measurement period (Tidentify intra with index or T identify intra without index).

- 9.2.4.3 Event Triggered Reporting Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses 10.1.2.1 (RSRP for FR1), 10.1.3.1 (RSRP for FR2), 10.1.7.1 (RSRQ for FR1), 10.1.8.1 (RSRQ for FR2), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2). The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled. The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

In RAN4#113 meeting, companies discussed event triggered reporting delay for L1 reporting in LTM. One issue was identified. For Event LTM3 (Beam of candidate cell becomes amount of offset better than beam of serving cell), when the RS periodicity for candidate cell and serving cell are different, how to determine the evaluation periodicity for event triggered reporting. In last meeting, one high level proposal was agreed as below, however it needs further discussion on which cell the below “L1-RSRP measurement period” refers to.

- Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

When serving beam and the beam from candidate cell have different periodicities, e.g., the periodicity of serving beam is 80 ms, and that of the candidate beam is 20 ms, if the evaluation periodicity is determined based on the periodicity of the candidate beam (i.e., 20 ms), then UE shall evaluate the configured event every 20 ms. It means that UE is required to evaluate the event LTM3 4 times within 80ms and at each evaluation, the latest measurement result of candidate cell would compare with “old” measurement result of serving cell. The potential risk is that when the result of serving cell is low due to channel fading, it is possible that UE mistakenly counts 4 instances and triggers the report to NW. Therefore one reasonable approach is that the event instance evaluation shall be based on new results from both current and new beam. It means that the evaluation period is supposed to be the maximum RS periodicity of serving cell and candidate cell.

We know there is a related LS in R19 MIMO5 topic. Although LTM can wait for more progress in MIMO5, the difference is that the current beams and new beams come from the same cell, so if the same RS periodicity is assumed, no interaction between two different cells is needed in MIMO topic. In LTM, things are different. Restriction on same RS periodicity requires some information exchange between serving cell and candidate cells.

Proposal 1: The event triggered measurement reporting delay shall be no larger than

- the L1-RSRP measurement period on the cell corresponding to the event (for Event LTM2, 4, 5).

- the maximum L1-RSRP measurement period on the cells corresponding to the event (for Event LTM3).

Another open issue regards with TTT.

- Issue 2-6: impact of time-to-trigger (TTT) on RRM <Candidate options> - Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi) <Recommended WF> - Continue discussion.

TTT is used for avoiding sudden change and it is a layer3 level configuration. In legacy L3 measurement requirements, an exception is added for L3 filter “When L3 filtering is used an additional delay can be expected.” and no TTT is considered. We think event triggered L1-RSRP measurement for LTM can follow the similar way.

Proposal 2: For TTT (time to trigger) is not considered in L1 event-triggered reporting delay requirements.

- Issue 2-10: whether and how to coordinate with R19 MIMO <Candidate options> - Option 1: (MTK) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events. - Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple) - Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia) <Recommended WF> - Continue discussion.

Although events for LTM and MIMO can be configured for the same beams and same cell, LTM and MIMO are the different features. From UE implementation perspective, to simplify the UE capability design, it is better to define separate capabilities for MIMO and LTM.

Proposal 3: Define separate capabilities for MIMO and LTM.

- Issue 2-11: L1 filtering assumption in event evaluation <Candidate options> - Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW) - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC) <Recommended WF> - Continue discussion.

It is agreed in RAN2 that it’s up to RAN1 whether the specified L1 filtering is needed or to leave it to UE implementation. RAN1 made the below agreement in RAN1#118bis meeting that no L1 specified filtering for time and frequency domain is introduced for gNB scheduled measurement reporting.  It means that no L1 filter related parameter is configured from network and UE is not required to use it. In addition, TTT is introduced in LTM and TTT can leverage the effect from signal quality fluctuation.

From RAN4 perspective, as one shot sample is applied for CSI-RS based L1-RSRP in serving cell, to ensure the fairness between the RSRP level from candidate cells and that of serving cell, L1 filtering on neighbour cell is NOT expected. Moreover the gain from early L1 measurements is to enable network quickly acquire the measurement results, however the introduction of L1 filter would vanish the gain.

RAN1 Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 4: From RAN4 perspective, no L1 filtering is needed for event triggered L1 report.

Another open issue is the impact of UL resource waiting time [2].

- Issue 2-15: Impact of UL resource waiting time. <Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. <Recommended WF> - Continue discussion.

RAN2 agreements The legacy SR procedure for resource allocation is the baseline to send the event triggered L1 measurements MAC CE. NW can configure a dedicated SR configuration for MR MAC CE transmission.

Also the legacy L3 measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. Although dedicated SR is introduced for MR MAC CE transmission in RAN2 LTM, the dedicated SR resource are still periodic. It means UE may need to wait for available UL resource by using either legacy SR or dedicated SR. So we don’t observe a need to have a different definition for event triggered L1 measurement reporting delay due to dedicated SR.

Proposal 5: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, regardless of legacy SR or dedicated SR.

Conclusion

The contribution provides analysis on Event triggered L1 measurement reporting. The following proposals are proposed:

Proposal 1: The event triggered measurement reporting delay shall be no larger than

- the L1-RSRP measurement period on the cell corresponding to the event (for Event LTM2, 4, 5).

- the maximum L1-RSRP measurement period on the cells corresponding to the event (for Event LTM3).

Proposal 2: For TTT (time to trigger) is not considered in L1 event-triggered reporting delay requirements.

Proposal 3: Define separate capabilities for MIMO and LTM.

Proposal 4: From RAN4 perspective, no L1 filtering is needed for event triggered L1 report.

Proposal 5: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, regardless of legacy SR or dedicated SR.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple