3GPP TSG-RAN WG4 Meeting # 114	R4-2501232

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	7.27.2.3

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

Regarding the third objective conditional intra-CU LTM, according to the latest revised WID, both conditional LTM and subsequent conditional LTM are to be supported. The analysis on the RRM requirements for conditional LTM and subsequent conditional LTM are provided in the followings.

Conditional LTM

One open issue regarding L3 measurement based conditional LTM was widely discussed during last RAN4 meeting [2].

Issue 1-1-2: Measurement requirements on conditional LTM < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting.

From RAN2 perspective, triggering condition of conditional LTM can be based on L3 measurement.

RAN2 agreements

The triggering condition of conditional LTM can be based on L3 measurement.

In the following, the analysis is to be provided from execution conditions, early TCI state activation and PDCCH order RACH perspective in C-LTM procedure in FR1.

Event LTM3 and LTM5 are defined as L1 LTM events in RAN2.

-CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

-CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2

For FR1 UE, as the RX antenna is omni-directional, the antenna reception gain has no difference for L1 and L3 measurements. However as L3 measurement result shall average multiple physical samples, the L3 measurement delay would be larger than that of L1 measurement. Then the event evaluation based on L3 measurement may be slower than L1 measurement. However from feasibility perspective, either L1 or L3 measurement can trigger conditional handover.

For FR1 UE, to perform early candidate TCI state activation, UE shall report beam information to network. In conditional LTM procedure for FR1, the SSB index or CSI-RS resource index can be reported by L3 measurement report or L1 measurement report. Although the reported beam index which is selected by L3 measurement may be different from the beam selected by L1 measurement, each scheme has its own advantage, the impact on mobility performance is hard to compare. Anyway from configuration and implementation perspective, it is feasible to perform L3 based early TCI state activation in FR1 in C-LTM.

Moreover the L3 based PDCCH order PRACH in FR1 has been supported in R18 LTM.

- 6.2.2C PDCCH ordered Random Access for LTM The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

Based on the above analysis, it is observed that in FR1 it is feasible to support L3 based C-LTM. The L3 based condition evaluation period may be slower than L1 based and RAN2 may need some clarification on L3 beam specific LTM events/conditions.

Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM.

Conditional LTM delay

Legacy conditional handover delay is defined as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. R18 LTM cell switch is defined as DLTM = Tcmd  + TLTM-interrupt ( where TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms). In our understanding, conditional LTM delay can take legacy CHO delay as a reference and update Tinterrupt according to TLTM-interrupt for LTM cell switch.

Proposal 2: Conditional LTM delay can take legacy CHO delay as a reference and update Tinterrupt according to TLTM-interrupt defined in R18 LTM cell switch delay.

In legacy CHO delay, the starting point is from UE receives a RRC message implying conditional handover. After UE successfully decodes the RRC signalling, UE waits for the time instance the conditional conditions are met and then performs handover to the target cell.

For C-LTM, LTM cell switch related condition configurations are also signalled by RRC. So it is straight forward that the starting point of conditional LTM is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 3: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

In last RAN4 meeting it is agreed that the ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell [2]. So the ending point issue is closed.

In legacy CHO delay, the component TEvent_DU is the delay uncertainty from UE decodes a CHO command to the moment when the condition is met at measurement reference point (i.e., real channel quality satisfies the condition of CHO).

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover

RAN2 agreed to support early TCI state activation/deactivation and UL sync on candidate cells during conditional LTM procedure. In last RAN2 meeting, some progresses are made accordingly.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

Regarding the sequence of early DL (e.g., TCI state activation), UL sync (PDCCH ordered based RACH) and execution condition evaluation, we think early DL and UL sync procedures on candidate cells shall be performed before UE evaluating the execution conditions. Taking PDCCH order RACH as an example, in Rel-18 network triggered LTM, both PDCCH order PRACH and LTM cell switch command are sent from gNB, so gNB can totally control the order of the two procedures. However, in conditional LTM, the gNB does not know when the condition is met at UE side. If gNB sends a PDCCH order very early, the TA may be out of date when the UE triggers cell switching later. Conversely, there is also a case that when the conditions are met in UE side, UE starts to switch to the target cell, however at the same time, network is triggering PDCCH ordered RACH, then UE failed to receive the PDCCH order. To our knowledge, RAN2 think it is left to gNB implementation to decide when to trigger PDCCH order PRACH, as gNB has some information of UE measurement reports.  It means that if gNB failed to trigger the PDCCH order PRACH in time ( e.g.,  UE failed to receive the command and has already switched to other cell), UE will still perform RACH based cell switch.

Similar, early TCI state activation before evaluating execution conditions is the similar status as PDCCH ordered RACH.

As per the existing definition of delay uncertainty of TEvent_DU (from decoding CHO command to a condition is met),  we think TEvent_DU can cover both cases in C-LTM: 1) UE first completes DL sync and UL sync and then performs execution condition evaluation, 2) neither DL sync nor UL sync is successfully performed before condition is met.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover

Proposal 4: TEvent_DU can be reused in conditional LTM delay, as it can cover both cases:

1) UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

2) neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

In legacy conditional handover, Tmeasure is the time duration when the condition event is really met at UE side (i.e., UE realize the condition of CHO is met).

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

As both L1 measurement and L3 measurement can be used to trigger conditions of C-LTM, Tmeas can be the L3 measurement period or L1 measurement period. Whether to support triggering condition based on  CSI-RS L3 measurement depends on RAN2 progress.

Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on CSI-RS based L3-RSRP measurement.

In legacy CHO delay, TCHO_execution is the UE execution preparation time for conditional handover. This time duration originally called TRRC_2 for RRC processing time of the target cell. We think TCHO_execution can be reused in conditional LTM. TCHO_execution can be up to 10ms.

Proposal 6: TCHO_execution can be reused in conditional LTM.

In legacy CHO delay, Tinterrupt comprises Tprocessing + TIU + T∆ + Tmargin. In legacy CHO procedure, interruptions happen from UE starts to execute CHO to target cell. However in C-LTM, interruptions can also happen during early RACH procedure. Therefore from terminology perspective, we suggest the term Tinterrupt  is not used when defining conditional LTM delay.

Proposal 7: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions can also happen in early PDCCH RACH procedure.

In C-LTM, upon condition is met, after UE finish related operations and leave the source cell (TCHO_execution), the steps which are defined in R18 LTM cell switch delay, i.e., TLTM-processing, Tfirst-RS , TRS-proc , TLTM-IU, shall be followed.

In R18 LTM cell switch delay,

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the uncertain time in transmit the first UL transmission.

Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM.

In summary, the C-LTM delay can be depicted as below equation:

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 9: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Subsequent conditional LTM

The requirements of subsequent CPA and subsequent CPC have been specified in R18. The delay for subsequent CPA/CPC is alike CHO delay in some extent. The difference is the starting point. In subsequent CPA/CPAC the starting point is from the time UE transmits RRC Reconfigurationcomplete message for the previous PSCell addition or change. For subsequent conditional LTM, in our understanding either RACH based or RACH-less cell switch, the starting point of the subsequent conditional cell switch delay can also reuse the point that UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 10: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: In FR1 it is feasible to support L3 measurement based C-LTM.

Proposal 2: Conditional LTM delay can take legacy CHO delay as a reference and update Tinterrupt according to TLTM-interrupt defined in R18 LTM cell switch delay.

Proposal 3: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 4: TEvent_DU can be reused in conditional LTM delay, as it can cover both cases:

1) UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

2) neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

Proposal 5: In C-LTM delay, Tmeas is the L1 measurement period

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

- If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

- FFS on CSI-RS based L3-RSRP measurement.

Proposal 6: TCHO_execution can be reused in conditional LTM.

Proposal 7: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions can also happen in early PDCCH RACH procedure.

Proposal 8: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM.

Proposal 9: C-LTM delay can be depicted as

DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

Proposal 10: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 11: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay, as most components in conditional LTM delay can be reused for subsequent C-LTM.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom