3GPP TSG-RAN WG4 Meeting #114	R4-2501266

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on UE CLI measurement for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for UE CLI measurement.

- Measurement methods

- Measurement resources

- Measurement period

- Side condition

- Measurement reporting

- Scheduling restriction

- Measurement restriction

- Report mapping

- Requirement applicability

In this paper, we will provide our views on RRM requirements for UE CLI measurement in SBFD.

Discussion

Measurement methods

In RAN1#119, it is agreed to support Method #3: UE measures RSSI within UL subband.

Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

In our view, RAN4 should define RRM requirements for Method #3. One key issue in defining requirements is the assumption on measurement timing. There are 2 options to be considered:

- Option 1: following DL timing, same as Method #1.

- Option 2: applying constant offset relative to DL timing, same as Method #2.

Considering Method #3 is measuring UL subband to identify existence of direct interference source, which is similar to Method #2, we believe it is reasonable to assume same measurement timing as Method #2.

For measurement period, RAN4#113 agreed that measurement period requirements for L1-CLI-RSSI are defined based on 1 sample. Although this agreement was made assuming Method #1, we see the rationales also apply for Method #3. For scheduling and measurement restriction, however, we understand it would be more similar to Method #2 rather than Method #1 as to be discussed in section 2.6.

Proposal 1: RAN4 to define L1 CLI measurement requirements for Methods #3.

- Measurement timing is same as Method #2 (L1-SRS-RSRP).

- Measurement period is based on single sample.

Measurement resources

In RAN1#119, the following are agreed regarding reporting of L1-CLI-RSSI.

Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19. Conclusion For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Based on RAN1 agreement, only single wideband reporting is supported for L1-CLI-RSSI. The remaining issue is whether L1-CLI-RSSI requirements would be impacted by the resource configuration (resource types #1 and #2).

In our view, RSSI measurement simply reflects the total received power on the indicated time and frequency resources, and the measurement performance (delay or accuracy) does not depend on the RB of measurement resource. Even with resource types #2, UE should be able to measure wideband RSSI from 2 DL subbands from a single resource occasion.

Therefore, we suggest RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource configuration.

Proposal 2: RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource types.

Measurement period

For L1-SRS-RSRP, RAN4 discussed whether to define requirements based on intra-cell or inter-cell scenario, and has sent LS [2] to RAN1 for clarification. In [3], RAN1 provided the following feedback.

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

RAN1 indicates intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement. On the other hand, RAN4 agreed following 2 options for further discussion.

- For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement

For intra-cell scenario, our simulation results show that 3-sample does not provide much gain over 1-sample. The gain in accuracy is ~ 0.2dB for all cases. Based on this observation, and the fact that main motivation to support L1 CLI measurement is to enable fast CLI status reporting, we support to define a single set of measurement period based on [1] sample for L1-SRS-RSRP. This would also simplify the specification work as well as UE implementation.

Proposal 3: For L1-SRS-RSRP, RAN4 to define a single measurement period based on [1] sample.

Side condition

On timing offset condition for L1-SRS-RSRP, RAN4#112-bis agreed 2 sets of conditions for simulation evaluation.

- Case 1: 4.67us for FR1, 3.67us for FR2

- Case 2: 1.67us for FR1, 0.67us for FR2

As RAN1 indicates that intra-cell scenario can be prioritized for the requirements of L1-SRS-RSRP measurement, which corresponds to Case 2, we suggest to define L1-SRS-RSRP accuracy requirements based on Case 2 of timing offset. It is also possible to define another set of accuracy requirement based on Case 1 of timing offset, but considering the performance would be worse than R16 L3 CLI accuracy, we do not see clear points to define such requirements.

Proposal 4: RAN4 to adopt Case 2 as the timing offset condition for L1-SRS-RSRP accuracy requirements.

In last meetings, companies have discussed aspects of Es/Iot and SRS PRB number.

On SRS PRB number, we suggest RAN4 to further discuss which between 24 RB and 48 RB is to be used as the side condition. The main question is whether defining requirements for >= 48 RB is enough, or in other words, how likely the BW of UL subband will be < 48 RB.

On Es/Iot, we suggest to re-use the R16 side condition of 1dB. Based on our results, the gain of 3dB over 1dB Es/Iot is not much, so it is meaningful to keep using 1dB to ensure the requirement coverage.

Proposal 5: RAN4 to stick to 1dB as the on Es/Iot condition, and further down select between 24 and 48 RB as the on SRS BW condition for L1-SRS-RSRP accuracy requirements.

Measurement reporting

In WID it is clarified that aperiodic reporting will be supported for L1 CLI measurement, so RAN4 should define corresponding measurement reporting requirements. In our view, the reporting requirements for L1-RSRP in clause 9.5.3.3 can be used as baseline since both are L1 measurements.

#### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

RAN4 can further discuss the reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 6: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Scheduling restriction

To define scheduling restriction for L1 CLI measurement, we first need to consider the measurement timing, i.e. if the measurement is with a constant offset relative to DL timing, then the restricted symbols include not only the symbols for the measurement resource, but also 1 or 2 symbols before the measurement resource. The restriction occurs no matter whether the measurement resource is QCL-ed with PDCCH/PDSCH or not.

In this sense, if our Proposal 1 (applying a constant offset relative to DL timing for L1-CLI-RSSI Method #3) is agreeable, L1-SRS-RSRP and L1-CLI-RSSI Method #3 should have same scheduling restriction, and R16 requirements can be re-used as baseline for both FR1 and FR2. The exact number of restricted symbols is depending on the side condition on timing offset (addressed by our Proposal 4), which can be discussed after side condition on timing offset is concluded.

For L1-CLI-RSSI Method #1, as the measurement is based on DL timing, scheduling restriction applies only in symbols for the measurement resource. For UL, scheduling restriction always applies because UE cannot transmit while performing DL measurement. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

Proposal 7: RAN4 to define scheduling restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline for both FR1 and FR2, and exact number of restricted symbols is FFS.

- For L1-CLI-RSSI Method #1, scheduling restriction applies only in OFDM symbols on which the UE performs the measurements. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

Measurement restriction

RAN4 agreed to adopt the same principle as for R16 L3 CLI measurement to define measurement restriction for L1 CLI measurement.

- For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements can be re-used as baseline because the CLI measurement cannot be performed at same time as other measurement due to different measurement timing.

- For L1-CLI-RSSI Method #1, as the measurement is based on DL timing and in DL subband(s), UE should be able to perform this measurement in parallel with other measurement, e.g. in FR1. We suggest to define measurement restriction such that the requirements for L1 CLI measurement do not apply only when UE cannot perform CLI measurement and other measurement at same time. The detailed condition where measurement restriction do not apply can be further discussed.

Proposal 8: RAN4 to define measurement restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline.

- For L1-CLI-RSSI Method #1, measurement restriction applies only when UE cannot perform CLI measurement and other measurement at same time.

Report mapping

RAN1#119 agreed the following regarding measurement reporting for L1 CLI measurement.

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

Based on RAN1 agreement, we understand RAN1 will discuss the report mapping for L1 CLI measurement as this will impact how the UCI payload is generated for the L1 reporting. To avoid duplicated discussion, we suggest RAN4 not to further discuss this issue unless triggered by RAN1.

Proposal 9: RAN4 not to discuss report mapping for L1 CLI measurement unless triggered by other WG.

Requirement applicability

In R16, following applicability is defined for SRS-RSRP and CLI-RSSI.

The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resouce is configured within active BWP

For L1-SRS-RSRP and L1-CLI-RSSI, RAN1#118 agreed following.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband FFS: Method#3: UE measures RSSI within UL subband

Given the RAN1 agreement, R16 applicability can be directly re-used.

Proposal 10: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Conclusions

In this paper we provided provide our views on RRM requirements for UE CLI measurement in SBFD.

Proposal 1: RAN4 to define L1 CLI measurement requirements for Methods #3.

- Measurement timing is same as Method #2 (L1-SRS-RSRP).

- Measurement period is based on single sample.

Proposal 2: RAN4 to confirm that measurement delay and accuracy requirements for L1-CLI-RSSI are agnostic to measurement resource types.

Proposal 3: For L1-SRS-RSRP, RAN4 to define a single measurement period based on [1] sample.

Proposal 4: RAN4 to adopt Case 2 as the timing offset condition for L1-SRS-RSRP accuracy requirements.

Proposal 5: RAN4 to stick to 1dB as the on Es/Iot condition, and further down select between 24 and 48 RB as the on SRS BW condition for L1-SRS-RSRP accuracy requirements.

Proposal 6: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 7: RAN4 to define scheduling restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline for both FR1 and FR2, and exact number of restricted symbols is FFS.

- For L1-CLI-RSSI Method #1, scheduling restriction applies only in OFDM symbols on which the UE performs the measurements. For DL, scheduling restriction applies when the measurement resource is not type-D QCL-ed with the DL reception in FR2.

Proposal 8: RAN4 to define measurement restriction for L1 based UE-to-UE CLI measurement

- For L1-SRS-RSRP and L1-CLI-RSSI Method #3, R16 requirements are re-used as baseline.

- For L1-CLI-RSSI Method #1, measurement restriction applies only when UE cannot perform CLI measurement and other measurement at same time.

Proposal 9: RAN4 not to discuss report mapping for L1 CLI measurement unless triggered by other WG.

Proposal 10: L1 based UE-to-UE CLI measurement requirements apply provided that

- when L1-SRS-RSRP measurement resource is fully confined within BW of DL active BWP

- when L1-CLI-RSSI measurement resource is configured within BW of DL active BWP

Reference

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN4

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN1