3GPP TSG-RAN WG4 #114	R4-2501273

Athens, Greece, 17 - 21 February, 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM requirements for Rel-19 ATG

Agenda Item:	7.12.3

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-Cell, we provide our analysis on the potential RRM impacts to support CA in case by case manner in the following part.

UE antenna type for inter-band CA

In the last meeting, we have an online understanding type, which is shown below:

FFS - Proposal 1: To support Case 4 antenna type, MRTD, reception power difference for two bands, and DL frequency separation should be restricted to avoid critical receving performance degradataion, e.g., conditions for intra-band NCCA operation. - Proposal 2: Confirm the understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible. - Proposal 3: Wait for more progress from RF session. - Proposal 3-1: If the implementation of Case 4 is not excluded in RF session, RRM requirement and related UE capability should be defined, details can be discussed later.

In our view, the UE antenna type in Case 4, or other similar antenna combinations, was not discussed in the RF session. All RF discussions regarding the n3 and n39 band combination have assumed an omnidirectional antenna for the UE. While using an antenna array in FR1 is technically feasible, sharing the antenna array still requires further study. If the RF can continue discussing Case 4, RRM can then proceed with addressing the requirements for this case.

Proposal 1: Wait for further RF conclusion/clarification on antenna type for inter-band CA.

MRTD

In the last meeting, we have an online understanding type, which is shown below:

FFS - Option 1: - For the MRTD except Case#4: 33µs. Further discuss for case 4. - Option 2: - For the MRTD except Case#4: 33µs. - If case 4 is not excluded based on RF discussion: 3µs. - Option 3: - For the MRTD for all cases: 33µs.

For inter-band CA, UE has separate RF chains. We see no implementation issue to maintain the 33 us as the MRTD. This requirement supports that UE has separate antenna panels on each band. Whether to define two MTRD requirements or redefine the MRTD requirement is up to the discussion conclusion of the UE antenna type issue.

Proposal 2: For the MRTD except Case#4: 33µs. Further discuss for case 4.

Interruption requirement

In the last meeting, there were some FFS bullets which are shown below:

Agreement: - Introduce following requirements for R19 ATG - Interruptions at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions at direct SCell activation - Interruptions at fast SCell activation - Interruptions during measurements on deactivated SCC - Interruptions due to SCell dormancy - Interruptions due to UE-specific CBW change - Interruptions due to Active BWP switching Requirement - Interruptions when identifying CGI of an NR cell with autonomous gaps - Not introduce following requirements for R19 ATG - Interruptions at UL carrier RRC reconfiguration - Interruptions at inter-frequency SFTD measurement - Interruptions at NR SRS carrier based switching - Interruptions due to PUCCH SCell activation/deactivation - Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18] - FFS whether following requirements need to be introduced - Interruptions at NR SRS antenna port switching - Further discuss the potential impact from UE antenna types - Further discuss how to reflect the interruption requirement for R19 ATG

NR SRS antenna port switching is used for channel estimation, allowing the UE to transmit SRS from different transmission antennas to the NW when Tx antenna number is less than Rx antenna number. Based on the results reported from these different antennas, the network can improve the downlink transmission. This SRS antenna port switching can also be used in ATG, and when applied, interruptions may occur and need to be introduced.

Proposal 3: Interruptions at NR SRS antenna port switching is able to be introduced in ATG.

In the last meeting, it was concluded that the interruption requirement for SRS carrier-based switching is not included. However, in the ATG scenario, this interruption is still necessary. SRS carrier-based switching is primarily used in downlink CA-only scenarios, particularly when the SCell operates on a TDD band. This approach allows the UE to send SRS on both the PCell and SCell, enabling the network to assess the SRS weight of the SCC, which in turn enhances downlink transmission.

The current discussed ATG band combination is n3+n39 which is an FDD band and a TDD band. This band combination is matched to the SRS carrier based switching applicability.

Proposal 4: The NR SRS carrier based switching interruption requirement can be introduced.

The discussion so far has focused on ATG CA enhancement for bands n3, n39, and n79. Therefore, the interruption requirements can be applied to these bands. However, for other bands, further discussions may be needed.

Proposal 5: The interruption requirements defined are applicable to the bands: n3, n39, and n79. If new bands have been introduced the interruption requirements need to be redefined.

SCell Activation and Deactivation Delay

In the last meeting, we already discussed the SCell activation well. The agreement is shown below:

FFS - Proposal 1: The SCell activation delay requirements will not be impacted by ATG UE antenna type. - Proposal 2: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type. - Proposal 3: SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

We have some concerns regarding the SSB-less requirements related to SCells. The below text is the Rel-18 38.133[2] SSB-less requirement definition. The highlighted section indicates that only type-A and type-C QCL relationships are applicable under the SSB-less SCell activation conditions. This indicated that the legacy requirement can not provide the beam-related information QCL.

For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell., and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

Given that the ATG UE might employ an array-type antenna in FR1, and the legacy SSB-less requirement does not address beam-related issues. Consequently, we believe that the SSB-less requirement is only applicable to the following antenna combination:

      - ATG UE with omnidirectional antenna on the bands of PCC and SCC.

      - ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

Proposal 6: Inter-band SSB-less SCell activation requirements can apply to the following antenna combination:

1.	ATG UE with omnidirectional antenna on the bands of PCC and SCC.

2.	ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

SCell Addition/Activation mechanism

In the last meeting, companies proposed to discuss the SCell addition/activation mechanism.

FFS - Proposal 1: Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements. - Proposal 2: Further consider distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition - Proposal 3: Should not consider distance triggered SCell addition and/or activation; existing activation methods are sufficient

In the discussion, the SCell activation or addition based on location information is feasible but we still need to consider the UE antenna type affection.

For instance, when the ATG UE equipment with the antenna array the location information matches the SCel activation condition the NW will activate the SCell. However, the beam of SCC may not aim at the direction of the NW SCell that is activated and needs to justify the beam direction from the original SCell to the new SCell direction.

On the other hand, if the UE equipment with the sharing panel antenna array this may be easy to achieve. The reason is that the PCC and SCC use the same panel and the beam direction of PCC and SCC is almost the same because of the co-located PCell and SCell. In this situation, the NW is able to activate SCell when the ATG UE is matched to the distance-triggered condition.

Proposal 7: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type.

Deactivated SCell measurement

The following is the conclusion in the WF[1]:

Agreement: - beam sweeping factor N1 is needed. - FFS on L3/L1 measurement sharing factor.

There are 5 types of UE antenna in ATG Rel-19 discussion. Due to variations in UE types, including omnidirectional and array antennas, the deactivated state does not have the L1 measurement, so we think the L3/L1 measurement sharing factor is not needed. If the RF have a conclusion on Case 4 this can be defined.

Proposal 8: For the L3/L1 measurement sharing factor Klayer1_measuremen, define or not this after the RF session has a conclusion on Case 4.

Conclusion

Proposal 1: Wait for further RF conclusion/clarification on antenna type for inter-band CA.

Proposal 2: For the MRTD except Case#4: 33µs. Further discuss for case 4.

Proposal 3: Interruptions at NR SRS antenna port switching is able to be introduced in ATG.

Proposal 4: The NR SRS carrier based switching interruption requirement can be introduced.

Proposal 5: The interruption requirements defined are applicable to the bands: n3, n39, and n79. If new bands have been introduced the interruption requirements need to be redefined.

Proposal 6: Inter-band SSB-less SCell activation requirements can apply to the following antenna combination:

1.	ATG UE with omnidirectional antenna on the bands of PCC and SCC.

2.	ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

Proposal 7: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type.

Proposal 8: For the L3/L1 measurement sharing factor Klayer1_measuremen, define or not this after the RF session has a conclusion on Case 4.

References

[1] R4-2420237 WF on RRM requirements for NR_ATG_enh

[2] 3GPP TS 38.133 V18.8.0