3GPP TSG-RAN WG4 Meeting #114	R4-2501285

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	7.10.3.4

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on those issues as follows.

# Discussion

#### Issue 1-1-2: Measurement(SSB index detection requirements):

Wayforward: - Option 1: - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Option 2: Two sets of requirements - Set 1: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to will be extended by [4] and [3] for intra-frequency and inter-frequency cases respectively. - Set 2: smaller delay with higher SNR side condition. - FFS on the exact number of delay to be extended base on the simulation results under the same simulation assumptions for TN less than 5MHz [R4-2306400] - FFS on the separate UE capability to support set 2 requirements

As per the discussions in the last meeting, we understand the concern about same requirements on NTN and TN, where NTN scenario may have different (typically simpler and less NLOS based on hypothesis) channel characteristics and the NTN UE has the orbit prediction capability, which are other than TN. However, it may be difficult to build another set of particular requirements for NTN at this moment, and it’s strange and unfair that only the SSB index detection delay is taken into account. It needs the proof by concrete simulations. We lean toward keeping the requirements the same as TN.

Proposal 1:  Support Option 1.

# Conclusion

Proposal 1:  Support Option 1.

# References

- R4-2420091, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi