3GPP TSG-TRAN4 Meeting # 114	R4-2501286

# Athens, Greece, 17th February - 21st February

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_IoT_NTN_req_test_enh-Core) CR for TS 38.133 on Link Recovery Procedures for less than 5MHz NTNNR_NTN_enh-Core) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add requirements for less than 5MHz NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PDCCH transmission parameters for beam failure instance for less than 5MHz NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No valid requirements for less than 5MHz NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5C.1, 8.5C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

## 8.5C	Link Recovery Procedures for Satellite Access

### 8.5C.1	Introduction

The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [3] in order to detect beam failure on PCell and the UE is configured with only PCell, which is served by satellite access node (SAN).

The RS resource configurations in the set q0 on PCell can be periodic CSI-RS resources and/or SSBs.

On each RS resource configuration in the set q0, the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell beams.

The threshold Qout_LR is defined as the level at which the downlink radio level link of a given resource configuration on set q0 cannot be reliably received and shall correspond to the BLERout = 10% block error rate of a hypothetical PDCCH transmission. For SSB based beam failure detection, Qout_LR_SSB is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5.2.1C-1, and table 8.5.2.1C-2 for a UE operating on a cell with less than 5 MHz BW. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in table 8.5C.3.1-1, and table 8.5.2.1C-2 for a UE operating on a cell with less than 5 MHz BW.

Upon request the UE shall deliver configuration indexes from the set q1as specified in TS 38.213 [3] , to higher layers, and the corresponding L1-RSRP measurement provided that the measured L1-RSRP is equal to or better than the threshold Qin_LR, which is indicated by higher layer parameter rsrp-ThresholdSSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained from an SSB. The UE applies the Qin_LR threshold to the L1-RSRP measurement obtained for a CSI-RS resource after scaling a respective CSI-RS reception power with a value provided by higher layer parameter powerControlOffsetSS. The RS resource configurations in the set q1 can be periodic CSI-RS resources or SSBs or both SSB and CSI-RS resources.

### 8.5C.2	Requirements for SSB based beam failure detection

#### 8.5C.2.1	Introduction

The requirements in this clause apply for each SSB resource in the set q0 configured for a serving cell, provided that the SSB configured for beam failure detection is actually transmitted within the UE active DL BWP during the entire evaluation period specified in clause 8.5C.2.2.

Table 8.5.2.1C-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Bandwidth (PRBs) | 24 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

Table 8.5.2.1C-2: PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5 MHz BW

| Attribute | Value for BLER |
| --- | --- |
| Channel BW | 3 MHz |
| [DL Transmission BW] | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 Note 1 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote2 |
| NOTE 1: PDCCH puncturing as defined in TS 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |

End of Change 1