3GPP TSG-RAN WG4 Meeting #114	R4-2501294

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	RRM requirements for NR_NTN_enh

Agenda Item:	5.21.3

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #113, the conclusions on RRM requirements for NR NTN enhancement were listed in WF [1]. We herein in this contribution present our views and proposals on the remaining open issues.

# Discussion

### Issue 5-2-S: Soft Satellite switch

FFS: A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17.

The concern on soft satellite switch by a UE not supporting parallelMeasurementWithoutRestriction-r17 is that the UE may not have chance to be scheduled since NW could not accurately identify the symbols for data scheduling for the UE, and at the end the soft satellite switch isn’t efficient as expected.

In our opinion, there are two sides to the problem.

- On one hand, requiring parallelMeasurementWithoutRestriction-r17 could increase the effectiveness of soft switch for UEs, which are capable of soft satellite switching.

- On the other hand, there exist some UEs not supporting parallelMeasurementWithoutRestriction-r17 for any reason, the requirement precludes them from supporting soft satellite switching and further makes soft switch more uncommon in real deployment.

Proposal 1: No strong view, we can support it since it may bring an efficient soft switch.

# Conclusion

Proposal 1: No strong view, we can support it since it may bring an efficient soft switch.

# References

- R4-2420235, WF on RRM requirements for NR_NTN_enh, Qualcomm