# 3GPP TSG-RAN WG4 Meeting #114	R4-2501302

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.26.4.1

Source: 	Ericsson

Title:	Simulation assumption on LP-WUS RRM measurement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following further agreement for simulation assumption[1]. In this contribution, we continue to share our views on simulation assumption on LP-SS RRM measurement.

Issue 2-1-3: Time/frequency error Agreement: Table 2: Cell-specific parameters Parameter; Unit; Cell 1; Cell 2. Frequency offset relative to UE frequency reference; Hz; OFDM based receiver [5] ppm OOK based receiver [510 20] ppm; N/A. Timing error; Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: company report - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4. Issue 2-1-4: Accuracy baseline for simulation Agreement: Candidate absolute baseband accuracy to derive the number of samples for simulation alignment purpose only: - For FR1: - For OOK based LP-WUR, {±3.5, ±2.5} dB for LP-RSRP and {±3.5, ±2.5} dB for LP-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - For OFDM based LP-WUR, {±3.5, ±2.5} dB for SS-RSRP and {±3.5, ±2.5} dB for SS-RSRQ - 1st priority: 3.5dB, 2nd priority: 2.5dB - Note 1: the numbers can be updated if needed based on the simulation results and/or the progress in RF session - Note 2: the numbers for accuracy requirement is a separate discussion, and different numbers for OOK based and OFDM based LP-WUS for accuracy requirement are not precluded.

- Simulation assumption

In last meeting, some simulation parameters were still uncertain in RAN4 simulation. To simplify the simulation workload, we proposed to choose a subset of agreed RAN1 simulation. The detail parameters are defined as follow,

Proposal 1: RAN4 to agree data and control channel subcarrier spacing as follow.

- OFDM based: The same as SSB subcarrier spacing

- OOK based: the same as one of the SCS(s) used for other NR transmission

Proposal 2: RAN4 to agree only using SCS = 30KHz in the simulation.

Proposal 3: RAN4 to align the LP-SS binary sequence pattern for M=4, such as Cell 1: [1010] Cell 2: [0101].

- Simulation setting

In last meeting, companies proposed initial simulation results, but it seems the results are not aligned very well. We suggest companies to first align the understanding about the simulation chain design and how to achieve the ideal results before aligning the simulation results.

The signal flow to calculate the RSRP is as follow.

Figure 1. The RSRP estimation flow

In our understanding, ideal LP-RSRP shall be obtained from the serving signal passing the channel in each sample. Similar to NR, to evaluate the LP-RSRP or LP-RSRQ accuracy, we need to compare the practical LP-RSRQ value with ideal. For the calculation of RSRP or RSRQ in NR, the receiver could make channel estimation, then calculate practical and ideal RSRP or RSRQ value respectively to evaluate RRM performance. However, in OOK-based receiver, channel estimation is probably skiped since no phase information after envelop detector. To measure practical LP-RSRP or LP-RSRQ, the signal used needs to consider the impact of noise, interference, channel model, timing error and frequency error, etc. Regarding to the ideal LP-RSRP, the signal used can be received ideal without the impact of timing error, frequency error, noise and interference. Considering the RSRQ reflects the signal quality, such as SINR in NR scenario that RSRQ somehow equals to the ratio of signal power to noise power. For LP-RSRQ, it can be considered as the ratio of signal power to total received power since LP-RSSI has been agreed as total power of ON and OFF symbols.  For the practical LP-RSRQ calculation, the practical receiving signal is used to calculate LP-RSRP and LP-RSSI. To evaluate ideal LP-RSRQ, ideal receiving sigal can be used to calculate LP-RSRP as above, but it can not used to calculate ideal LP-RSSI since LP-RSSI should carry the power of noise and interference.

Proposal 4: RAN4 to first align the ideal results in simulation setting before aligning the simulation results.

- Ideal RSRP is derived from the serving cell signal after the fading channel

- Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI.

- Summary

In this contribution, we have initially discussed the LP-WUS RRM measurement simulation assumption. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to agree data and control channel subcarrier spacing as follow.

- OFDM based: The same as SSB subcarrier spacing

- OOK based: the same as one of the SCS(s) used for other NR transmission

Proposal 2: RAN4 to agree only using SCS = 30KHz in the simulation.

Proposal 3: RAN4 to align the LP-SS binary sequence pattern for M=4, such as Cell 1: [1010] Cell 2: [0101].

Proposal 4: RAN4 to first align the ideal results in simulation setting before aligning the simulation results.

- Ideal RSRP is derived from the serving cell signal after the fading channel

- Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI.

- Reference

[1]. R4-2420100, “WF for RRM requirements for LP-WUS/WUR”, vivo