# 3GPP TSG-RAN WG4 Meeting #114	R4-250xxxx

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.25.2.2

Source: 	Ericsson

Title:	Discussion on RS and channel adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, we achieved some agreements as follow[1].

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress. Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

In this contribution, we continue to discuss on the Rel-19 NES operation in Obj. 3 from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

Scenario

In last RAN1 meeting, RAN1 made the conclusion that no consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell. Thus, RAN4 can confirm that not to define the SSB adaptation for PCell.

RAN1 #119 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Proposal 1: RAN4 not to define the SSB adaptation requirement for PCell in Rel-19.

In RAN1 reply LS, it mentioned RAN1 had agreed that at least SSB burst periodicity is adapted and no any agreement to adapt the time location of the SSB burst other than the periodicity. At the same time, RAN1 reply LS also mentioned the frequency location and the spatial relation for the same SSB index are same before and after SSB adaptation. Thus, RAN4 can only focus on SSB burst periodicity adaptation.

R1-2410917 Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Proposal 2: RAN4 to only focus on SSB periodicity adaptation.

Requirement

In last meeting, an LS related to SSB adaptation in L3 measurement was sent out to RAN2. Thus, RAN4 can wait RAN2 ‘s feedback and focus on other issues. If new SMTC will be introduced, UE shall follow the adapted SMTC. Otherwise, UE shall follow the same SMTC although the SSB periodicity changes.

Proposal 3: RAN4 to wait RAN2’s LS response about SMTC adaptation in L3 measurement.

In RAN4 #112 meeting, a previous agreement is as follow.

Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point.

We think it’s reasonable to introduce SSB adaptation requirement for the L1 measurement procedure, such as BFD, CBD, L1-RSRP. However, we doubt the use case of SSB adapatation during some one shot procedures, such as SCell activation, TCI state switching. NW can easily indicate the SSB adaptation before or after the procedure. Thus, we propose not to consider SSB adaptation during SCell activation, TCI state switching.

Proposal 4: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching.

Proposal 5: RAN4 to define L1 based SSB adaptation requirement for BFD, CBD and L1-RSRP.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we made the following proposals and observations:

Proposal 1: RAN4 not to define the SSB adaptation requirement for PCell in Rel-19.

Proposal 2: RAN4 to only focus on SSB periodicity adaptation.

Proposal 3: RAN4 to wait RAN2’s LS response about SMTC adaptation in L3 measurement.

Proposal 4: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching.

Proposal 5: RAN4 to define L1 based SSB adaptation requirement for BFD, CBD and L1-RSRP.

- References

- R4-2420242, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei