# 3GPP TSG-RAN WG4 Meeting #114	R4-2501331

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.17.2.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1] and summary [2].

- Discussion

Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1] and summary [2]:

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276. - Option 1 (NTT DCM): - For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. - For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.. - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 4 (vivo): RAN4 to consider additional conditions for triggering FBS for L3 measurement for power consumption issue, including active condition and de-active conditions. - Option 5 (Ericsson): RAN4 to study the below FBS deactivation conditions: - After the UE have activated FBS: - The condition of activating FBS is invalid. - Define a minimal during time for FBS/non-FBS. - The condition of activating FBS still is valid but, - L3 measurement with FBS has been conducted for a time period with respect to a pre-defined measurement period or time. - Internal overheating is detected or other limitation from UE perspective. (similar as option 2) - Meet activation conditions but not activate FBS yet: - Internal overheating is detected or other limitation from UE perspective. (similar as option 2) - UE shall report NW of deactivating FBS once the deactivation condition is met. - Option 6 (Samsung): Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off - Dynamic indication; Semi-static configuration - Option 7 (Samsung, Ericsson): - Introduce validity timer configuration with certain ranges via dedicated RRC signalling - Option 8 (Samsung): - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event - Option 9 (Nokia): - When L3 event-triggered reporting conditions are used as a trigger to start FBS (e.g. event A2), UE is allowed to return back to normal beam sweeping when event condition (e.g. A2) is no longer met, or when another event condition (e.g. A1) is met, depending on network configuration. - Recommended WF [Moderator]: discuss the following options(wording can be revised/polished) and companies can add more details if needed. For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - Option 2 (LGE, OPPO, Ericsson, MTK, CTC): UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification. - Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”, when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction, we prefer to introduce UE indication to the NW to assist to NW’s better decision and scheduling.

Proposal 1: UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

Status in the WF [1]:

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

It was agreed to introduce an additional processing delay (2ms) after the last SMTC periodicity for scenario 1/2/3/4/7. For scenario 5, there are three options to be discussed. For scenario 5 handover scenario, Tmargin is time for SSB post-processing. Tmargin can be up to 2ms, there is no room for UE to process multiple beams received in a SMTC, therefore, an additional processing delay need to be considered during handover procedure when multiple beams received in a SMTC. In our views, in order to keep alignment for BSF reduction enhancement solutions in scenario 1/2/3/4/5/7, and considering only two simultaneous Rx beams are supported in the BSF reduction enhancement, we support to consider an additional processing delay (2ms) in the handover delay equation.

Proposal 2: For scenario 5, to consider an additional processing delay (2ms) in the handover delay equation.

Requirement of L3 measurement delay reduction by optimizing Rx BSF

Status in the WF [1] and summary [2]:

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276. - Option 1 (CATT): - The reduced Rx BSF can be applied to the following formula according to the trigger conditions to be specified: - TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement with and without gap; - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement with and without gap; - Tsearch for FR1-FR2 HO and FR2-FR2 HO; - Tidentify_intra_NR and Tidentify_inter_NR, i for RRC Re-establishment delay; - Tidentify-NR for RRC connection release with redirection to NR. - The reduced Rx BSF can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact. - Option 2 (NTT DCM): For each scenario, the highlighted factors should be replaced with capability value: Scenarios; Factors. 1; Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 2; Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6. 3; Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3. 4; Same as scenario 3. 5; If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1.. 7; RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell... - Option 3(Nokia): To apply the scaling factor Nreduced to the fixed value of Tidentify_intra/inter_NR e.g., MAX (Nreduced x 1000 ms, Nreduced x 104 x TSMTC, i). - Recommended WF [Moderator]: can use option 2 as starting point for discussion.

For delay requirements of L3 measurement delay reduction by optimizing Rx BSF, option 1 and option 2 in issue 1-2-2 of the summary [2] are not conflicted, for scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

Proposal 3: For scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction.

Proposal 2: For scenario 5, to consider an additional processing delay (2ms) in the handover delay equation.

Proposal 3: For scenario 1/2/3/4/5/7, the legacy beam sweeping factor 8 shall be replaced with the value in the UE capability.

- References

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418276, Topic summary for [113][218] NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #113, 18 – 22 November, 2024.