# 3GPP TSG-RAN WG4 Meeting #114	R4-2501334

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.3.2

Source:	China Telecom

Title:	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, RRM impacts for SBFD operation for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM impacts for SBFD operation for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

#### Issue 1-2-2: Requirements for CSI-RS based L1 measurement - Agreements (from online session) - For CSI-RS colliding with UL transmission - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission. - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed. - For CSI-RS puncturing in frequency domain in SBFD symbols, - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs. - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs. - FFS for the DUD configuration. - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold. - FFS the measurement behavior for DUD configuration. - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement. - For CSI-RS distributed over SBFD and non-SBFD symbols, - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

For CSI-RS puncturing in frequency domain in SBFD symbols, it was agreed in last meeting that for CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs, and existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold. Besides, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions. In our views, the measurement performance on CSI-RS RB bandwidth at least 48 PRBs across two subbands would be worse than the measurement performance on CSI-RS RB bandwidth with consecutive 48 PRBs, probably the existing accuracy requirements for CSI-RS based measurements could not be applied. However, it’s open to discuss that whether existing accuracy requirements for CSI-RS based measurements can be applied if the related additional side conditions are stricter to achieve better measurement performance on CSI-RS RB bandwidth at least 48 PRBs across two subbands.

Proposal 1: The measurement performance on CSI-RS RB bandwidth at least 48 PRBs across two subbands would be worse than the measurement performance on CSI-RS RB bandwidth with consecutive 48 PRBs, the existing accuracy requirements for CSI-RS based measurements could not be applied.

Status in the summary [2]:

- Issue 1-2-8: Requirements for UL resource muting - Proposals - Proposal 1 (LGE, CTC, vivo, E///): - RAN4 to confirm there is no RRM impact due to UL resource muting. - Proposal 2 (MTK): - RAN4 RRM should close this issue of UL resource muting without agreement on whether there is impact or not. If companies identify there is impact in the future, then companies can bring further proposals. - Recommended WF - As baseline, there is no RRM impact due to UL resource muting. - This can be revisited if impacts are identified in the future based on further RAN1 agreements.

For UL resource muting requirements, we support there is no RRM requirement impacted by UL resource muting.

Proposal 2: There is no RRM impact due to UL resource muting.

- Conclusion

The contribution presents our views on RRM impacts for SBFD operation for evolution of NR duplex operation, with the following proposals:

Proposal 1: The measurement performance on CSI-RS RB bandwidth at least 48 PRBs across two subbands would be worse than the measurement performance on CSI-RS RB bandwidth with consecutive 48 PRBs, the existing accuracy requirements for CSI-RS based measurements could not be applied.

Proposal 2: There is no RRM impact due to UL resource muting.

- References

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418281, Topic summary for [113][223] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #113, 18 – 22 November, 2024.