# 3GPP TSG-RAN WG4 Meeting #114	R4-2501335

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.25.2.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, On-demand SSB SCell operation for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

General OD-SSB requirements

Status in the WF [1] and the summary [2]:

Issue 1-1-2: L1 measurement before SCell activation - Proposals - Option 1: L1 measurements are not supported for deactivated SCell. - Option 2: Wait RAN1’s further clarification on the applicable scenario. Issue 1-1-3: L1 measurement after SCell activation - Proposals - Option 1: CMCC - Unified design between L1 and L3 measurement is preferred, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement. - Option 2: CATT, Ericsson, Xiaomi - RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress. - Option 3: LG, Apple - RAN4 to update the L1 measurement requirement based on OD-SSB. - TSSB = the periodicity of the activated OD-SSB configured for L1-RSRP measurement. - Recommended WF - Further discussion. Issue 1-1-4: OD-SSB accuracy - Proposals - Option 1: CMCC, Ericsson, Qualcomm, China Telecom, Apple - The legacy measurement accuracy is applied for OD-SSB based measurement. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - The legacy measurement accuracy is applied for OD-SSB based measurement.

L1 measurements on deactivated SCell are not supported in current specification, we prefer not to consider L1 measurements before SCell activation.

Proposal 1: L1 measurements are not supported for deactivated SCell.

After SCell activation, OD-SSB based L1 measurement is feasible in NES scenario, RAN4 to define OD-SSB based L1 measurement requirements after SCell activation.

Proposal 2: RAN4 to define OD-SSB based L1 measurement requirements after SCell activation.

For OD-SSB accuracy, we support the legacy measurement accuracy is applied for OD-SSB based measurement.

Proposal 3: The legacy measurement accuracy is applied for OD-SSB based measurement.

OD-SSB based deactivated SCell L3 measurement (Case 1)

Status in the WF [1] and the summary [2]:

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement - Option 1: The same CSSF as legacy - Option 2: - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. Other option is not precluded. - Background - The deactivated SCell measurement requirement in FR1 is as follow. DRX cycle; T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - Proposals - Option 1: CMCC, Mediatek, Ericsson, Apple - The same CSSF as legacy. - Option 2: vivo - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. - Recommended WF - Further discussion

In order to avoid significant impact and complex behavior on UE measurement, it’s preferred to follow the same CSSF as legacy.

Proposal 4: Prefer to follow the same CSSF as legacy.

Status in the WF [1]:

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2? FFS.

For the time window on OD-SSB based deactivated SCell measurement, it was agreed in last meeting that the time window is based on the number of OD-SSB samples and UE provided one valid report, the uncertainty time should also be considered and UE should have enough number of OD-SSB samples to achieve the measurement requirement.

In our views, for the number of OD-SSB samples, the samples of legacy requirements on deactivated SCell measurement period and cell identification specified in TS 38.133 clause 9.2.5 can be used as baseline, e.g., Ceil(5 x Kp) x OD-SSB periodicity x CSSFintra for FR1, and Ceil(M x Kp) x OD-SSB periodicity x CSSFintra for FR2, where M is Mmeas_period_w/o_gaps or Mpss/sss_sync_w/o_gaps for deactivated SCell measurement period and cell identification, respectively. Besides, no lower bound is preferred for the requirements on OD-SSB based deactivated SCell measurement just as legacy requirements specified.

When certain conditions are fulfilled, e.g., T2 is shorter than a threshold, UE can perform deactivated SCell measurement based on measurement period without cell identification. The conditions and the corresponding threshold can be further discussed.

Proposal 5: For the number of OD-SSB samples, the samples of legacy requirements on deactivated SCell measurement period and cell identification specified in TS 38.133 clause 9.2.5 can be used as baseline, e.g., Ceil(5 x Kp) x OD-SSB periodicity x CSSFintra for FR1, and Ceil(M x Kp) x OD-SSB periodicity x CSSFintra for FR2, where M is Mmeas_period_w/o_gap or Mpss/sss_sync_w/o_gaps for deactivated SCell measurement period and cell identification, respectively.

Proposal 6: No lower bound is preferred for the requirements on OD-SSB based deactivated SCell measurement.

Proposal 7: When certain conditions are fulfilled, e.g., T2 is shorter than a threshold, UE can perform deactivated SCell measurement based on measurement period without cell identification.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Status in the WF [1]:

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

For Case 2 OD-SSB based deactivated SCell L3 measurement, when UE receives the OD-SSB activation indication and the serving cell is known, it means that UE has measured deactivated SCell based on always-on SSB, thus UE follows legacy requirements based on measCycleSCell.

In addition, when UE receives the OD-SSB activation indication and the serving cell is unknown, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. The measurement periodicity is OD-SSB periodicity.

Proposal 8: For Case 2 OD-SSB based deactivated SCell L3 measurement, when UE receives the OD-SSB activation indication and the serving cell is known, UE follows legacy requirements based on measCycleSCell.

Proposal 9: For Case 2 OD-SSB based deactivated SCell L3 measurement, when UE receives the OD-SSB activation indication and the serving cell is unknown, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. The measurement periodicity is OD-SSB periodicity.

OD-SSB based SCell activation

Status in the summary [2]:

Issue 1-4-2: The known cell condition – period of known condition - Proposals - Option 1: CMCC, Nokia, LG - Reuse the legacy known/unknown cell condition, max(5*measCycleSCell, 5*DRX cycles) in FR1. - Option 2: Intel - In Case 1, update the known condition period with the max(5 * [measCycleODSSB], 5*DRX cycle). - In Case 2, update the known condition period with the max(5*measCycleSCell, 5*[measCycleODSSB], 5 * DRX cycle). - Recommended WF - Further discussion

UE performs deactivated SCell measurement based on OD-SSB within a time window, and after the time window, UE performs deactivated SCell measurement with a slow mode, which means UE shall meet legacy deactivated SCell measurement requirement after the time window. Therefore, it’s better to reuse the legacy period of known/unknown cell condition, i.e., max(5*measCycleSCell, 5*DRX cycles) in FR1, to cover both the case within the time window and the case after the time window.

Proposal 10: Reuse the legacy period of known/unknown cell condition, max(5*measCycleSCell,  5*DRX cycles) in FR1.

Others

Status in the WF [1]:

Issue 1-5-1: The type of OD-SSB based SCell activation FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios. - Option 1: Direct SCell activation at SCell addition - Option 2: Multiple SCells activation - Option 3: Deactivated PUCCH SCell activation - Option 4: Rel-17 Fast SCell Activation - Option 5: Rel-18 SCell Activation with the L3 reporting during activation

Considering the workload and limited meeting time, we can firstly consider OD-SSB based SCell activation delay requirements on direct SCell activation at SCell addition after OD-SSB based SCell activation delay requirements on MAC CE SCell activation is complete. For other options, we are open to discuss whether to define requirements in Rel-19.

Proposal 11: OD-SSB based SCell activation delay requirements on direct SCell activation at SCell addition can be considered after OD-SSB based SCell activation delay requirements on MAC CE SCell activation is complete.

- Conclusion

The contribution presents our views on On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: L1 measurements are not supported for deactivated SCell.

Proposal 2: RAN4 to define OD-SSB based L1 measurement requirements after SCell activation.

Proposal 3: The legacy measurement accuracy is applied for OD-SSB based measurement.

Proposal 4: Prefer to follow the same CSSF as legacy.

Proposal 5: For the number of OD-SSB samples, the samples of legacy requirements on deactivated SCell measurement period and cell identification specified in TS 38.133 clause 9.2.5 can be used as baseline, e.g., Ceil(5 x Kp) x OD-SSB periodicity x CSSFintra for FR1, and Ceil(M x Kp) x OD-SSB periodicity x CSSFintra for FR2, where M is Mmeas_period_w/o_gap or Mpss/sss_sync_w/o_gaps for deactivated SCell measurement period and cell identification, respectively.

Proposal 6: No lower bound is preferred for the requirements on OD-SSB based deactivated SCell measurement.

Proposal 7: When certain conditions are fulfilled, e.g., T2 is shorter than a threshold, UE can perform deactivated SCell measurement based on measurement period without cell identification.

Proposal 8: For Case 2 OD-SSB based deactivated SCell L3 measurement, when UE receives the OD-SSB activation indication and the serving cell is known, UE follows legacy requirements based on measCycleSCell.

Proposal 9: For Case 2 OD-SSB based deactivated SCell L3 measurement, when UE receives the OD-SSB activation indication and the serving cell is unknown, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. The measurement periodicity is OD-SSB periodicity.

Proposal 10: Reuse the legacy period of known/unknown cell condition, max(5*measCycleSCell,  5*DRX cycles) in FR1.

Proposal 11: OD-SSB based SCell activation delay requirements on direct SCell activation at SCell addition can be considered after OD-SSB based SCell activation delay requirements on MAC CE SCell activation is complete.

- References

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418282, Topic summary for [113][224] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #113, 18 – 22 November, 2024.