# 3GPP TSG-RAN WG4 Meeting #114	R4-2501336

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.25.2.2

Source:	China Telecom

Title:	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, adaptation of common signal/channel transmission for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

SSB adaptation – Other requirements impacts

Status in the summary [2]:

Issue 1-4-1: SCell activation requirement for SSB adaptation - Proposals - Option 1: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching (CATT, Ericsson, Vivo, Samsung, OPPO) - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT) - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson) - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo) - Option 1c: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception. (Samsung) - Option 1d: RAN4 not to define L3 measurement based on SMTC periodicity before and during SCell activation if OD-SSB is triggered before the complete of SCell activation. (Oppo) - Option 2a: (CTC) - After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity (CTC) - After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs. (CTC) - Option 2b: (Huawei) - UE shall use SSB after adaption for SCell activation regardless of the SMTC configuration when SSB adaption happens before or at the same time as SCell activation. Deprioritize the requirements when SSB adaptation happens during SCell activation. - Detailed triggering on SSB adaptation depends on further RAN1 discussion. - Option 2c: RAN4 to study the SCell activation and deactivation delay impact. (Samsung) - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements - Recommended WF: - No requirements when SSB adaptation happens during SCell activation. - FFS the requirements impact when SSB adaptation happen before or at the same time as SCell activation considering. Issue 1-4-2: TCI state switching requirements for SSB adaptation - Proposals - Option 1: No impact is expected for SCell activation and TCI state switching due to SSB adaptation. (CATT) - Option 1a: RAN4 not to define SSB adaptation requirement during SCell activation, TCI state switching. (Ericsson) - Option 1b: RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation. (Vivo) - Option 2: RAN4 to consider the L1 measurement impact from SSB periodicity adaptation both to L1-RSRP measurement delay for active SCells for BFD and CBD, and to TCI state switching delay. (Nokia) - Recommended WF: - No requirements when SSB adaptation happens during TCI state switching.

We support that SCell activation and TCI state switching are not the typical cases for SSB adaptation, since the procedures during SCell activation and during TCI state switching are one shot procedures and do not take place too long time, it’s no need to define SSB adaptation requirements during SCell activation and during TCI state switching, on the contrary, it’s necessary to define SSB adaptation requirements before and after SCell activation, and before and after TCI state switching, which are the common cases and beneficial for network energy savings.

Proposal 1: No requirements when SSB adaptation happens during SCell activation.

Proposal 2: No requirements when SSB adaptation happens during TCI state switching.

- Conclusion

The contribution presents our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: No requirements when SSB adaptation happens during SCell activation.

Proposal 2: No requirements when SSB adaptation happens during TCI state switching.

- References

- R4-2420242, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418283, Topic summary for [113][225] Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #113, 18 – 22 November, 2024.