3GPP TSG RAN WG4 Meeting #114	R4-2501339

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.21.1

Source:	vivo

Title:	Discussion on general aspects for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

Based on this SID, RAN4 had further general discussion on priorities from scenarios-level, use-cases level and sub-use -cases level in the last meeting. In this contribution, we provide our views on general aspects for AI/ML mobility based on the related progress captured in WF[2].

- Discussion

- 2.1 general aspects in AI/ML for mobility

Issue 1-1-1: Scenario priorities - Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case. Scenario number; Priority; Evaluation scenario; Target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell. - Proposals and Observations: - Proposal 1: RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study. - Recommended WF: - Companies to discuss if Proposal 1 is agreeable.

In RAN4#112bis, RAN4 has agreed to start the discussion on scenario 2, 3, and 4 in SI, in which these three scenarios are given the first priority in RAN2. In the last meeting, some companies suggested that it also needs to involve scenario 6 in SI for RAN4 study. For this scenario, our opinion is as follows:

For the spatial domain prediction corresponding to scenario 6, we think this discussion may be similar to RAN4's discussion of beam management prediction in AI/ML. The contents of RAN4 study required for these two prediction scenarios are also highly overlapping. The main difference may be that the study done by beam management in AI/ML is limited to predictions for the serving cell, The predictions for Scenario 6 also include predictions for intra-frequency for inter-cell and intra-cell. The scenario for beam management in AI/ML is a fundamental case, and the details (such as predictive performance evaluation and KPIs definition) are still being evaluated. Therefore, we believe that this case can be studied in RAN4 after certain progress is made there.

Therefore, we believe that we can maintain the conclusion of scenario priorities, and study the three high-priority scenarios of scenario 2, 3 and 4 first. For scenario 6, the study can be considered after further progress is made by AI/ML beam management.

Proposal 1: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management.

Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

As for the priority of use cases, RAN4 had the above working plan in RAN4#112bis meeting. According to the above schedule, the initial discussion is allowed for the measurement event prediction use-case in this meeting. Next, we will make some preliminary analysis of measurement event prediction use-case based on the latest RAN2 progress. Below, we have taken some of the conclusions reached at RAN2#127bis and RAN2#128 meetings for reference.

RAN2#127bis - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (e.g., event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference. RAN2#128 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model 2 The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed. 3 For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional. 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’

In the box above, we have highlighted a few key conclusions about the progress achieved at the two meetings of RAN2. The main summary is shown as follows:

- Indirect measurement event prediction and direct measurement event prediction are both within the scope of RAN2 research. In particular, the difference between the two scenarios is mainly reflected in: for the scenario of indirect measurement event prediction, in addition to the expected occurrence time of a certain measurement event as final output, there is also a further RSRP of serving/neighboring cells as intermediate output.

- Specifically, For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

- For indirect measurement event prediction, based on consideration and studied scenarios, there will be the following four combinations:

  - Indirect measurement event prediction for temporal domain case A

  - Indirect measurement event prediction for temporal domain case B

  - Indirect measurement event prediction for frequency domain

  - Indirect measurement event prediction for spatial domain

- For KPIs for measurement event prediction, RAN2 to adopt the following KPIs for measurement event prediction use case:

  - for indirect: the RSRP difference, missed event detection, false event detection, F1 score, time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction.

  - for direct: missed event detection, false event detection, F1 score.

Combined with the above conclusions, from RAN4 perspective, for indirect scenario, the intermediate result reported is the predicted value of the service cell/neighborhood (A3 event). This is the same as the current RRM measurement prediction use case for temporal domain. The discussion and conclusion on RSRP difference prediction accuracy in RRM measurement prediction use case can be directly reused without extra work effort. For additional KPIs proposed by RAN2, for example, time difference of true time event reporting triggered and predicted time event reporting triggered, since RAN2 is also conducting further performance evaluations on these KPIs, RAN4 can wait for further evaluation results of RAN2, and then discuss whether to define relevant KPIs in RAN4.

In particular, for the studied scenario in RAN4, since we already discussed the priorities of scenarios in issue 1-1-1, we can correspondingly deprioritize scenario 6 first and study scenario 2,3, and 4 first in measurement event prediction.

Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

Issue 1-1-2: Sub-use-case priorities - RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Proposals and Observations: - Proposal 1 (Nokia): RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction. - Proposal 2 (Ericsson): For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements. - Proposal 3 (Ericsson): Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as: - Scenario 2, 3 and 4, the 3 sub-cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. - Proposal 4 (Qualcomm): RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - Recommended WF: - Companies to further discuss Proposal 1 and Proposal 2.

For the prediction performance of the above three sub-use cases, based on simulation evaluation in RAN2, the performance difference is observed: In a specific scenario (with a shorter prediction window), case 2 has the highest prediction accuracy than the other two cases. In addition, compared with the other two cases, the input and output of case 2 are both predictions at the same point (point C), and the implementation process is relatively simple. Therefore, regarding case 2, it can be considered as one of the cases in RAN4 study.

For L3 filtering, RAN2 discussed and agreed on two L1 filtering options, i.e., sliding and non-sliding L1 filtering. These two methods are still on the table for further evaluation.

Figure 1 Sliding L1/L3 filtering Figure 2 non-sliding L1/L3 filtering In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period. In both L1/L3 filtering options, the filtered L1 measurement result is obtained based on the non-filtered L1 measurement results within one measurement period. And the filtered L3 measurement result is obtained as specified in section 5.5.3.2 of TS 38.331

In our understanding, these two implementations may differ in prediction performance. For the sliding L1 filtering, as it utilizes previously measured L3-RSRP values for calculation, This could be what makes the L1 prediction results smoothed. For the non-sliding L1 filtering, only the predicted L1-RSRP participated in the subsequent L3 filtering. Considering the implementation method has a certain impact on the performance of case 1, it is difficult to say whether case 1 or case 3 performs better for prediction scenarios with L1 results as input and L3 results as output. Therefore, we believe that neither case can be excluded or deprioritized before conducting the necessary simulation evaluation or a concrete conclusion on how to choose the implementation of these two cases.

Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases

- Summary

Proposal 1: RAN4 to study the three high-priority scenarios of scenario 2, 3 and 4 first for measurement prediction use-case. For scenario 6, the study can be considered after enough progress is made in AI/ML beam management.

Proposal 2: For measurement event prediction use-case, RAN4 to deprioritize the scenario on indirect measurement event prediction for spatial domain

Proposal 3: Before performing enough evaluation and reaching an aligned conclusion prediction performance in terms of three sub-use cases, RAN4 not to have any deprioritization given to any sub-use cases

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2420095, WF on AI/ML Mobility, Nokia