3GPP TSG RAN WG4 Meeting #114	R4-2501340

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.21.2

Source:	vivo

Title:	Discussion on impacts on RAN4 requirements for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we further provide our views on the impacts on RAN4 requirements for AI/ML mobility based on the latest WF[2].

- Discussion

- 2.1 KPIs discussion in AI/ML for mobility

In the RAN4#113 meeting, the issues related to KPIs definitions were discussed. The latest outcomes are shown as follows.

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2 Issue 2-1-1: Absolut and/or Relative RSRP Prediction Accuracy Way Forward: Session Chair: Encourage companies to bring proposals on the definition of relative RSRP and absolute RSRP. Subsequently, companies may also consider following candidate options to introduce RSRP accuracy requirements: - Option 1: RAN4 to consider relative RSRP accuracy requirements only. - Option 2: RAN4 to consider absolute RSRP accuracy requirements only. - Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

First, from testing perspective, RAN4 has a preliminary alignment for the definition of L3 RSRP prediction accuracy. The KPI is defined in a similar way to the definition of L1-RSRP prediction accuracy in AI/ML beam management. There has also been some discussion in AI/ML about the FFS part on the ground truth definition. In RAN4#111, there will be the following agreement:

Issue 2-3: Reported measurements and ground truth Agreements: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

- Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

As can be seen from the above, ground truth on L1-RSRP prediction is defined as the reported RSRP measurement with certain side conditions. For L3-RSRP prediction, we believe that the analysis should combine with temporal domain case A and case B:

In the temporal domain case A, continuous measurement results in the prediction window are predicted by continuous historical measurement result(s) in the observation window. Given that the RRM measurement is performed continuously, the ground truth of each time instance can be obtained from the UE measurement.

For temporal domain case B, considering the measurement in the prediction window may be skipped directly, the RSRP measurement value corresponding to the time instance cannot be obtained. If the ground truth is defined as reported RSRP measurement results, then the UE is required to keep measuring, and the UE behavior is similar to case A. Therefore, we believe that a further study on temporal domain case B is needed before the reported RSRP measurement results are regarded as the ground truth.

Observation 1: For temporal domain case B, considering the measurement in the prediction window may be skipped directly, the reported RSRP measurement for the corresponding time instance may not be available as the ground truth of L3 RSRP to be considered

In addition to the above definitions, another definition of ground truth is also proposed in the discussion of AI/ML BM:

- Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

In this way, the test environment corresponding to the predicted time is restored, and then the RSRP corresponding to the time instance is measured. For the reducibility of the measured RSRP, it is necessary to ensure that the channel environment characteristics are consistent at the same time point. Compared with Option 1, this definition is the same in both temporal domain case A and temporal domain case B. Therefore, we believe that this way of definition can also be used as a candidate Option for further study

Proposal 1: For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options:

- Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

- Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

Furthermore, as to whether relative RSRP accuracy needs to be defined, according to some companies' proposals, for frequency domain prediction, the benefit of having multiple cells as output is more obvious. In this scenario, relative RSRP is necessary. However, as far as we know, the discussion and simulation of RAN2 are still limited to scenarios where single/multiple cells are used as inputs and single cells are used as outputs. So we can start with the KPI of absolute RSRP accuracy, and then if there are new scenarios supported, RAN4 can further consider relative RSRP accuracy.

Observation 2: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output.

Proposal 2: RAN4 to first consider absolute RSRP accuracy requirements. And for the relative RSRP accuracy, it can be further considered if the necessary scenarios are supported

- 2.2 Simulation aspects

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Background Following candidate options were identified in RAN4#112bis meeting’s moderator’s summary. - Candidate options (in the TDoc summary): - Option 1: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error. - Option 2: suggest RAN2 to take into account the measurement errors in RAN2 evaluation. - Option 3: Wait for RAN2 evaluation results and if RAN2 doesn’t consider measurement errors then RAN4 should do it after RAN2 evaluations. - Option 4: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility. - Proposals and Observations: - Proposal 1 (OPPO): - Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works.. - Proposal 2 (ZTE, CMCC, Huawei, MediaTek): - RAN4 should study the impact of measurement error on the prediction accuracy requirements for AI mobility - Proposal 3 (Apple): - RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. - Proposal 4 (Qualcomm, vivo): - RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM - Proposal 5 (Xiaomi): - RAN4 to discuss which part of procedure agreed in AI BM can be re-used for measurement error analysis of AI mobility. - Proposal 6 (Samsung): - RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error - For FR2 temporal domain prediction, suggest to wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility - For FR1 temporal and frequency domain predictions, suggest RAN4 to discuss how to evaluate the impacts of measurement errors on the prediction accuracy performance for AI mobility - Proposal 7 (Nokia): - To study the impact of measurement error, RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4). - Proposal 8 (Ericsson): - RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. - Discuss and identify RAN4 specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility. Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase (MTK) - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase (vivo, OPPO) - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. (Apple, E///, NTT DCM, Nokia, Xiaomi, CATT) - RAN4 to discuss and identify specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility. - Recommended WF - Discuss above listed options.

RAN2 has agreed the simulation assumptions as Table 5.1-1 in TR 38.744. Both beam level and cell level results for training and inference all derived from the system level simulation in UMi and UMa channel. From RAN4 perspective, it may need to consider additional factors that may not be considered in RAN2 evaluation on prediction accuracy, e.g., measurement error modelling. At the initial study stage in RAN4 discussion, it can be considered as one of the study aspects. For beam level results, the measurement errors can be modelled as some types of probability distributions, or derived from link level simulations. System level simulation based beam level RSRPs are used as ideal dataset and add the beam level errors into the ideal dataset to derive the nonideal dataset to evaluate the measurement error impact is one feasible option which is also discussed in AI BM simulation assumptions in RAN4#112bis and RAN4#113[3]. Besides, if the model input is the cell-level result, there are two options for error modelling. First is to use the L3 filter to the nonideal beam-level results mentioned above to derive the nonideal cell-level results, second is to directly modelling the cell-level errors to add into the ideal cell-level dataset and the modelling method needs to be further studied and both options could be considered in RAN4.

However, considering it is still in SI phase, the relevant impact of the error is discussed when the requirements are defined. This part of the work, such as evaluating the performance when considering measurement errors, is more appropriate for future WI phases. In addition, in AI/ML, there is still no clear conclusion on the impact of measurement error on requirements and whether to consider the error when defining requirements. Therefore, before incorporating the measurement error impacts into the consideration of AI mobility, we suggest that RAN4 to wait for more conclusion in AI/ML.

Proposal 3: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

- Summary

Observation 1: For temporal domain case B, considering the measurement in the prediction window may be skipped directly, the reported RSRP measurement for the corresponding time instance may not be available as the ground truth of L3 RSRP to be considered

Proposal 1: For testing, the ground truth definition of L3-RSRP for FR1 and FR2 can be further studied based on the following candidate Options:

- Option 1: To define the ground truth of L3 RSRP as the reported RSRP measurement under certain SNR

- Option 2: To define measured RSRP from t1 to t2 as the ground truth under the condition that the channel from t0 to t1(prediction window) is the same as the channel from t1 to t2

Observation 2: For the prediction case on frequency domain, the discussion and simulation of RAN2 is still limited to the scenario of single/multiple cells as input and a single cell as output.

Proposal 2: RAN4 to first consider absolute RSRP accuracy requirements. And for the relative RSRP accuracy, it can be further considered if the necessary scenarios are supported

Proposal 3: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI .

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2420095 WF on AI/ML Mobility, Nokia

- R4-2420480, Updated simulation assumptions for beam prediction, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT