3GPP TSG RAN WG4 Meeting #114	R4-2501341

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.21.3

Source:	vivo

Title:	Discussion on	testability and interoperability for AI mobility

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we will further provide our views on testability and interoperability issues for AI/ML mobility based on the latest WF[2].

- Discussion

- 2.1	Testability issues in AI/ML based beam management

In AI/ML air interface discussion, many testability issues are raised and discussed in AI BM sub-case, where the key issue is whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics. A potential new FR2 OTA test system for AI BM will bring high cost and complexity for the related vendors and many technical issues are raised for the test setup.

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

- 2.2 Potential testability issues in AI/ML for mobility

Either beam-level metric and cell level metric are needed to be emulated in the test system. For the ground truth in the test system, it depends on the discussion of performance metrics issues in RAN4 requirment part.

In RAN4#112bis meeting, conducted testing is agreed as the baseline for FR1 (scenario 2 and 3). When these two scenarios are further considered in conjunction with three sub-use cases, where for sub case#1 and #3 (model input is L1 beam-level measurement result and the model output is L3 Cell-level measurement result(s)). Potential testability issues that may need to be considered are whether a certain number of beams need to be generated in the test environment. In our understanding, the beams used for the subsequent layer 3 have a high probability of only including a few strongest beams after selection. From this perspective, we can consider not simulating all the beams but only generating a few strongest beams in the test environment to simplify the test complexity. Considering all the test setups can be generated by software in test instruments, we believe that no testability issue to be concerned.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Observation 2: There are no testability issues for the conducted testing in scenario 2 and 3.

And for testing in FR2 (scenario 4), OTA testing should be used as the baseline and at least channel model and UE trajectory issues need to be raised for testability.

2.2.1 Channel model

According to Fig.1 depicted in TS38.300, A and A1 points are all applicable in the current stage.

Fig. 1 Measurement Model

However, both processes are beam-level results based, and testability issues against cost and complexity should also be noticed. In AI/ML based beam management case 1, Top K beams are predicted in spatial domain and multi-probe anechoic chamber test system are needed to emulate the spatial channel environment. If one-probe setup or flat channel model is used, the real performance of AI model could not be reflected. In AI mobility, time and frequency domain prediction is performed and AI model performance can be verified in a test system without any spatial characteristics. For example, the AI model uses the measurement results of beam i, beam j and beam k during the observation window in t1 and predicts the corresponding results based on beam i, beam j and beam k during the prediction window in t2. The spatial characteristics in t1 and t2 are the same and no other beam-level results are needed to derive the eventual performance. Based on this understanding, 1 probe could emulate different Tx beams by changing the power weight of reference signal in FR2 for one cell.

Thus, there is no need to considered the CDL channel since there no spatial characteristics needs to be reconstruced in the chamber. The AI prediction can also operate normally without spatial characterists since all scenarios are in temporal/frequency domain and AWGN/TDL channels are enough for OTA testing.

Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

Proposal 2: RAN4 to consider using AWGN/TDL channel model as the baseline for testing.

2.2.2 UE trajectory

In legacy test, the dynamic characteristic are only reflected by changing the doppler for generating the channel coefficients and changing the SNR. However, the AI model may be trained under field data/UMa data in a continues time with temporal/spatial consistency and correlations considering UE is moving and rotating. If only change the dopper and SNR, the test environment may be looks randomly from UE-sided AI model perspective and mismatch may be occurred. Therefore, we proposal to study a new method to model UE trajectory, method in TR 38.762 for dynamic OTA testing can be referred to, e.g., modelling a UE trajectory with some waypoints and generate channel model in every waypoint, as shown in Fig. 2. UE could be ratoted in a 3D turnable with the reference LOS AoA direction and change the probe power weight to emulated the dynamic environment.

Fig. 2 Base station and the UE route in the TR 38.762 [3].

Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power.

Based on the analysis in section 2.2.1 and 2.2.2, table 1 shows the potential testability considerations for the 3 scenarios from our perspective

Table 1 potential testability considerations for the 3 scenarios

| scenario number | Sub-scenario | Frequency Range | Test method | Channel model (as start point) | Min cell number | Min probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | FR1 | Conducted | AWGN/TDL | 1 serving cell | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) |  |  |  |
| 3 | 1 | FR1 | Conducted | AWGN/TDL | 2 (1 serving cell, 1 neighbour cell) | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 3 (2 co-located neighbour cell, 1 serving cell) |  |  |  |
| 4 | 1 | FR2 | OTA | AWGN/TDL | 1 serving cell | 1 | Post SNR of the strongest beam pair | Change doppler, SNR, UE rotation, and change probe power |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) | 2 |  |  |

Proposal 4: RAN4 to take the potential testability setups for the 3 scenarios into consideration as the following:

| scenario number | Sub-scenario | Frequency Range | Test method | Channel model (as start point) | Min cell number | Min probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | FR1 | Conducted | AWGN/TDL | 1 serving cell | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) |  |  |  |
| 3 | 1 | FR1 | Conducted | AWGN/TDL | 2 (1 serving cell, 1 neighbour cell) | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 3 (2 co-located neighbour cell, 1 serving cell) |  |  |  |
| 4 | 1 | FR2 | OTA | AWGN/TDL | 1 serving cell | 1 | Post SNR of the strongest beam pair | Change doppler, SNR, UE rotation, and change probe power |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) | 2 |  |  |

2.2.3 Test environment mismatch

Moreover, training methods and datasets are important factors that need to be taken it account. For training datasets, they are usually derived from simulations and field measurements. When we consider using fixed 8 beams pattern as the model input and 32 beams as the model output for deriving the spatial domain beam prediction dataset (set B = 8 and set A = 32) to train the model, one issue may be noticeable is that, once the model is already trained, only correct format of inference dataset can be applicable. If the data format is not matched, e.g., using 4 beams dataset as inference, it may cause the model to become unworkable since the model is only applicable for 8 beams format input. In AI mobility, the specific format issue also exists, which requires that AI model is capable of generalizing most types of input formats to guarantee a stable performance in the field since many Tx beams are usually drowned in the noise and may not be seen on UE side. From test perspective, bulid a simplified test system is the main goal and most of the Tx beams are skipped in the test lab, which will cause the input dimension mismatch issue. The issue is illustrated in Fig. 3, where ti is the time instance for measurement and tj is the time instance for prediction. RAN4 needs to consider this aspect in the following research and further perform evaluations before defining requirements and test methodologies.

Fig. 3 Illustration of model input dimension mismatch issue

Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab.

Proposal 5: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

- Summary

Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion.

Observation 2: There are no testability issues for the conducted testing in scenario 2 and 3.

Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system.

Proposal 1: RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

Proposal 2: RAN4 to consider using AWGN/TDL channel model as the baseline for testing.

Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power.

Proposal 4: RAN4 to take the potential testability setups for the 3 scenarios into consideration as the following:

| scenario number | Sub-scenario | Frequency Range | Test method | Channel model (as start point) | Min cell number | Min probe number | SNR | UE trajectory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1 | FR1 | Conducted | AWGN/TDL | 1 serving cell | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) |  |  |  |
| 3 | 1 | FR1 | Conducted | AWGN/TDL | 2 (1 serving cell, 1 neighbour cell) | N/A | legacy SNR | Change doppler and SNR |
|  | 2 |  |  |  | 3 (2 co-located neighbour cell, 1 serving cell) |  |  |  |
| 4 | 1 | FR2 | OTA | AWGN/TDL | 1 serving cell | 1 | Post SNR of the strongest beam pair | Change doppler, SNR, UE rotation, and change probe power |
|  | 2 |  |  |  | 2 (1 serving cell, 1 neighbour cell) | 2 |  |  |

Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab.

Proposal 5: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2420095 WF on AI/ML Mobility, Nokia

- R4-2418755 Discussion on impacts on RAN4 requirements for AI mobility, vivo