3GPP TSG-RAN WG4 Meeting #114                                                     R4- 2501348

Athens , GR, 17th – 21st, Feb. 2025

Source:		Nokia

Title:	UE RF requirements (MIMO Phase5)

Agenda item:		7.22.2

Document for:	Discussion

# Introduction

Based on the agreement in the WF from RAN4 #113 meeting [1], in this paper, we’d like to present our view in 3Tx and UL TRP related issues.

# <Topic 1> 3Tx ## <Sub-topic 1-1> Power class <Agreement>: PC for 3Tx requirement - PC for 3Tx requirement - PC3 - PC2 - FFS on PC1.5 - FFS on UE type, i.e., FWA and/or handheld UE - FFS on support of full power mode 0 - RAN4 understanding is that TxD for 3Tx is not in the scope of this WI - Full power mode 0 apply to PC2 and PC3 only. - FFS on for which UE types it should apply. <Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx ## <Sub-topic 1-2> PA configuration <Agreement>: Assumption for power splitting between antenna ports - Equal power split among non-zero ports according to RAN1 spec <Agreement>: PA configurations - For MPR evaluation, the assumptions of PA architecture are: - For PC3, 3x23dBm - For PC2 - 3x23dBm - 3x26dBm is not precluded and interested company can provide the evaluation ## <Sub-topic 1-3> MPR <Agreement>: Different MPR for different UE type - Define separate MPR requirements for handheld and FWA UE <Way forward>: MPR derivation method for 3Tx - Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia) - Option 2: Further evaluate with 3PA configuration (Apple) ## <Sub-topic 1-4> Supported layer for mode 0 <Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0 # <Topic 2> Asymmetric TRPs ## <Sub-topic 2-1> UE RF impact <Agreement>: Whether to assume different TCI state between DL and UL - Assume different TCI state between DL and UL <Way forward>: Whether to study FR2 enhancements for asymmetric TRPs scenario in Rel-19 - RAN4 will not study FR2 enhancements until RAN1 considers the scenario in Option 1 - Option 1: Yes, UL-specific TRPs are assumed to have limited DL capability - Option 2: No, need more check - Option 3: Check with RAN1 sending an LS

The agreement from RAN1 in the LS [2]:

# Overall Description RAN1 thanks RAN4 for the LS related to UE RF issues for Rel-19 MIMO Enhancement. Per request, RAN1 provides responses to the questions. (NR MIMO 9.2.4) Enhancement for asymmetric DL sTRP/UL mTRP scenarios Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID. (NR MIMO 9.2.3) 9.2.3 Support for 3-antenna-port codebook-based transmissions RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks as below for example but not with 1Layer 3-port codebooks. 13100100, 13100001, 13 001001. Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks. Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s). Regarding Questions 2 and 3, RAN1 would like to inform RAN4 that UL full power Mode 0 delivers full power for any precoder in the 3-port codebook, regardless of the number of layers in the precoder. Reply for Question 2: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders. Reply for Question 3: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

# Discussion

  - PA configuration and MPR

In Release 17, HPUE for NR FR1 FDD band n77 and n78 for power class 1.5 has been introduced, which covered smartphones and fixed wireless access (FWA) devices. The PC1.5, PC2 handheld devices may face power/battery issue support 2/3-layer ULFPTx mode 0 for by one single RF chain. Therefore, it is reasonable to use two PAs instead of one PA to achieve 26dBm and 29 dBm output power for one layer. Or one PA to support 26dBm and 29 dBm output power for one layer, and use two PAs to achieve 26dBm and 29 dBm for another layer.

Therefore, the MPR need to be defined based on PA configurations. For example, for a single 26dBm PA, if the MPR is 3 dB, then using two such PAs to achieve 29dBm, the MPR should be 6dB. In order to clear distinguish such difference, suggest defining MPR based on single PA configuration.

Observation 1: Handheld UE may face power/battery issue support ULFPTx mode 0. Using two 23 dBm PAs to achieve 26 dBm or using two 26dBm PAs to achieve 29dBm is an option.

Observation 1: The overall MPR will be different for using one PA to achieve 29dBm or two PAs with less output power to achieve 29dBm.

Proposal 1: Suggest defining MPR based on single PA configuration. Make a note that if two PAs are used to support to ULFPTx mode 0 for one layer with higher power output, then calculate MPR based on single PA configures.

  - UL TRP UE RF impact

The UL link only TRP is used to increase the UL throughput, and this TRP is used only for receiving data from UEs. Therefore, the TCI states used for UL TRP need to be specifically configured. However, this will not impact the RF parameters of how UE connect to UL TRP, and the current beam corresponding specification in Section 6.6 in 38.101-2 can be used directly.

Due to no changes in the current specification for UL TRP case (in LS [2], the SSB can be send to UE or UE need to use other implementation agnostic methods, such as sweeping, to find where is UL TRP in FR2). It seems there is no need to modify or adding information to repeat the specification in beam correspondence. However, we do see the UE supporting UL TRP can be considered as a capability. It can be mentioned in the corresponding standards.

Observation 3: No parameter changes in the current specification for UL TRP case, it seems there is no need to modify or adding information to repeat the specification in beam correspondence

Proposal 2: UE supporting UL TRP can be considered as a capability. It can be mentioned in the corresponding standards.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: Handheld UE may face power/battery issue support ULFPTx mode 0. Using two 23 dBm PAs to achieve 26 dBm or using two 26dBm PAs to achieve 29dBm is an option.

Observation 1: The overall MPR will be different for using one PA to achieve 29dBm or two PAs with less output power to achieve 29dBm.

Proposal 1: Suggest defining MPR based on single PA configuration. Make a note that if two PAs are used to support to ULFPTx mode 0 for one layer with higher power output, then calculate MPR based on single PA configures.

Observation 3: No parameter changes in the current specification for UL TRP case, it seems there is no need to modify or adding information to repeat the specification in beam correspondence

Proposal 2: UE supporting UL TRP can be considered as a capability. It can be mentioned in the corresponding standards.

# References

- R4-2417116, WF on UE RF requirements for MIMO enhancement

- R1-2410870, Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement