3GPP TSG-RAN WG4 Meeting #114	R4-2501352

Athens, Greece, 17th – 21st February 2025

Source:	Ericsson

Title:	On general issues for Rel-19 MMSE-IRC receiver under interference

Agenda Item:	7.18.1

Document for:	Discussion

# 1	Introduction

During the RAN#104 meeting, the revised WID on NR Demodulation Performance: Phase 5 was approved [1]. The WID outlines the following objectives for the UE and BS components:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provided our view on general issues for UE and BS demodulation requirement with interference.

# 2	Discussion

In this section, we address the general issues related to the UE performance requirements for 8Rx with interference and BS performance requirements with IRC.

2.1	BS performance requirements

Applicability rule for different test scenarios

- Proposals on applicability for Homnet and Hetnet scenarios:

  - Proposal 1: HomNet requirements are not applicable for Local Area BS

  - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS

- Proposals on additional applicability rule:

  - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

  - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested

Manufacturer declarations

- Proposals:

  - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

According to [2], both homogeneous and heterogeneous network scenarios have been agreed upon for defining BS requirements for Rel-19 MMSE-IRC. Consequently, a manufacturer declaration should be introduced, followed by the appropriate applicability in Table 4.6 of TS 38.141-1 for BS types 1-C and 1-H.

Proposal 1: Introduce new manufactory declarations and applicability rules.

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | X | X |

Proposal 2:

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

For applicability rules in different test scenarios, in LTE (TS 36.141 Sec. 8.2.6.5), a note was added to limit enhanced PUSCH performance requirements to Wide Area BS under synchronous interference. However, a similar approach would require adding a note in multiple test requirement tables for PUSCH MMSE-IRC in TS 38.141-1 Sec. 8 for NR. A more efficient way would be to declare the applicability rule once in the beginning of the Sec. 8.2.x as follows:

Proposal 3:

8.2.x Performance requirements for PUSCH with synchronous interference

8.2.x.1 Definition and applicability

The performance requirements for PUSCH with synchronous interference only apply to Wide and Medium Area Base Stations for INR set 1, and Local Area Base Station for INR set 2 based on manufactory declaration D.2 in Table 4.6-1 (refer to Table 8.2.x.x.x-x for INR set 1 and 2).

Further, mentioning both INR sets corresponding to homogeneous and heterogeneous network scenarios explicitly regarding the applicability seems more feasible to prevent any implicit assumptions. For instance, proposal 1 in open issues may imply that heterogenous network requirements are applicable for all BS classes.

Also, in LTE (TS 36.141 Table 8.2.6.4.2-2, Note 2), a note in the test parameters to define the DIP value and corresponding DIP set 1 and set 2 is added, for homogenous and heterogenous network scenarios, respectively. However, INR-based interference modeling is agreed in the previous meeting [3]. Therefore, for NR TS 38.141-1 and TS 38.104, a similar approach like LTE can be followed adding a note in the test parameters.

Proposal 4: Adding a note to PUSCH with synchronous interference test parameters to define INR value and corresponding set 1 and set 2 for respective homogenous and heterogenous scenarios.

Table 8.2.x.x.x-x: Test parameters for testing PUSCH with synchronous interference

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The respective received average power spectral density of each interferer cell relative to Noc(j) is defined by its associated INR value as specified in clause B.x.x. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. |  |  |  |  |

Since the applicability rules for different test scenarios are already defined independently in Proposal 3, there is no need for an additional applicability rule. For a homogeneous network, both Wide Area BS and Medium Range BS are required to pass the test. In contrast, for a heterogeneous network, only Local Area BS needs to pass the test.

Proposal 5: Do not define additional applicability rule for homogenous and heterogenous network scenarios.

## 2.2	UE performance requirements

Receiver capability

As the 8Rx receiver defined in Rel-18 is only covering SU-MIMO scenario, companies have discussed whether to introduce new UE capabilities in this release or make some modifications on the existing one. Following options are on the table:

Definition of MMSE-IRC receiver capability for 8Rx UE - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

In our view, defining a new capability for MU-MIMO is a bit redundant, and has the potential to be misunderstood in the future because the SU-MIMO 8Rx receiver and MU-MIMO 8Rx receiver definitions are quite similar.

Since we only enlarge the applicable scope for the Rel-18 8Rx receiver but to keep every other thing the same, we are of the opinion for option 4 to simply revise the existing definitions in 38.101-4 section 3.1 to remove the description of ‘SU-MIMO’.

Proposal 6: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Receiver coverage

During the meeting of RAN4 #112bis in Hefei, companies have reached agreements on the issue of 8Rx UE MMSE-IRC receiver coverage:

8Rx UE MMSE-IRC receiver coverage Agreement: - Cover both Baseline 8Rx UE MMSE-IRC receiver and simplified 8Rx UE MMSE-IRC Receiver for the initial simulation analysis. - TBD: Criteria used to decide if one set of requirements can be introduced covering both receiver types. - Note: Test coverage shall cover both receivers

After the RAN4 #113 meeting in Orlando, there are still three options left for further discussions.

8Rx UE MMSE-IRC receiver coverage - Proposals - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

To help reach agreements, we provided simulation results for both baseline receiver and simplified receiver to better understand the performance gap so that to determine whether it is feasible to introduce one set of requirements.

We shared our results for the following cases (2 interference cells):

Table 2.2-1 Simulation results assuming baseline receiver and simplified receiver

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| Baseline 8Rx receiver |  |  |  |  |  |  |  |  |
| 1 | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.65 | 11.07 | 2.42 |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 3 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 9.21 | 14.47 | 5.26 |
| Simplified 8Rx receiver |  |  |  |  |  |  |  |  |
| 1a | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.17 | 11.10 | 2.93 |
| 2a | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 9.07 | 13.25 | 4.18 |
| 3a | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 11.24 | 14.46 | 3.22 |

From the results above we can observe 1.81dB and 2.03dB performance difference comparing the baseline 8Rx receiver with the simplified 8Rx receiver.

If we look at the performance for Rel-18 8Rx PDSCH demodulation, around 2dB gap between the baseline 8Rx receiver and the simplified 8Rx receiver has been observed among different companies results, and it’s agreed to defined two sets of requirements to cover both receivers.

Observation 1: Around 2dB gap has been observed between the performance of the baseline 8Rx receiver and the simplified 8Rx receiver.

Proposal 7: Define two sets of requirements for the baseline receiver and the simplified receiver respectively.

# 3	Summary

In this contribution, we shared our views on the general issues related to the IRC receiver under interference scenario. Our observations and proposals are summarized as follows:

Observation 1: Around 2dB gap has been observed between the performance of the baseline 8Rx receiver and the simplified 8Rx receiver.

Proposal 1: Introduce new manufactory declarations and applicability rules.

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | X | X |

Proposal 2:

8.1.2.1.x.x	Applicability of PUSCH with synchronous interference

Unless otherwise stated, PUSCH with synchronous interference requirements shall apply only for a BS declare to support MMSE-IRC receiver.

Proposal 3:

8.2.x Performance requirements for PUSCH with synchronous interference

8.2.x.1 Definition and applicability

The performance requirements for PUSCH with synchronous interference only apply to Wide and Medium Area Base Stations for INR set 1, and Local Area Base Station for INR set 2 based on manufactory declaration D.2 in Table 4.6-1. (refer to Table 8.2.x.x.x-x for INR set 1 and 2).

Proposal 4: Adding a note to PUSCH with synchronous interference test parameters to define INR value and corresponding set 1 and set 2 for respective homogenous and heterogenous scenarios.

Table 8.2.x.x.x-x: Test parameters for testing PUSCH with synchronous interference

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The respective received average power spectral density of each interferer cell relative to Noc(j) is defined by its associated INR value as specified in clause B.x.x. INR set 1 and set 2 are derived respectively in homogeneous and heterogeneous network scenarios. |  |  |  |  |

Proposal 5: Do not define additional applicability rule for homogenous and heterogenous network scenarios.

Proposal 6: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 7: Define two sets of requirements for the baseline receiver and the simplified receiver respectively.

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2] 	R4-2419780, “Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS”, China Telecom, MediaTek, RAN4#113, Orlando, USA.

[3] 	R4-2416548, “WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, RAN4#112-bis, Hefei, China.