# 3GPP TSG-RAN WG4 Meeting #114	R4-2501355

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:			NTN Ku bands – system parameters

Agenda item:		7.11.3

Document for:		Approval

# Introduction

In last RAN#106, the NTN KU-band WI was revised ([1]) and captured the agreement that NTN Ku-bands shall be defined for both FR1-NTN and FR2-NTN.

Based on our other contribution analyzing the WI update impacts [2], this contribution is studying the relevant system parameters for the NTN Ku-bands considering previous RAN4 agreements as well.

# Discussion

## Definition of the NTN frequency ranges

As highlighted in our contribution analyzing the WI update impacts [2], each new NTN Ku-band shall be specified in FR1-NTN and FR2-NTN. Therefore, the FR1-NTN and FR2-NTN frequency ranges definition shall be updated to booth include the NTN Ku-bands frequency range.

The current assumption is that there is no issue having FR1-NTN and FR2-NTN overlapping but this is still pending on RAN1 and RAN2 confirmation.

Proposal1: Update the definition of NTN frequency ranges according to the following table:

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

## NTN operating bands

Based on the agreements made in previous RAN4 meetings ([3] and [4]), considering that each new NTN Ku-band shall be specified both in FR1-NTN and FR2-NTN, the following bands shall be added to the list of NTN operating Bands.

Note that we haven’t include yet the band 10.70 – 12.75 GHz (Space-to-Earth) and 13.75 – 14 GHz (Earth-to-Space) as it was agreed to specify that band as a second priority. This would require additional study of the corresponding Regulations, and this could be done when the work related to the NTN KU-bands specifications will be almost done, this to not jeopardize the WI schedule.

Also, looking at published ECC Decisions, published ETSI Harmonized Standards and FCC 25.202 (see below), the frequency range 13.75-14.0 GHz has not been allocated to ESIM operation. Considering the WI scope reduction (restricted to Mobile VSAT), this frequency range should then not be considered in the NTN Ku-bands definition for those countries.

On top of this, as specified in FCC 47 CFR 25.202, FCC has only allocated the frequency range 10.7-12.2 GHz for VMES or ESIM operations (communicating with NSGO and/or GSO satellite). This would impact the definition of the US NTN Ku-band, as the revision of the last NTN Ku-bands WI () restricted its scope to Mobile VSAT.

Observation1: Following the RAN4 agreement ([4]), the band 10.70 – 12.75 GHz (Space-to-Earth) and 13.75 – 14 GHz (Earth-to-Space) should be considered as second priority.

Observation2: There is no ECC Decision nor ETSI Harmonized standards covering the frequency range 13.75 – 14 GHz.

Observation3: FCC has allocated the frequency ranges 10.7-12.2 GHz and 14.0-14.5GHz for VMES and ESIM operations.

Following the same approach agreed for the Ka band for US, 2 bands should be defined: one supporting ESIM operation and fixed VSAT (n511) and one supporting fixed VSAT operation only (n510). Also, as fixed VSAT is now out of scope, to be consistent with the updated WI, RAN4 should only defined the band corresponding to the frequency range which was allocated for ESIM operation by ESIM. The other band could of course be specified in a later stage.

Observation4: For the NTN Ka-bands in US, RAN4 decided to allocate 2 bands: one for the frequency range allocated to ESIM and Fixed VSAT operations (n511) and one for the frequency range allocated to Fixed VSAT only (n510). RAN4 should better have the same approach for the NTN Ku-bands definition in US.

Based on those observations and the above explanation, we would make the following proposals:

Proposal2: Add the following NTN Ku-bands in the list of satellite operating bands in FR1-NTN:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n251 (NOTE 2) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n250 (NOTE 3) | 14000 MHz – 14500 MHz | 10700 MHz – 12200 MHz | FDD |
| n249 (NOTE 4) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from n256. NOTE 2: This band is applicable in Japan and the countries subject to CEPT. NOTE 3: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 4: This band is applicable in Taiwan, Australia |  |  |  |

Proposal3: Add the following NTN Ku-bands in the list of satellite operating bands in FR2-NTN:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 (NOTE 4) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n508 (NOTE 2) | 14000 MHz – 14500 MHz | 10700 MHz – 12200 MHz | FDD |
| n507 (NOTE 5) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable in Japan and the countries subject to CEPT. NOTE 5: This band is applicable in Taiwan, Australia |  |  |  |

## SAN channel bandwidths

As highlighted in our contribution analyzing the WI update impacts [2], the following FR1-NTN channel bandwidths 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz should be added, all new channel bandwidths supporting 30kHz SCS only.

Proposal4: Add the specification of the following transmission bandwidth configuration for FR1-NTN:

| SCS (kHz) | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | N/A | N/A | N/A | N/A | N/A |
| 30 | 65 | 92 | 133 | 189 | 273 |
| 60 | N/A | N/A | N/A | N/A | N/A |

Proposal5: Add the specification of the following minimum guard band for FR1-NTN:

| SCS (kHz) | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | N/A | N/A | N/A | N/A | N/A |
| 30 | 785 | 925 | 1045 | 965 | 845 |
| 60 | N/A | N/A | N/A | N/A | N/A |

## SAN channel bandwidths operating band

Based on the updated WI ([1]) and the past RAN4 agreements on NTN Ku-bands (see 2.2), we make the following proposals.

Proposal6: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR1-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n251 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n250 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n240 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Proposal7: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR2-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n507 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

## Channel raster for the NTN Ku-bands

It should be noted that the specification of the NR-ARFCN parameters is not impacted by the latest update to the WID.

Observation5: The current specification of the NR-ARFCN parameters for the global frequency raster remains valid even if FR1-NTN and FR2-NTN are overlapping.

Note that the step size could be further discussed based on satellite operators’ wish. To keep flexibility in the channel location, we propose then a step size of 2 for FR1-NTN and a step size of 8 for FR2-NTN.

Proposal8: The following NR-ARFCN for the NTN Ku-bands in FR1-NTN shall be added:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n251 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1250000 |
| n250 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1213333 |
| n249 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1250000 |

Proposal9: The following NR-ARFCN for the NTN Ku-bands in FR2-NTN shall be added:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333333– <8> – 1366667 | 1113333– <8> – 1250000 |
| n508 | 120 | 1333333– <8> –1366667 | 1113333– <8> – 1213333 |
| n507 | 120 | 1333333– <8> – 1366667 | 1113333– <8> – 1250000 |

## Synchronization raster for the NTN Ku-bands

It should be noted that the specification of the GSCN parameters for the global frequency raster is not impacted by the latest update to the WID.

Observation6: The current specification of the GSCN parameters for the global frequency raster remains valid even if FR1-NTN and FR2-NTN are overlapping.

Note that the proposed step size below could be further discussed. The proposed values below are based on:

- The proposed channel raster step size above.

- The maximum value sync raster step size that satisfies the equation:

ΔFSC,Raster ≤ BWConfig – BWSSB + ΔFCH,Raster

Proposal10: The following SS raster entries for the NTN Ku-bands in FR1-NTN shall be added:

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n251 | 30 kHz | Case C | 12850 – <1> – 14266 |
| n250 | 30 kHz | Case C | 12850 – <1> – 13884 |
| n249 | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal11: The following SS raster entries for the NTN Ku-bands in FR2-NTN shall be added:

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | [Case D] | 12864 – <12> – 14256 |
| n508 | 120 kHz | [Case D] | 12864– <12> – 13872 |
| n507 | 120 kHz | [Case D] | 12864– <12> – 14256 |

# Conclusion

In this contribution, we analyzed the system parameters for the new NTN Ku-bands considering the last update of the WID. We made the following observations and proposals:

Proposal1: Update the definition of NTN frequency ranges according to the following table:

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Observation1: Following the RAN4 agreement ([4]), the band 10.70 – 12.75 GHz (Space-to-Earth) and 13.75 – 14 GHz (Earth-to-Space) should be considered as second priority.

Observation2: There is no ECC Decision nor ETSI Harmonized standards covering the frequency range 13.75 – 14 GHz.

Observation3: FCC has allocated the frequency ranges 10.7-12.2 GHz and 14.0-14.5GHz for VMES and ESIM operations.

Observation4: For the NTN Ka-bands in US, RAN4 decided to allocate 2 bands: one for the frequency range allocated to ESIM and Fixed VSAT operations (n511) and one for the frequency range allocated to Fixed VSAT only (n510). RAN4 should better have the same approach for the NTN Ku-bands definition in US.

Proposal2: Add the following NTN Ku-bands in the list of satellite operating bands in FR1-NTN:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n251 (NOTE 2) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n250 (NOTE 3) | 14000 MHz – 14500 MHz | 10700 MHz – 12200 MHz | FDD |
| n249 (NOTE 4) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from n256. NOTE 2: This band is applicable in Japan and the countries subject to CEPT. NOTE 3: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 4: This band is applicable in Taiwan, Australia |  |  |  |

Proposal3: Add the following NTN Ku-bands in the list of satellite operating bands in FR2-NTN:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 (NOTE 4) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n508 (NOTE 2) | 14000 MHz – 14500 MHz | 10700 MHz – 12200 MHz | FDD |
| n507 (NOTE 5) | 14000 MHz – 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable in Japan and the countries subject to CEPT. NOTE 5: This band is applicable in Taiwan, Australia |  |  |  |

Proposal4: Add the specification of the following transmission bandwidth configuration for FR1-NTN:

| SCS (kHz) | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | N/A | N/A | N/A | N/A | N/A |
| 30 | 65 | 92 | 133 | 189 | 273 |
| 60 | N/A | N/A | N/A | N/A | N/A |

Proposal5: Add the specification of the following minimum guard band for FR1-NTN:

| SCS (kHz) | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | N/A | N/A | N/A | N/A | N/A |
| 30 | 785 | 925 | 1045 | 965 | 845 |
| 60 | N/A | N/A | N/A | N/A | N/A |

Proposal6: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR1-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n251 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n250 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
|  | 15 |  |  |  |  |  |  |  |  |  |  |
| n240 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |

Proposal7: The following SAN channel bandwidths and SCS for the NTN Ku-bands in FR2-NTN shall be added:

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n509 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n508 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n507 | 60 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 400 |

Observation5: The current specification of the NR-ARFCN parameters for the global frequency raster remains valid even if FR1-NTN and FR2-NTN are overlapping.

Proposal8: The following NR-ARFCN for the NTN Ku-bands in FR1-NTN shall be added:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n251 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1250000 |
| n250 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1213333 |
| n249 | 30 | 1333333 – <2> – 1366667 | 1113333– <2> – 1250000 |

Proposal9: The following NR-ARFCN for the NTN Ku-bands in FR2-NTN shall be added:

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333333– <8> – 1366667 | 1113333– <8> – 1250000 |
| n508 | 120 | 1333333– <8> –1366667 | 1113333– <8> – 1213333 |
| n507 | 120 | 1333333– <8> – 1366667 | 1113333– <8> – 1250000 |

Observation6: The current specification of the GSCN parameters for the global frequency raster remains valid even if FR1-NTN and FR2-NTN are overlapping.

Proposal10: The following SS raster entries for the NTN Ku-bands in FR1-NTN shall be added:

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n251 | 30 kHz | Case C | 12850 – <1> – 14266 |
| n250 | 30 kHz | Case C | 12850 – <1> – 13884 |
| n249 | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal11: The following SS raster entries for the NTN Ku-bands in FR2-NTN shall be added:

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | [Case D] | 12864 – <12> – 14256 |
| n508 | 120 kHz | [Case D] | 12864– <12> – 13872 |
| n507 | 120 kHz | [Case D] | 12864– <12> – 14256 |

# References

- RP-243314, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2501354, NTN Ku bands - WI update impacts, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat