# 3GPP TSG-RAN WG4 Meeting #114	R4-2501370

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:	(NR_NTN_enh-Core) Doppler shift issues for guard band and transmission bandwidth configuration

Agenda item:		5.21.1

Document for:		Approval

# Introduction

In last RAN4#113, it was proposed [1]) to introduce a new UE capability to indicate if this UE could support “deployment in NGSO scenario when the edge RB fall into the guard band”. This proposal was triggered by a potential issue of an RB shifted in the guard band due to Doppler shift.

No agreement was made and further discussion was expected in RAN4#114 meeting.

This contribution is further addressing this point, looking at the regulation and how Doppler is supposed to be handled.

# Discussion

## Radio Regulation definition

In chapter 1 (Terminology and technical characteristics), the Radio Regulations ([2]) provides the following definitions:

1.147	 assigned frequency band: The frequency band within which the emission of a station is authorized; the width of the band equals the necessary bandwidth plus twice the absolute value of the frequency tolerance. Where space stations are concerned, the assigned frequency band includes twice the maximum Doppler shift that may occur in relation to any point of the Earth’s surface.

1.152	necessary bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

Observation1: According to the Radio Regulations, the assigned frequency band includes twice the maximum Doppler shift that may on Earth to the NTN UE.

## SM 1541

According to ITU SM 1541 ([3]), the OoB domain emissions are defined according to the following:

Any emission outside the necessary bandwidth which occurs in the frequency range separated from the assigned frequency of the emission by less than 250% of the necessary bandwidth of the emission will generally be considered an emission in the OoB domain.

Observation: the out of band emissions are specified based from the assigned frequency edges based on the necessary bandwidth.

Observation2: According to SM 1541, the out of band domain is defined from the assigned frequency.

## Conclusion

Based on previous observations, the Doppler shift is considered in the assigned frequency band, meaning there is no issue with any potential RB shifting in the guard band of a NR channel bandwidth signal due to Doppler shift as it was mentioned in XX.

Proposal: There is no issue with any RB shifting in the guard band of a NR channel bandwidth signal due to Doppler shift. This Doppler shift is already handled in the assigned frequency band.

# Conclusion

In this contribution, based on the Regulation definitions, we further investigated the potential issue of RB shifting in the guard band of a NR channel bandwidth signal due to Doppler shift. We made the following observations and proposal:

Observation1: According to the Radio Regulations, the assigned frequency band includes twice the maximum Doppler shift that may on Earth to the NTN UE.

Observation2: According to SM 1541, the out of band domain is defined from the assigned frequency.

Proposal: There is no issue with any RB shifting in the guard band of a NR channel bandwidth signal due to Doppler shift. This Doppler shift is already handled in the assigned frequency band.

# References

- R4-2419057, (NR_NTN_enh-Core) Discussion on potential solution on Doppler shift issues for guard band and transmission bandwidth configuration

- Radio Regulations, ITU, Edition of 2024

- SM 1541.7, Unwanted emissions in the out-of-band domain, ITU, 09/2024