3GPP TSG-RAN WG4 Meeting #113	R4-2501378

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	7.26.2.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

# Topic #2: REFSENS, ASCS and ACS requirements ### Sub-topic 2-1 SNR simulation and values Issue 2-1-1: Target FR1 SNR value for LP-WUS/WUR Agreements: - Use worst case waveform of OOK-4 M=4 to derive SNR. - Some detailed aspects should be also confirmed, e.g., CRC bit (8), sequence length (8, 16), codepoint length of known part of the sequence) Issue 2-1-3: Link level simulation assumption for FR2 LP-WUR Agreements: - Further discussions and alignment on FR2 RF simulation assumption not precluded. - Use the following as starting point Attributes; Assumptions; Notes. Case name (waveform); OOK-1 waveform; OOK-4 waveform. Center frequency; 24300MHz; n258 example band. WUS structure; Total 8/16/32 bits; (placeholder for RAN1 decision). Channel structure; not needed. Chip rate; M=1; M=1/2/4. Coding; 1/2 rate Manchester coding. Time error; up to UE implementation; Difficult to separate time and frequency errors in realistic receivers. Residual Frequency error; 0/10/20 ppm. SCS; 120kHz. UE Channel BW; 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2. WUS RB; - Fixed 11RB; Based on agreement in RAN1#118-Bis. Position within channel; - Center for ASCS, edge for ACS [assume no ASCS impact]. Guardband of NR channel, both wanted cell and interfer cell (ACS); - Refer to 5.3.3 in TS38.101-2. Guard RB; - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB. Filter; - 3th/5th Order lowpass Butterworth - Other order lowpass filter is not precluded. ASCS; PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal. ACS; PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; NOTE: decide the interference level depending on SNR. Wanted signal level; For ACS, REFSENS + 14 dB for LP-WUS. Sampling rate; TBD. ADC bit width; 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison. Phase noise; FFS. Non-linearities; Not modelled. Power boosting; 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation. Channel Model; Option 1: TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options; The procedure for using CDL channel models in demod analysis is not fully established.. ### Sub-topic 2-2 NF and REFSENS requirements Issue 2-2-1: General views on REFSENS for LP-WUS Agreements: - Discuss REFSENS for the two types of receivers respectively, and then check if one single value or two values are specified pending on the final REFSENS values. Issue 2-2-3: companies input on SNR, NF and IM value for FR1 OOK-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OOK-based LR. (Moderator) FR1 OOK-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 3; 2.5; 2.5; 2.5; 1; 2.5. NF(dB) (assume MR=9dB); 17; 10; 17; 16; 15; 11.5; 15; 16; 12; 14; 12; 12. SNR(dB); -2; -7~-2; -3; -2; -1 for sequence length of 18; -2; -2.5; -2; -1.5~ -5. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Use 2.5dB IM as a starting point, encourage companies to report the difference aspects due to OOK waveform receiver. The corresponding band should also be reported. - The total value of NF+IM can be considered. - On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage - Capture the table in WF as background information Issue 2-2-4: companies input on SNR, NF and IM value for FR1 OFDM-based LR - Proposals - Proposal 1: check the following input from companies and converge the range for OFDM-based LR. (Moderator) FR1 OFDM-based receiver; Huawei; Sony; Spread trum; Apple; vivo; QC; Samsung; LGE; CMCC; ZTE; Nokia; E//. IM(dB); 2.5; 2.5; 2.5; >=2.5; 2.5; 2.5; 2.5; 2.5; 1; 2.5. NF(dB)(assume MR=9dB); 11; 10; 11; 16; 11; 12; 9; 11; 12; 12~13. SNR(dB); -2; -7~-2; -3; -2 or -5; -4.7; -5; -3.2. Note: the SNR value may be updated based on new simulation results. RAN1 waveform design is not finalized as of this WF. Agreements: - Further discuss next meeting Issue 2-2-6: Baseline OFDM-based architecture for FR2 LP-WUR Agreements: - FR2 LPWUR adopts zero-IF topology baseline assumption. FFS details - Multiple frequency conversion in RF domain is not precluded - For deriving a sensitivity requirement, the baseline OFDM-based FR2 LPWUR architecture is assumed to have the following features: - It comprises 2 Rx chains, and the antennas attached to the respective chains are mutually orthogonally polarized - FFS on whether and how to combine two Rx chains - Antenna gain is that of a single element - Coverage target should be considered ### Sub-topic 2-3 ASCS simulation and requirements Issue 2-3-1: Required number of guard RB for ASCS Agreements: - [0RB] as guard RB for ASCS. Issue 2-3-2: Detailed Test case for ASCS WF: - RAN4 further discuss the test case next meeting ### Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Phase noise for simulation assumption Agreements: - Companies can use different phase noise assumptions, the phase noise assumption should be reported Issue 2-4-2: General ACS requirements value and guard RB Agreements: - For ACS evaluation, companies are encouraged to provide the simulation results by assuming - Baseline: the same interference experienced by LP-WUR and main radio. - Optional: the different interference level for LP-WUR and main radio Issue 2-4-4: Test parameters for LP-WUR ACS case WF: - RAN4 further discuss detailed test parameters table and how to modify spec.

Discussion

Coverage

During RAN1#117, following agreement regarding the coverage assumption of LP_WUR was made [2]. In our understanding the SNR values defined here represent the coverage targets that RAN1 is using as a basis for signal design. The channel assumptions for these values are different compared to what RAN4 had agreed to derive the SNR value. This SNR value cannot be taken directly for RAN4 REFSENS calculations.

RAN4 is going to define the NF value based on feasible RF architecture. Further, based on RAN4 simulation assumptions, agreed in [3], required SNR value will be calculated based on RAN4 channel assumptions. However, to do that RAN4 has to wait for RAN1 to finish the signal design. Once that is done and RAN4 agrees to a SNR value, RAN4 can do a sanity check on the SNR number i.e., the SNR has to be higher than RAN1 target SNR based on LP-WUR NF value at least by the amount of the implementation margin.

Wait for RAN1 channel design before doing a sanity check regarding coverage assumption.

RAN4 proposed SNR shall be better than RAN1 target SNR by at least the implementation margin.

Implementation margin

For E-UTRA and NR in FR1, 2.5dB of IM had been used for the main receiver in case of 5 MHz channel bandwidth. However, for a LP-WUR there is no interference from its own transmitter as the transmitter should not be working because UE is in deep or ultra-deep sleep. Further, there is only a single receiver, thus no additional losses which can be attributed to multiple receivers.

Some companies have concerns regarding antenna sharing architecture and its impact on NF, and thus want to increase the IM. Our view is that it should be accounted under NF as that is the more appropriate place and not IM.

In our view, IM covers impairments related to BB artifacts, non-linearities, and channel estimation errors that are not accounted for anywhere else. Further, it accounts for production and temperature variations. Thus, we propose to tighten the IM to a value less than 2.5dB.

There is no self-desensitization because of own transmission for LP-WUR.

There are no multiple receivers associated losses for LP-WUR.

Implementation margin accounts for BB artifacts, along with production and temperature variations only.

Agree to have implementation margin to be 1dB for both OOK and OFDM based receivers.

Additional impact on NF because of antenna sharing architecture should be accounted for in NF, not in IM.

NF value

In the last meeting, various companies have presented different values of possible NF values. In order to converge on one number to be used for REFSENS calculations, multiple options were suggested in the WF, repeated below for readability.

- On how to handle the NF values: - Option 1: For NF whether average value can be adopted. - Option 2: two sets NFs for different LP-WUR types with different power consumption target - Option 3: also consider coverage

In our opinion, it is possible to arrive to a reasonable NF number by the averaging method. We believe that power consumption target is solely an LP_WUR implementation issue just like False alarm ratio, so it should not be used for classifying the LP_WUR. They should be solely classified based on detector type.

LP_WUR power consumption target is an implementation specific issue only.

Based on company inputs in RAN4#113, we can deduce that for envelope detector based LR a NF value of 14dB (based on input from 12 companies) and for OFDM based LR a NF value of 11.6dB (based on input from 10 companies.

LP_WUR power consumption target should not be used for NF classification.

For REFSENS calculations, use 14 dB as NF for an envelope detector based LP_WUR.

For REFSENS calculations, use 11.6 dB as NF for an OFDM based LP_WUR.

SNR value

As the channel design is still not complete at RAN1, so RAN4 should wait before deciding the final SNR values to be used in REFSENS calculations.

Wait for RAN1 to finish channel design.

ASCS requirement

The guard RBs for ASCS requirement have been tentatively agreed to [0] in the last meeting. Our simulation results have shown that 0 number of guard RBs are sufficient [4], thus it can be finalized. Further, as the interferer and LP-WUS signal has to have same PSD, so the only question remains is what that value should be. For legacy UEs, the ACS is tested at two different power levels, one at REFSENS + 14 dB and another at a fixed level of -56.5 dBm. This enables testing filter suppression performance across both optimal and worst-case dynamic range scenarios. In our opinion this power level should be significantly higher than the REFSENS level in order to eliminate the impact of noise. Thus, we propose to use a signal level of  REFSENSLR + 14 dB for the ASCS requirement tests. It has already been agreed that for ASCS case, the NR signal will have the same PSD as the wanted WUS signal. Given the WUS signal is 11 PRB wide and its signal level is specified in absolute terms, so NR signal should also be 11 PRBs wide.

NR Interferer signal will have the same PSD as the LP_WUS signal.

For the ASCS requirement side conditions, agree to have zero guard RB for the ASCS case i.e., there is no offset between NR signal and LP_WUS.

For the ASCS requirement side conditions, agree to have the power of LP_WUS and NR signal in dBm as REFSENSLP_WUR + 14 dBm.

For the ASCS requirement side conditions, agree to have the bandwidth of LP_WUS and NR signal to be fixed at 11 PRBs irrespective of SCS.

ACS requirement

Till now there has been no agreement on neither the ACS requirement value nor the side conditions for it. Given the limited capabilities of the LP_WUR, it is bound to have lower dynamic range compared to the MR. This makes it challenging to just reuse the legacy ACS requirements and side conditions for the LP_WUR. In order to figure out the requirements, LLS were conducted for the envelop-detector based LP_WUR with the simulation setup illustrated in in Figure 1. For the simulation two parameters are considered:

- The ACS interference level is swept from 0 to 31.5 dB,

- The test is performed with number of guard RBs ranging from 1 to 4.

Figure 1 Illustration of the simulation test setup.

In Figure 2 and Figure 3 the ACS simulation results are illustrated. For the AWGN channel it can be seen that the MDR=1% can be obtained when the interferer level is <17dB (guard of 1RB), <23dB (guard of 2RBs) and <28 dB (guard of 3RBs). The test with guard is 4RBs is not illustrated in the plot as the interferer can be > 31.5 dBm

Figure 2 Illustration of the simulation results for an AWGN channel with a guard band of 1-3 RB. The interfere level is swept from 0-31.5 dB.

The simulation of the ACS performance for TDLC300 channel is performed with guard of 1-4 RB. For the figure it can be seen that the MDR=1% can be obtained when the interferer level is <12dB (guard of 1RB), <17dB (guard of 2RBs) , <22 dB (guard of 3RBs) and <27 dB (guard of 4RB).

Figure 3 Illustration of the simulation results for a TDLC300 channel with a guard band of 1-4 RB. The interfere level is swept from 0-31.5 dB.

During the SI phase, there was no consensus on whether the guard RBs for ACS case have to be empty or any NR signal can go. In our opinion they should be empty as other NR signal can have PSD larger than WUS and this will impact the performance of LP_WUR. This means that these guard RBs are just waste of spectrum. Thus, there should be a balance between the ACS requirement and guard RBs. The NR interferer configuration can be reused from the legacy test parameters.

LP_WUR dynamic range will be less than the dynamic range of MR.

In AWGN channel, for an envelope detector receiver, four guard RBs are sufficient to satisfy the current MR ACS requirement.

With one guard RB, the ACS requirement has to be less than 17 dB.

There should be a balance between number of guard RBs and ACS requirement.

For the ACS requirement side conditions, agree to have two guard RBs.

Agree to have the ACS requirement as 22 dB.

Agree to have the BW of the interferer i.e. BWinterferer as 5 MHz.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Wait for RAN1 channel design before doing a sanity check regarding coverage assumption.

Proposal 2: RAN4 proposed SNR shall be better than RAN1 target SNR by at least the implementation margin.

Observation 1: There is no self-desensitization because of own transmission for LP-WUR.

Observation 2: There are no multiple receivers associated losses for LP-WUR.

Observation 3: Implementation margin accounts for BB artifacts, along with production and temperature variations only.

Proposal 3: Agree to have implementation margin to be 1dB for both OOK and OFDM based receivers.

Proposal 4: Additional impact on NF because of antenna sharing architecture should be accounted for in NF, not in IM.

Observation 4: LP_WUR power consumption target is an implementation specific issue only.

Proposal 5: LP_WUR power consumption target should not be used for NF classification.

Proposal 6: For REFSENS calculations, use 14 dB as NF for an envelope detector based LP_WUR.

Proposal 7: For REFSENS calculations, use 11.6 dB as NF for an OFDM based LP_WUR.

Proposal 8: Wait for RAN1 to finish channel design.

Observation 5: NR Interferer signal will have the same PSD as the LP_WUS signal.

Proposal 9: For the ASCS requirement side conditions, agree to have zero guard RB for the ASCS case i.e., there is no offset between NR signal and LP_WUS.

Proposal 10: For the ASCS requirement side conditions, agree to have the power of LP_WUS and NR signal in dBm as REFSENSLP_WUR + 14 dBm.

Proposal 11: For the ASCS requirement side conditions, agree to have the bandwidth of LP_WUS and NR signal to be fixed at 11 PRBs irrespective of SCS.

Observation 6: LP_WUR dynamic range will be less than the dynamic range of MR.

Observation 7: In AWGN channel, for an envelope detector receiver, four guard RBs are sufficient to satisfy the current MR ACS requirement.

Observation 8: With one guard RB, the ACS requirement has to be less than 17 dB.

Observation 9: There should be a balance between number of guard RBs and ACS requirement.

Proposal 12: For the ACS requirement side conditions, agree to have two guard RBs.

Proposal 13: Agree to have the ACS requirement as 22 dB.

Proposal 14: Agree to have the BW of the interferer i.e. BWinterferer as 5 MHz.

- References

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R1-2405708, Summary # 5 of discussions on LP-WUS and LP-SS design, Vivo, RAN1#117, Fukuoka, Japan, 20th – 24th May, 2024.

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, Vivo, RAN4#111, Fukuoka, Japan, 20th – 24th May, 2024.

- R4-2415707, Fundamental RX requirements for LP-WUR, Nokia, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.