# 3GPP TSG-RAN WG4 Meeting #114	R4-2501399

Athens, Greece, 17th – 21st February 2025

Source:			Ericsson

Title:				On co-existence simulation results for NR NTN Ku band

Agenda item:		7.11.2

Document for:		Information

# 1	Introduction

In 3GPP RAN#104, a new Work Item (WI) was approved for Introduction of Ku band for NR NTN. The WI was further updated in RAN#106 [1]. One of the objectives of the WI is to –

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

WF [2] captured initial agreements on co-existence simulation assumptions in the last meeting.

This contribution provides co-existence simulation results based on the agreements so far.

# 2	Discussion

In the following sections, we present our simulation results based on the agreed simulation parameters in WF [2]. We have only simulated on the 14 GHz scenarios, as we have a proposal in our companion contribution [3] to use TN BS antenna parameters for 11 GHz frequency from the TR 38.922 study on 8 GHz. This section is further divided based on the NTN scenarios – GEO and LEO600 for GSO.

## 2.1 	GEO

In this sub-section, we share the simulation results for Case 1-4 for GEO NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees, 25 degrees and 35 degrees.

  - Additional information for 35 degrees is presented in [3].

- Fixed VSAT and Mobile VSAT

  - We also present results for Fixed VSAT as some companies showed interest during the last meeting.

  - Mobile VSAT is L-ESIM.

- VSAT NF is 2.5 dB and 6 dB

- VSAT antenna pattern is parabolic.

| ACIR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |  |  |  |  |
|  |  |  |  | 90 degrees |  |  | 25 degrees |  |  | 35 degrees |
|  |  |  |  | Mobile VSAT | Fixed VSAT |  | Mobile VSAT | Fixed VSAT |  | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 6 dB NF | 2.5 dB NF | 2.5 dB NF | 6 dB NF | 2.5 dB NF |
| VSAT UL | UE UL | 1 | 14 GHz | 1 | 0 | 0 | 12 | 17 | 17 |  |
| UE UL | VSAT UL | 2 | 14 GHz | 16 | 16 | 16 | 22 | 20 | 20 |  |
| VSAT UL | BS DL | 3 | 14 GHz | 0 | 0 | 0 | 0 | 0 | 0 |  |
| BS DL | VSAT UL | 4 | 14 GHz | 6 | 6 | 6 | 36 | 36 | 36 | 26 |

Higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 10 dB at 35 degrees when compared to 25 degrees elevation angle.

## 2.2	LEO600

In this sub-section, we share the simulation results for Case 1-4 for LEO600 NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees, 25 degrees and 35 degrees.

  - Additional information for 35 degrees is presented in [3].

- Fixed VSAT and Mobile VSAT

  - We also present results for Fixed VSAT as some companies showed interest during the last meeting.

  - Mobile VSAT is L-ESIM.

- VSAT NF is 2.5 dB and 6 dB

- VSAT antenna pattern is parabolic.

| ACIR |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Elevation angle |  |  |  |  |  |  |
|  |  |  |  | 90 degrees |  |  | 25 degrees |  |  | 35 degrees |
|  |  |  |  | Mobile VSAT | Fixed VSAT |  | Mobile VSAT | Fixed VSAT |  | Mobile VSAT |
| Aggressor | Victim | Scenario | Frequency | 2.5 dB NF | 2.5 dB NF | 6 dB NF | 2.5 dB NF | 2.5 dB NF | 6 dB NF | 2.5 dB NF |
| VSAT UL | UE UL | 1 | 14 GHz | 0 | 0 | 0 | 8 | 22 | 22 |  |
| UE UL | VSAT UL | 2 | 14 GHz | 16 | 16 | 16 | 12 | 12 | 12 |  |
| VSAT UL | BS DL | 3 | 14 GHz | 0 | 0 | 0 | 0 | 0 | 0 |  |
| BS DL | VSAT UL | 4 | 14 GHz | 9 | 9 | 9 | 31 | 32 | 32 | 22 |

Higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 9 dB at 35 degrees when compared to 25 degrees elevation angle.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 10 dB at 35 degrees when compared to 25 degrees elevation angle.

Observation 2	Higher throughput is observed at 35 degrees elevation angle, resulting in the ACIR value to drop by 9 dB at 35 degrees when compared to 25 degrees elevation angle.

# References

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2405874, “WF for NR_NTN_Ku_Band_General”, 3GPP RAN4#113, Moderator

- R4-2501398, “On Co-existence simulation assumptions for NR NTN Ku band”, Ericsson, 3GPP RAN4#114