3GPP TSG-RAN WG4 Meeting #114	R4-2501404

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On Event-triggered L1 reporting requirements for LTM

Agenda item:	7.27.2.1

Document for:	Discussion

Introduction

In this contribution we continue to discuss L1 event-triggered measurement reporting requirements for LTM based on the RAN4#113 WF R4-2420101.

Discussion

Event-triggered reporting delay

It has been agreed by RAN4 that the event triggered reporting delay is the time from when the condition that triggers the report exists at the UE, until the UE starts to transmit the report over the air interface.

What remained open is the evaluation delay, which is the time from when the reporting condition becomes fulfilled until the UE realizes the condition is fulfilled and triggers the report. It was captured on a high level agreement under issue 2-5 that the delay shall be at maximum the time it takes the UE to measure the cells that are part of the event that triggers the report:

Issue 2-5: Event triggered reporting delay for L1 reporting <Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

L1 measurement period for Rel-18 LTM SSB-based measurements takes into account all the candidate cells the UE is measuring. The number of cells may depend on the TCI state activation status. The UE will be measuring all cells according to the requirements during the event evaluation, and it is not defined when exactly the UE measures each of the configured cells within this time. Hence, the time it may take for the UE to realize the condition for reporting is met may require at maximum the whole L1 measurement period. Therefore, we propose to reuse the SSB-based L1 measurement period for the reporting delay.

When the delay involves both serving and candidate cell, then the delay should be the maximum time it takes for the UE to measure both serving cell and candidate cell. Treport component can be ignored. For CSI-RS based event-triggered reporting, evaluation delay to follow after measurement requirements are available.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement delay for the cells involved in the event, with Treport ignored.

Issue 2-6: impact of time-to-trigger (TTT) on RRM <Candidate options> - Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi) <Recommended WF> - Continue discussion.

Time-to-trigger, similar to L3 event-triggered reporting, is agreed to be used for L1 based event-triggered reporting based on network configuration. The details are not fully complete, but the idea is the same as in L3 event-triggered reporting. Network may configure TTT which tells the UE to trigger a report only if the condition continues to be met for the configured time duration (TTT). This is to prevent unnecessary reports based on too fast UE decisions.

In L3 event-triggered reporting delay, TTT was not taken into account in the test cases or the reporting delay. However, it is clear that when TTT is configured, the reporting delay is longer than if sending the report is based on a single measurement, as the UE shall not report unless the condition continues to be met for the whole TTT duration. We think for requirement clarity, it would be good to include description of this aspect also in the RAN4 specification.

Clarify in the event-triggered L1 reporting delay requirement that if TTT is configured, the UE shall only trigger the report after TTT, if condition continues to be met.

Issue 2-15: Impact of UL resource waiting time. <Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. <Recommended WF> - Continue discussion.

Regarding the time the UE is waiting for UL reporting resources, we think similar approach as for L3 event-triggered reporting can be used. Meaning, this time is considered as an uncertainty time and is not taken into account in the reporting delay as a separate component.

Reuse L3 event-triggered reporting approach regarding the UE waiting time for UL reporting resources i.e. do not specify a specific delay component for this time.

Option 1 under issue 2-15 discussed the scenario where the UE performs measurements during the time it is waiting for reporting resources. Whether the UE reports this new measurement or the one that triggered the report, and whether the new measurement is to be considered for event evaluation, is in our view to be defined by RAN1/2, not RAN4.

It is up to RAN1 to define how the UE handles measurement that is performed during the time UE is waiting for reporting resources.

Impact on other LTM requirements

In the last meeting it was requested by the moderator to provide more detailed analysis on the following issue:

<Candidate solutions> - Option 1: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: (Nokia, E///) - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements - Option 2: RAN4 to postpone discussions on the options listed under ‘Issue 1-12: Impact on other LTM requirements’ in the WF (R4-2416862) approved at RAN4#112bis until the group achieves a certain level of detailed requirements for the other components. (QC) <Agreement> - Proponents of option 1 to bring detailed analysis on the potential impact.

Regarding the known TCI state condition with event-triggered reporting, companies have preferred not to change the legacy definition, and hence we believe event-triggered reporting has no impact on the known TCI state condition itself. However, following the known TCI state condition for cell switch delay requirement applicability in FR2 means that cell switch command needs to be sent within 1280 ms from the event-triggered L1 report. Otherwise, the target TCI state is unknown and the requirements do not apply. Network needs to send TCI state activation command, PDCCH order, and cell switch command within this time for the relevant requirements to apply. For FR1, a L3 report with SSB index within 5 s is sufficient to guarantee requirement applicability, which is less limiting, but for FR2, a L1 report is needed within 1280 ms as the target TCI state needs to be known.

In FR2, cell switch and TCI state activation delay requirements only apply for 1280 ms after the UE has sent the event-triggered L1 report, if there is no report after this, as otherwise the target TCI state is not known.

Event-triggered report may be a one-time report or the event may trigger periodic reporting. Additionally, network may configure the UE to send report “on leave” when the event condition stops being fulfilled. When a single event-triggered report is sent, the condition may remain met and the UE continues to measure after the report, but according to the current requirements, after 1280 ms the requirements stop applying.

As the event-triggered reporting is not so frequent, we think that it would make sense to update the requirement applicability to relax the limitation of Observation 1, when the UE is configured with event-triggered L1 reporting. If the reporting condition continues to be met (e.g. UE has not sent report on leave), we think the requirements for cell switch and TCI state activation delay should apply until the UE receives a cell switch command.

If the UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, the related delay requirements are applicable.

As we have proposed in the previous meetings, we think event-triggered report can be used as one condition to trigger early ASN.1 decoding and DL synchronization:

Issue 2-17: Impact on fast ASN.1 decoding <Candidate solutions> - Option 1: Event-triggered L1 report or reporting condition becoming fulfilled triggers fast ASN.1 decoding. (Nokia) - Option 2: postpone. (QC) <Recommended WF> - Continue discussion. Issue 2-18: Impact on early TCI state activation <Candidate solutions> - Option 1: Event-triggered L1 report or reporting condition becoming fulfilled triggers DL synchronization for the candidate cell’s active TCI states, if not valid at the time of report triggering. (Nokia) - Option 2: postpone. (QC) <Recommended WF> - Continue discussion.

More specifically, what we propose is:

In addition to the existing conditions, TLTM_RRC-processing = 0 if the UE has sent event-triggered L1 report for the target cell before receiving the cell switch command.

In addition to the existing conditions, Tfirst-RS = 0 if the target TCI state is on the active TCI state list and the UE has sent event-triggered L1 report within X ms before receiving the cell switch command.

We think these additional conditions are a logical addition, as the event-triggered report is a clear indication that the candidate cell the report considers is the most likely target cell.

Furthermore, we have proposed to use event-triggered report as a similar condition as TCI state activation command to focus the L1 measurements on the cell(s) that fulfil the event condition:

Issue 2-19: Impact on L1 measurement <Candidate solutions> - Option 1: RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cells fulfilling the reporting condition. (Nokia) - Option 2: postpone. (QC) <Recommended WF> - Continue discussion.

We continue to propose this condition in addition to the TCI state activation condition:

RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cell(s) fulfilling the reporting condition.

UE capabilities

Regarding UE capability of the number of supported events, there were some proposals in the previous meetings.

Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category <Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> - Continue discussion.

Although it may be too early to make agreements on the exact number of events to be supported, our view is that L1 events need to be added on top of the existing L3 events in the related requirements.

Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS.

Issue 2-10: whether and how to coordinate with R19 MIMO <Candidate options> - Option 1: (MTK) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events. - Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple) - Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia) <Recommended WF> - Continue discussion.

Regarding whether to coordinate or define a common capability between MIMO and mobility events, we do not think these capabilities should be considered together. These are two separate features, and the related work items are ongoing in parallel, so coordination is not straightforward and UE may also not support both features.

Do not define a shared capability for MIMO UEIBM and mobility LTM L1 events to be supported.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement delay for the cells involved in the event, with Treport ignored.

Proposal 2: Clarify in the event-triggered L1 reporting delay requirement that if TTT is configured, the UE shall only trigger the report after TTT, if condition continues to be met.

Proposal 3: Reuse L3 event-triggered reporting approach regarding the UE waiting time for UL reporting resources i.e. do not specify a specific delay component for this time.

Proposal 4: It is up to RAN1 to define how the UE handles measurement that is performed during the time UE is waiting for reporting resources.

Observation 1: In FR2, cell switch and TCI state activation delay requirements only apply for 1280 ms after the UE has sent the event-triggered L1 report, if there is no report after this, as otherwise the target TCI state is not known.

Proposal 5: If the UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, the related delay requirements are applicable.

Proposal 6: In addition to the existing conditions, TLTM_RRC-processing = 0 if the UE has sent event-triggered L1 report for the target cell before receiving the cell switch command.

Proposal 7: In addition to the existing conditions, Tfirst-RS = 0 if the target TCI state is on the active TCI state list and the UE has sent event-triggered L1 report within X ms before receiving the cell switch command.

Proposal 8: RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cell(s) fulfilling the reporting condition.

Proposal 9: Number of supported L1 events is to be added on top of the currently supported L3 events. Details FFS.

Proposal 10: Do not define a shared capability for MIMO UEIBM and mobility LTM L1 events to be supported.