3GPP TSG-RAN WG4 Meeting #114	R4-2501406

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On L3 faster beam sweeping requirements

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion about L3 beam sweeping factor reduction.

Discussion

Condition for triggering FBS for measurements

In the last meeting it was agreed that a signal quality-based condition is to be used for L3 measurements to trigger multi-Rx for FBS. Mobility status-based condition is not to be introduced in Rel-19. Earlier it was agreed that the condition is to be configured by the network. See agreement from [1]

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF (including activation condition and de-activation conditions): Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

One obvious aspect that needs to be clarified is whether the serving cell condition for FBS is to be introduced as a new network-configured condition or is some existing signal-based condition to be reused for triggering FBS. Such existing condition can be e.g. the A2 event condition for event-triggered L3 reporting (serving cell signal quality becomes worse than threshold), as we have proposed in the previous meetings. The details of the condition configuration are up to RAN2, but we think it would be good that RAN4 at least clarifies whether the intention is to introduce a new condition or to reuse the existing condition potentially with some additional signalling/indication.

RAN4 needs to make an agreement whether the configured signal-based condition for FBS is provided to the UE through a new configuration or whether an existing condition can be reused to trigger FBS.

Another aspect to discuss is to clarify the visibility of how the UE is measuring between the UE and the network. When triggering of FBS is purely based on a condition, the condition becoming met is not visible to the network. If the condition is the same as a reporting condition, then the network would know from a received report how the UE is measuring. Due to this reason, we think a preferable option is to reuse the reporting condition to trigger FBS.

RAN4 to agree that the configured signal-based condition for FBS is provided to the UE using the existing event-triggered reporting condition (e.g. event A2 condition). RAN2 to define the related signalling based on RAN4 LS.

On top of the signal-based condition, we think it still makes sense that conditional handover configuration would trigger FBS i.e. FBS should apply when the UE is configured with conditional handover condition. One goal of CHO was to improve robustness through letting the UE execute the HO when condition becomes fulfilled, to avoid failure due to too late handover command. If UE is doing FBS, it can realize such condition to become fulfilled faster, which contributes to the robustness goal.

Conditional handover configuration to trigger FBS.

Furthermore, there was discussion about the exit condition for FBS. We understand that it is not feasible from the UE power saving point of view to be mandated to perform fast beam sweeping for a long time duration, which is why an exit condition is understandably needed. An obvious condition to exit FBS would be that the condition to enter FBS is no longer met. However, some companies also proposed to introduce a timer to exit FBS mode. Also, if event A2 is reused to trigger FBS, then network can also configure event A1 or event on leave reporting as an exit condition, as we proposed in the last meeting.

UE may exit FBS mode if the condition to enter FBS mode is no longer met.

In case RAN4 decides to deactivate FBS based on a timer, we should also analyse whether the FBS should be reactivated under certain condition. We give one example considering the case that RAN4 agrees on a timer deactivation. If the UE moves to the cell edge and stops, the FBS will be activated for the duration of the timer. If after some time the UE moves again away from the cell centre, the FBS would be deactivated, and slow measurements might not be enough to trigger fast mobility actions. Therefore, if a timer is introduced, it is necessary to define a condition to reactivate the FBS after the timer expires on the cell edge. In one example, the UE could monitor the RSRP variation after entering cell edge to decide whether FBS is reactivated.

RAN4 to discuss how to reactivate FBS if exit condition including timer for deactivation is agreed.

SSB processing delay

In the last meeting it was agreed to add 2 ms in the measurement period due to additional processing time the UE needs due to measuring more than one beam simultaneously.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC Agreement: - For scenario 1/2/3/4 - additional processing time can be added after the last SMTC periodicity for intra/inter-frequency L3 measurement. - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra +TSSB_processing) ms - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_pe riod_inter +TSSB_processing) ms - where TSSB_processing = 2 ms Agreement: - For scenario 7 - to consider an additional processing delay (2ms) after the last SMTC periodicity [Wayforward] - For scenario 5 - FFS: - Option 1: to consider an additional processing delay (2ms) in the handover delay equation - Option 2: In the handover delay equation, SSB processing time (2ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Option 3: no need to add additional processing time in the delay requirement.

For scenario 5 (handover delay), some companies wanted to add a similar delay. However, in this case we see no justification for such an additional delay.

The handover delay is illustrated in Figure 1 below. Beam sweeping applies only for Tsearch, which is different from zero only if the target cell is unknown. For an unknown intra-frequency cell Tsearch = N* Trs. For an unknown inter-frequency cell Tsearch = N*3* Trs. After Tsearch comes T∆, which is time for fine time tracking and acquiring full timing information of the target cell. T∆ =  Trs. During T∆ the UE is waiting for the SSB of the target beam and after measuring this, there is Tmargin, which is the SSB processing time 2 ms. Tmargin is 2 ms regardless of whether Tsearch is zero or non-zero. Hence, following the same logic as for the measurements, the UE can process each SSB after it measures it during Tsearch and the last measured SSB during Tsearch can be processed within the time that is given for T∆. There is no justification to extend Tmargin, which is the processing time related to T∆, as during T∆ the UE is not doing beam sweeping.

Figure 1: HO delay

No additional processing time is introduced in handover delay due to fast beam sweeping.

Transition between FBS and non-FBS

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2418276.

With the transition between FBS mode and non-FBS mode, the intention is to clarify the measurement period that applies during the transition. In the legacy, similar requirement is defined e.g. for transition between DRX and non-DRX, where the longer measurement period applies during the transition.

For FBS, as this is not triggered by a network message but a condition, the moment when the UE switches to FBS mode is not visible to the network, or in a test scenario, to the test equipment. Hence, we think such transition period is quite difficult to define, as it is not clear when the transition exactly begins. Unless this becomes clear through some agreement, we do not think there is a need to define requirements for transition between FBS and non-FBS.

Unless the point at which UE switches between FBS and non-FBS is clear, there is no need for requirements for the transition period.

How to capture the new requirements for FBS

In the last meeting it was agreed that the N-factor when UE is supporting Rel-19 FBS depends on UE capability.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Agreement: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability: - FFS: details of the capability

Hence, N may get different values such as 2, 4, or 6, the normal value being 8. It remained open how such reduction would be captured in different requirements. We analyze the requirements in the following.

Scenario 1-4 (measurement delay):

Beam sweeping is taken into account in the M-factors, where for PC3 the factor is defined as:

Mpss/sss_sync_w/o_gaps = 24

Mmeas_period_w/o_gaps = 24

Same values apply for intra- and inter-frequency measurements with and without gaps. The BSF 8 is hence merged in the M-values. When BSF is reduced, either the M value needs to be changed or an additional BSF reduction factor needs to be defined. For this case we think it is simpler to define an additional scaling factor that takes the beam sweeping reduction into account.

For L3 measurement delay requirements, cover the impact of Rel-19 BSF reduction through an additional scaling factor NFBS = [reduced BSF]/8, where the reduced BSF depends on UE capability.

Scenario 5 (handover delay):

In handover requirements, beam sweeping is taken into account in the Tsearch component, where as an example, for an unknown intra- and inter-frequency target cell in FR3:

Tsearch = N* Trs ms (N=8)

Here the beam sweeping factor reduction can either be implemented similar to Rel-18 multi-Rx beam sweeping factor reduction by defining different values for N, or through an additional factor for a BSF reduction capable UE. For this case we propose to use the same approach as for multi-Rx i.e. define N-value depending on the UE capability of BSF reduction.

For HO delay requirements, define the value for N in Tsearch depending on UE capability for BSF reduction.

Scenario 7 (RRC re-establishment delay):

For RRC re-establishment, the impact of faster beam sweeping would apply for the definition of Tidentify_intra_NR and Tidentify_inter_NR, i. These are defined in tables 6.2.1.2.1-1 and 6.2.1.2.1-1.

Here RAN4 will need to discuss how to take into account BSF reduction in both values of the max-function. An additional factor for faster beam sweeping capable UE may be the simplest solution to capture the reduction so that it applies for both sides of the max-function.

For RRC re-establishment delay, o apply the scaling factor NFBS to the fixed value of Tidentify_intra/inter_NR e.g., NFBS x MAX (1000 ms, 104 x TSMTC, i), where NFBS = [reduced BSF]/8.

Furthermore, RAN4 has agreed about the applicability of FBS requirements, which needs to be captured in the specification. To avoid repeating the same text in multiple parts of the specification, we propose to use the same approach as for Rel-18 multi-Rx work item, where the applicability of the requirements was defined in section 3.6 and the requirements referred to this section.

Capture the applicability of L3 FBS requirements in section 3.6.

Conclusion

The following observations and proposals were made in this paper:

Observation 1: RAN4 needs to make an agreement whether the configured signal-based condition for FBS is provided to the UE through a new configuration or whether an existing condition can be reused to trigger FBS.

Proposal 1: RAN4 to agree that the configured signal-based condition for FBS is provided to the UE using the existing event-triggered reporting condition (e.g. event A2 condition). RAN2 to define the related signalling based on RAN4 LS.

Proposal 2: Conditional handover configuration to trigger FBS.

Proposal 3: UE may exit FBS mode if the condition to enter FBS mode is no longer met.

Proposal 4: RAN4 to discuss how to reactivate FBS if exit condition including timer for deactivation is agreed.

Proposal 5: No additional processing time is introduced in handover delay due to fast beam sweeping.

Proposal 6: Unless the point at which UE switches between FBS and non-FBS is clear, there is no need for requirements for the transition period.

Proposal 7: For L3 measurement delay requirements, cover the impact of Rel-19 BSF reduction through an additional scaling factor NFBS = [reduced BSF]/8, where the reduced BSF depends on UE capability.

Proposal 8: For HO delay requirements, define the value for N in Tsearch depending on UE capability for BSF reduction.

Proposal 9: For RRC re-establishment delay, o apply the scaling factor NFBS to the fixed value of Tidentify_intra/inter_NR e.g., NFBS x MAX (1000 ms, 104 x TSMTC, i), where NFBS = [reduced BSF]/8.

Proposal 10: Capture the applicability of L3 FBS requirements in section 3.6.

- References

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1