3GPP TSG RAN WG4 Meeting #114	R4-2501413

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Agenda item:			7.17.2.2

Source:	vivo

Title:	Further discussion on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

    - Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement in RAN#112 meeting: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

- Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority.

- Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

  - Aspect 1: SSB based Intra-frequency measurement without MG, including:

    - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

    - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

  - Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered.

- Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion.

Proposal 1: For UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, OK with option 3 or option 1.

    - Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Option 1 (Nokia): To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards.

- Option 2 (Ericsson): We suggest covering SA, EN-DC and NE-DC in the scope without deprioritization any of them.

Consider the practice in the field option 1 is preferred.

Proposal 2: For issue 2-3, option 1 is preferred.

Issue 2-4: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]:

FFS the following solution:

  - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured,

    - Reuse the legacy principle for neighbour cell measurement

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band or

      - Measure SCC with neighbor cell MO or

      - UE implementation

    - FFS whether indication is per-UE or per-band

      - Other options are not precluded.

  - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Proposal 3: For Issue 2-4-1: Tentative solution 1: measure one serving CC per band, OK with the recommended solution in the WF. Other NW indication method could be FFS.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

- Option 1 (CTC, Apple, HW, vivo, OPPO, ZTE, Ericsson): UE can change the searcher occupancy ratio of measurement among the measured CCs

  - Option 1A(CATT): UE can change the searcher occupancy ratio of measurement among the measured SCCs based on network indication, e.g., NW can indicate the priority of the SCCs. If the priority is not provided, the measurement order is up to UE implementation which is same as the legacy.

  - Option 1B(Nokia, CATT): The solution of changing searcher occupancy ratio shall not relax the measurement requirement on PCC/PSCC measurement.

  - Option 1C(CTC, Apple, HW, vivo, OPPO, ZTE, Ericsson): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

    - (Apple) The conditions can be not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

    - (vivo) Dynamic ratio changed as, e.g.,

      - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

      - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

      - FFS details and X value, e.g., 50.

      - FFS whether specific conditions for this ratio change are needed

    - (Ericsson, ZTE) NW indicates the ratio between PCC and SCC

      - When two searchers are applied, NW to indicate the candidate PCC/PSCC/SCC(s) or the frequency range of the PCC/PSCC/SCC(s) which applies a specified percentage X% of the searcher resource, the other SCC(s) together with PCC/PSCC share the other (1-X)% of the searcher resource. If no NW indication, UE follows the legacy searcher sharing rules. (Ericsson)

      - Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point. (Ericsson)

- Recommended WF

  - Moderator option 2 for merging all options based on option 1C, please CATT and Nokia check if it’s acceptable

  - Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

    - NW indicates the ratio between PCC, PSCC and SCC.

      - Measurement ratio is configurable by NW, the MeasGapSharingScheme in MG can be a start point.

    - Details is FFS

Proposal 4: For issue 2-4-2, the following items of option 1C is preferred and option 2 in the WF is also OK.

UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

- Dynamic ratio changed as, e.g.,

  - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

  - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

  - FFS details and X value, e.g., 50.

- FFS whether specific conditions for this ratio change are needed

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is:

    - FFS: FR1 scenario (CA and NR-DC) only.

    - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

    - FFS: All scenarios.

Regarding the 3 searchers, for this WI, our preference is the 3 searcher scenario should be limited and we are ok to consider 3 searchers for either FR1 scenario (CA and NR-DC) only or FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

Proposal 5: For 3 searchers for CSSF, consider 3 searchers for either FR1 scenario (CA and NR-DC) only or FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Proposal 1: For UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, OK with option 3 or option 1.

Proposal 2: For issue 2-3, option 1 is preferred.

Proposal 3: For Issue 2-4-1: Tentative solution 1: measure one serving CC per band, OK with the recommended solution in the WF. Other NW indication method could be FFS.

Proposal 4: For issue 2-4-2, the following items of option 1C is preferred and option 2 in the WF is also OK.

UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

- Dynamic ratio changed as, e.g.,

  - PCC/PSCC (1st searcher 100%) to PCC/PSCC (1st searcher X%);

  - SCC (1st searcher 0% and 2nd searcher 100%) to SCC (1st searcher 1-X% and 2nd searcher 100%);

  - FFS details and X value, e.g., 50.

- FFS whether specific conditions for this ratio change are needed

Proposal 5: For 3 searchers for CSSF, consider 3 searchers for either FR1 scenario (CA and NR-DC) only or FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 113, Apple