3GPP TSG-RAN WG4 Meeting #114	R4-2501417

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Source: 	vivo

Title: 	Discussion on simulation assumptions and results for LP-WUR

Agenda Item:	7.26.4.1

Document for:	Discussion

Introduction

In RAN Plenary #102 meeting, WI for “Low-power wake-up signal and receiver for NR” was agreed [1]. The WI was further updated at RAN Plenary #103 meeting as [2]. The initial discussion on simulation assumptions was triggered at RAN4 111 meeting and the simulation assumptions and results were further discussed at following RAN4 meetings. In this contribution we provide our further consideration on simulation assumptions and also provide further simulation results from our side.

Discussion

The objectives of WI are duplicated as below:

The objectives of the work item are the following:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

  - Specify necessary RRM requirements

The WF and contribution summary for this WI at RAN4 113 meeting are [3] and [4]. In this contribution, we provide our considerations on LP-WUR simulation assumptions and initial simulation results.

On simulation assumptions and parameters

Based on [3], the following issues are further discussed.

Issue 2-1-1: SINR setting

- Proposals

  - P1: Include SINR = -6dB to the {-3dB; -0.5dB; 2dB} set for simulation assumptions. FFS whether -6dB is a low priority case. (Apple MTK)

  - P2: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR. (Huawei)

Proposals for this issue point to different directions where P1 suggest besides all existing SINR values, one extra value -6dB should be considered whereas P2 suggests to firstly consider -3dB.

Regarding -6 dB, in [R1-2409243] the following agreements are available:

Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority))

To our understanding if -6 dB is included, which means more lower SINR is considered, then the necessity to have high SINR such as 2dB is not strong. Based on current simulation results so far, most simulation results from companies using -3dB to check how many samples will be used to achieve target 3.5 dB accuracy. To combine P1 and P2 and further align with RAN1 agreement, it is suggested -3dB SINR is used as the base and -6 dB is low priority and depending on company’s input.

- Proposal 1: In RAN4 simulation, -3dB SINR is used as the side condition of accuracy requirements for both OOK based and OFDM based LR. -6 dB SINR is low priority and depending on company input.

Issue 2-1-3: Time/frequency error

For the time/frequency error, we have the following agreement at RAN4 112bis meeting based on RAN1 agreement:

Residual timing error:

T= 5us for OOK-1

T= 2us for OOK-4 with M=2

T= 1us for OOK-4 with M=4

To our understanding, in the simulation some companies will perform sync before RRM measurement hence the residual timing error will be input from sync part to the RRM measurement part and the maximum value should be less than the above agreements. Some companies may simulate the RRM measurement performance directly and the above agreed residual timing error can be used as the input for the RRM measurement part. In our view it is not necessary to mandate the simulation methodology, i.e., either perform sync before RRM measurement or not, hence the above agreed residual timing error will be viewed as the upper bound of the timing error after sync. In practical simulation the value can still be better than these values.

- The agreed residual timing error is the upper bound of the timing error after sync process.

Issue 2-1-4: On LP-SS sequence for simulation purpose

- Proposals

  - P1: Define at least {M,L} pairs {2, 8} and {4, 16} for alignment purposes. FFS other pairs, upon need. Define at least 2 binary sequences, one for L=8 and one for L=16, to be agreed offline during the week of RAN4#113 preferably (Apple)

  - P2: Define a specific binary sequence for serving cell and a specific binary sequence for the interference cell. Interfering binary sequence shouldn’t be known to the UE, but also should not be random (Apple)

  - P3: RAN4 to align the LP-SS binary sequence pattern for M=4, such as Cell 1: [1010] Cell 2: [0101] (Ericsson)

  - P4: For OOK-based LP-WUR, use LP-SS binary sequence L=24 for OOK-4 (M=2) and L=56 for OOK-4 (M=4). The binary sequence is selected to meet the target timing accuracy agreed by RAN1 and RAN4. (MTK)

Regarding LP-SS sequence for simulation purpose, to our understanding RAN1’s plan is to fix the LP-SS sequence in the RAN1 120 meeting. Hence the LP-SS sequence for simulation will be available after RAN4 113 meeting. On the other hand LP-SS sequences with the following {M, L} are also provided for RAN4 alignment in case the RAN1 plan is changed.

LP-SS sequence:

- M=1 LP-SS Len=8 	seq:1 0 1 0 1 0 1 0;

- M=2 LP-SS Len=8 	seq:1 0 0 1 1 0 0 1;

- M=4 LP-SS Len=16 	seq:1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1;

Proposal 2: LP-SS sequence used for RAN4 simulation should be updated based on RAN1’s conclusion after RAN1 #120 meeting. In case no detail LP-SS sequences after RAN1 #120, the following LP-SS sequence could be used for RAN4 simulation alignment.

- M=1 LP-SS Len=8 	seq:1 0 1 0 1 0 1 0;

- M=2 LP-SS Len=8 	seq:1 0 0 1 1 0 0 1;

- M=4 LP-SS Len=16 	seq:1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1;

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

- Proposals

  - P1: Ideal RSRP is derived from the serving cell signal after the fading channel; Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI (Ericsson)

  - P2: Suggest to discuss and align the ideal RSRP calculation (vivo)

  - Method 1: The first one is the RSRP is pass the fading channel and with the perfect channel estimation and the real RSRP pass the same channel.

  - Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel.

Regarding how to obtain ideal RSRP, especially on ideal RSRP under fading channel (TDL-C), traditionally the RSRP signal is passed the fading channel and with the assumption that perfect channel estimation will be applied to the ideal RSRP, the impact from the fading channel can be eliminated in the end, which is a reasonable assumption for traditional MR type receiver which has strong capability. However, for the OOK based receiver, if RSRP is passed by the fading channel, due to the limitation of the OOK receiver, even with the perfect channel estimation, the effect of the fading channel may not be fully eliminated in the end. Hence there could be two methods on the ideal RSRP:

- Method 1: The first one is the traditional method where RSRP is pass the fading channel and with the perfect channel estimation, i.e., fully knowledge of the fading channel, and the real RSRP pass the same channel. Although the OOK receiver does not have any advanced capability to counteract the impact of the fading channel, due to the same fading channel is used for ideal RSRP and real RSRP, the fading channel impact will be almost cancelled each other. however still assumes the impact of the fading channel can be fully removed, hence in simulation the ideal RSRP need not pass the fading channel.

- Method 2: The second one assumes the impact of the fading channel can be completely removed hence the ideal RSRP does not need pass the channel whereas the real RSRP will pass the fading channel. Due to the fading channel between ideal and real RSRP does not counteract each other, this method, to some extent, shows the impact of the fading channel on the performance due to limited capability of the receiver (OOK based). Actually, this method is close the worst case due to the limited capability of the OOK receiver.

Proposal 3: For ideal RSRP in simulation, suggest to discuss and align the ideal RSRP calculation, either method is ok.

Issue 2-1-6: On SCS in simulation

- Proposals

  - P1: RAN4 to agree data and control channel subcarrier spacing as follow. (Ericsson)

    - OFDM based: The same as SSB subcarrier spacing

    - OOK based: the same as one of the SCS(s) used for other NR transmission

  - P2: RAN4 to agree only using SCS = 30KHz in the simulation. (vivo Ericsson)

Recommendations:

Check whether only using SCS = 30KHz in the simulation is agreeable

At RAN1 118bis meeting the following agreement is available:

Agreement

From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

However, considering the simulation working amount, it is suggested to still focus on 30K Hz SCS.

Proposal 4: In simulation, SCS is still based on 30K Hz.

Issue 2-1-7: On OOK interference

- Proposals

  - P1: RAN4 RRM to clarify simulation assumptions related to potentially colliding OOK into OOK interference. For OFDM interference, confirm the assumption of a random QPSK source (Apple)

Recommendations:

It is ok to align the interference LP-SS and the alignment can be done after the LP-SS sequences are fixed by RAN1.

Issue 2-1-8: On overlaid OFDM sequences in LP-SS

- Proposals

  - P1: For overlaid sequences, RAN4 to consider two scenario 1) no ZC sequence for both cells, and 2) different ZCs for each cell. FFS details (Apple)

Regarding overlaid OFDM sequences in LP-SS, the following RAN1 agreements are available:

RAN1 Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

In RAN4 LP-WUR core part, whether to define requirements for LP-SS with overlaid OFDM sequences are under discussion. However even if requirements for LP-SS with overlaid OFDM sequences will be defined, its requirement will reuse the OOK based LP-WUR requirement hence there is no necessity to have simulation and align simulation parameter for the overlaid OFDM sequence for LP-SS.

Proposal 5: No necessity to align simulation parameter for the overlaid OFDM sequence for LP-SS.

Issue 2-1-9: On LP-SS preamble and LP-SS periodicity

- Proposals

  - P1: RAN4 RRM to consider LP-SS periodicities of 320ms and above and to discuss the implications on LP-SS performance the use of a preamble in LP-WUS (Apple)

To our understanding the preamble for LP-WUS discussion should be in RAN1 scope.

Issue 2-1-11: On simulation campaign

- Proposals

  - P1: RAN4 RRM to compile a simulation results summary spreadsheet to be updated during every RAN4 meeting based on the contributions of the interested companies. RAN4 RRM to agree on an initial summary of simulations spreadsheet format, including an interested company to take care of it, by the end of RAN4#113. (Apple)

  - P2: Agree on a set of simulation scenarios each company provides to the next meeting (Nokia)

Recommendations:

The principle of P1 is ok and the following table is provided as an example as one summary table in the spreadsheet.

Proposal 6: Use the following table as one summary table in the spreadsheet used for simulation result alighment.

| Number of samples for 3.5 dB accuracy, LP-SS signal (SSB), AWGN(TDL-C), synchronous case (asynchronous case), SINR = [-3]dB, SCS = 30kHz; [10] ppm Frequency offset Set 1: SNR of cell 2 is 6 dB lower than cell 1; Set 2: SNR of cell 2 is 9 dB lower than cell 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company |  | Company |  | Company |  | Company |  |
|  | Set 1 | Set 2 | Set 1 | Set 2 | Set 1 | Set 2 | Set 1 | Set 2 |
| M=1,LP-SS=8 |  |  |  |  |  |  |  |  |
| M=2,LP-SS=8 |  |  |  |  |  |  |  |  |
| M=4,LP-SS=16 |  |  |  |  |  |  |  |  |

Simulation results

Our simulation results are provided below and consider the latest RAN4 agreement on accuracy for simulation purpose. The corresponding simulation parameters can be found at appendix.

- For FR1:

  - For OOK based LP-WUR, {3.5, 2.5} dB for LP-RSRP and {3.5, 2.5} dB for LP-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

  - For OFDM based LP-WUR, {3.5, 2.5} dB for SS-RSRP and {3.5, 2.5} dB for SS-RSRQ

    - 1st priority: 3.5dB, 2nd priority: 2.5dB

- Ideal RSRP simulated using method 1 in Issue 2-1-5 (pass channel)

- OOK

Table 1 Delta RSRP/RSRQ value for OOK receiver, asynchronous case with ideal RSRP pass channel, TDL-C Channel with 3ms delay for neighbour interference

| TDL-C (3ms Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.54 | 1.95 | 1.59 | 1.94 |  |
|  | M=2,LP-SS=8 | 1.24 | 2.01 | 1.34 | 2.03 |  |
|  | M=4,LP-SS=16 | 1.36 | 1.99 | 1.45 | 2.02 |  |
| Set2 | M=1,LP-SS=8 | 1.52 | 1.93 | 1.74 | 1.99 |
|  | M=2,LP-SS=8 | 1.22 | 1.97 | 1.58 | 2.19 |
|  | M=4,LP-SS=16 | 1.32 | 1.98 | 1.67 | 2.15 |

Table 2 Delta RSRP/RSRQ value for OOK receiver with ideal RSRP pass channel, synchronous case, TDL-C Channel with CP/2 delay for neighbour interference

| TDL-C(CP/2 Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.51 | 1.91 | 1.51 | 1.91 |
|  | M=2,LP-SS=8 | 1.24 | 1.99 | 1.25 | 1.99 |
|  | M=4,LP-SS=16 | 1.37 | 1.98 | 1.35 | 1.97 |
| Set2 | M=1,LP-SS=8 | 1.42 | 1.89 | 1.48 | 1.91 |
|  | M=2,LP-SS=8 | 1.23 | 1.98 | 1.26 | 2.02 |
|  | M=4,LP-SS=16 | 1.31 | 1.98 | 1.35 | 2.01 |

- OFDM

Table 3 Delta RSRP/RSRQ value with SSB signal for OFDM receiver in ideal RSRP pass channel, , AWGN channel and interference from neighbour SSB signals

| AWGN |  | CP/2 delay for neighbor interference | 3ms delay for neighbor interference |
| --- | --- | --- | --- |
| RSRP 1 sample SSB interference (unit dB) | Set1 | 0.851 | 0.846 |
|  | Set2 | 0.829 | 0.816 |
| RSRQ 1 sample SSB interference (unit dB) | Set1 | 0.737 | 0.733 |
|  | Set2 | 0.711 | 0.708 |

Table 4 Delta RSRP/RSRQ value with SSB signal for OFDM receiver in ideal RSRP pass channel,  TDL-C channel and interference from neighbour SSB signals

| TDL-C |  | CP/2 delay for neighbor interference | 3ms delay for neighbor interference |
| --- | --- | --- | --- |
| RSRP 1 sample SSB interference (unit dB) | Set1 | 1.171 | 1.171 |
|  | Set2 | 1.139 | 1.137 |
| RSRQ 1 sample SSB interference (unit dB) | Set1 | 2.004 | 2.003 |
|  | Set2 | 2.005 | 2.005 |

We have the following observations from the above simulation results:

- For RRM measurement purpose, LP-SS based LR can satisfy RSRP/RSRQ measurement accuracy based on almost 1 LP-SS sample and 1 LP-SS samples at SINR = -3dB under AWGN and TDL-C channel, respectively, based on 320ms periodicity of LP-SS.

- For RRM measurement purpose, SSB based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 samples at SINR = -3dB under TDL-C channel.

- Ideal RSRP simulated using method 2 in Issue 2-1-5 (not pass channel)

- OOK

Table 5 Delta RSRP/RSRQ value for OOK receiver in ideal RSRP not pass channel, asynchronous case, AWGN Channel with 3ms delay for neighbour interference

| AWGN (3ms Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.130 | 1.076 | 1.128 | 1.068 |
|  | M=2,LP-SS=8 | 0.928 | 0.852 | 0.959 | 0.801 |
|  | M=4,LP-SS=16 | 0.970 | 0.864 | 0.920 | 0.815 |
| Set2 | M=1,LP-SS=8 | 1.068 | 1.031 | 1.348 | 1.160 |
|  | M=2,LP-SS=8 | 0.884 | 0.804 | 1.251 | 0.953 |
|  | M=4,LP-SS=16 | 0.913 | 0.815 | 1.043 | 0.839 |

Table 6 Delta RSRP/RSRQ value for OOK receiver in ideal RSRP not pass channel, synchronous case, AWGN Channel with CP/2 delay for neighbour interference

| AWGN (CP/2 Delay) |  | RSRP 1 sample NR interference (unit dB) | RSRQ 1 sample NR interference (unit dB) | RSRP 1 sample LP-SS interference (unit dB) | RSRQ 1 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 1.127 | 1.079 | 1.077 | 1.037 |
|  | M=2,LP-SS=8 | 0.954 | 0.872 | 0.924 | 0.827 |
|  | M=4,LP-SS=16 | 0.950 | 0.877 | 0.936 | 0.846 |
| Set2 | M=1,LP-SS=8 | 1.059 | 1.016 | 1.038 | 0.993 |
|  | M=2,LP-SS=8 | 0.913 | 0.817 | 0.901 | 0.806 |
|  | M=4,LP-SS=16 | 0.896 | 0.816 | 0.903 | 0.826 |

Table 7 Delta RSRP/RSRQ value for OOK receiver in ideal RSRP not pass channel, asynchronous case, TDL-C Channel with 3ms delay for neighbour interference

| TDLC(3ms Delay) |  | RSRP 3 sample NR interference (unit dB) | RSRQ 3 sample NR interference (unit dB) | RSRP 3 sample LP-SS interference (unit dB) | RSRQ 3 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 2.851 | 2.323 | 2.841 | 2.299 |
|  | M=2,LP-SS=8 | 2.943 | 2.058 | 2.961 | 2.001 |
|  | M=4,LP-SS=16 | 2.927 | 2.066 | 2.904 | 2.046 |
| Set2 | M=1,LP-SS=8 | 2.863 | 2.309 | 2.867 | 2.318 |
|  | M=2,LP-SS=8 | 2.925 | 2.017 | 2.985 | 2.056 |
|  | M=4,LP-SS=16 | 2.898 | 2.018 | 2.975 | 2.083 |

Table 8 Delta RSRP/RSRQ value for OOK receiver in ideal RSRP not pass channel, synchronous case, TDL-C Channel with CP/2 delay for neighbour interference

| TDLC(CP/2 Delay) |  | RSRP 3 sample NR interference (unit dB) | RSRQ 3 sample NR interference (unit dB) | RSRP 3 sample LP-SS interference (unit dB) | RSRQ 3 sample LP-SS interference (unit dB) |
| --- | --- | --- | --- | --- | --- |
| Set1 | M=1,LP-SS=8 | 2.806 | 2.309 | 2.806 | 2.327 |
|  | M=2,LP-SS=8 | 2.925 | 2.003 | 2.952 | 2.047 |
|  | M=4,LP-SS=16 | 2.873 | 2.042 | 2.934 | 2.064 |
| Set2 | M=1,LP-SS=8 | 2.835 | 2.281 | 2.824 | 2.307 |
|  | M=2,LP-SS=8 | 2.950 | 1.984 | 2.951 | 2.019 |
|  | M=4,LP-SS=16 | 2.867 | 2.002 | 2.909 | 2.064 |

We have the following observations from the above simulation results:

- For RRM measurement purpose, when ideal RSRP/RSRQ is not passed channel, LP-SS based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 LP-SS sample and 3 LP-SS samples at SINR = -3dB under AWGN and TDL-C channel, respectively, based on 320ms periodicity of LP-SS.

- OFDM

Table 9 Delta RSRP/RSRQ value with SSB signal for OFDM receiver in ideal RSRP not pass channel, AWGN channel and interference from neighbour SSB signals

| AWGN |  | CP/2 delay for neighbor interference | 3ms delay for neighbor interference |
| --- | --- | --- | --- |
| RSRP 1 sample SSB interference (unit dB) | Set1 | 0.851 | 0.845 |
|  | Set2 | 0.830 | 0.816 |
| RSRQ 1 sample SSB interference (unit dB) | Set1 | 0.736 | 0.733 |
|  | Set2 | 0.711 | 0.707 |

Table 10 Delta RSRP/RSRQ value with SSB signal for OFDM receiver in ideal RSRP not pass channel,  TDL-C channel and interference from neighbour SSB signals

| TDL-C |  | CP/2 delay for neighbor interference | 3ms delay for neighbor interference |
| --- | --- | --- | --- |
| RSRP 1 sample SSB interference (unit dB) | Set1 | 2.840 | 2.841 |
|  | Set2 | 2.878 | 2.878 |
| RSRQ 1 sample SSB interference (unit dB) | Set1 | 1.818 | 1.820 |
|  | Set2 | 1.798 | 1.802 |

We have the following observations from the above simulation results:

- For RRM measurement purpose, when ideal RSRP/RSRQ is not passed channel, SSB based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 samples at SINR = -3dB under TDL-C channel.

Conclusion

In this contribution, we provide our considerations on the LP-WUS/WUR WI and have the following observations and proposals.

- The agreed residual timing error is the upper bound of the timing error after sync process.

- For RRM measurement purpose, LP-SS based LR can satisfy RSRP/RSRQ measurement accuracy based on almost 1 LP-SS sample and 1 LP-SS samples at SINR = -3dB under AWGN and TDL-C channel, respectively, based on 320ms periodicity of LP-SS.

- For RRM measurement purpose, SSB based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 sample at SINR = -3dB under TDL-C channel.

- For RRM measurement purpose, when ideal RSRP/RSRQ is not passed channel, LP-SS based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 LP-SS sample and 3 LP-SS samples at SINR = -3dB under AWGN and TDL-C channel, respectively, based on 320ms periodicity of LP-SS.

- For RRM measurement purpose, when ideal RSRP/RSRQ is not passed channel, SSB based LR can satisfy RSRP/RSRQ measurement accuracy based on 1 sample at SINR = -3dB under AWGN channel and 1 sample at SINR = -3dB under TDL-C channel.

- Proposal 1: In RAN4 simulation, -3dB SINR is used as the side condition of accuracy requirements for both OOK based and OFDM based LR. -6 dB SINR is low priority and depending on company input.

Proposal 2: LP-SS sequence used for RAN4 simulation should be updated based on RAN1’s conclusion after RAN1 #120 meeting. In case no detail LP-SS sequences after RAN1 #120, the following LP-SS sequence could be used for RAN4 simulation alignment.

- M=1 LP-SS Len=8 	seq:1 0 1 0 1 0 1 0;

- M=2 LP-SS Len=8 	seq:1 0 0 1 1 0 0 1;

- M=4 LP-SS Len=16 	seq:1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1;

Proposal 3: For ideal RSRP in simulation, suggest to discuss and align the ideal RSRP calculation, either method is ok.

Proposal 4: In simulation, SCS is still based on 30K Hz.

Proposal 5: No necessity to align simulation parameter for the overlaid OFDM sequence for LP-SS.

Proposal 6: Use the following table as one summary table in the spreadsheet used for simulation result alighment.

| Number of samples for 3.5 dB accuracy, LP-SS signal (SSB), AWGN(TDL-C), synchronous case (asynchronous case), SINR = [-3]dB, SCS = 30kHz; [10] ppm Frequency offset Set 1: SNR of cell 2 is 6 dB lower than cell 1; Set 2: SNR of cell 2 is 9 dB lower than cell 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Company |  | Company |  | Company |  | Company |  |
|  | Set 1 | Set 2 | Set 1 | Set 2 | Set 1 | Set 2 | Set 1 | Set 2 |
| M=1,LP-SS=8 |  |  |  |  |  |  |  |  |
| M=2,LP-SS=8 |  |  |  |  |  |  |  |  |
| M=4,LP-SS=16 |  |  |  |  |  |  |  |  |

- References

# [1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), RAN 102

# [2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR ((LP-WUS/WUR), RAN 103

# [3] R4-2418284, Topic summary for [113][226] NR_LPWUS, RAN4 113,vivo

# [4] R4-2420100, WF on RRM requirements for NR_LPWUS, RAN4 113, vivo

# [5] TS38.133, V18.4.0

# Appendix Simulation parameters

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection M=1 LP-Sslen=8 seq: 1 0 1 0 1 0 1 0; M=2 LP-Sslen=8 seq: 1 0 1 0 0 1 0 1; M=4 LP-Sslen=16 seq: 1 0 0 1 0 1 1 0 0 1 1 0 1 0 0 1 ; | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |