3GPP TSG-RAN WG4 Meeting #114							        	R4-2501421

Athens, GR, 17th-21st February 2025

Title:	RRM Core requirements on event triggered L1 measurement reporting

Source:	China Telecom

Agenda item:			7.27.2.1

Document for:	Discussion

- Introduction

In the RAN4 #113 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 2-6: impact of time-to-trigger (TTT) on RRM

<Candidate options> Option 1: RAN4 to discuss whether to take TTT (time to trigger) into account in the L1 event-triggered reporting delay requirements once more agreements from RAN1/2 are available. (Nokia, Xiaomi) <Recommended WF> Continue discussion.

For time-to-trigger (TTT) of event triggered L1 measurement, RAN2 had achieved some conclusions, which are duplicated as follows:

RAN2-127bis: 11. TTT operates only based on a timer (like TTT used in L3 event triggered MR). RAN2-128: 1. TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. 2. TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. 3. TTT is applied to the leaving condition.

RAN2 has agreed that TTT (time to trigger) operates only based on a timer, like TTT used in L3 event triggered MR, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

In the previous meeting, we have agreed that the starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. However, RAN2 requests the condition needs to be satisfied for the whole duration of TTT. So, the event triggered reporting delay for L1 reporting is related to the duration of TTT. If it is configured as zero, there is no impact of time-to-trigger (TTT) on RRM.

Proposal 1: The event triggered reporting delay for L1 reporting is related to the duration of TTT.

Issue 2-7: whether to introduce/update requirements for reporting criteria per measurement category

<Candidate options> - Option 1: Yes (CATT, MTK, ZTE, Apple, OPPO) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC, Ericsson) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> Continue discussion.

Issue 2-8: whether to define a separate capability for supported number of events triggered L1 measurement reporting

<Candidate options> - Option 1: Yes (MTK, Samsung, CMCC) - Option 2: RAN4 to postpone the detailed discussion on ‘Requirements for reporting criteria per measurement category’ until the RRM requirements on the remaining issues are sorted out (QC) - Option 2a: RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support. (E///) - Option 3: RAN4 to align common understanding that the existing number of reporting criteria per category is defined for L3 measurement and reporting only. For L1 measurements, the number of events to be supported by UE shall be discussed in RAN1 first. (vivo) <Recommended WF> Continue discussion.

In legacy, requirements for reporting criteria per measurement category for L3 measurement and reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the requirements for reporting criteria per measurement category shall be defined too.

Both L3 measurement reporting and L1 measurement reporting could trigger LTM, no need for UE to be able to support different events of L3 and L1 in parallel per category. So we think a separate capability is better.

Proposal 2: Introduce requirements for reporting criteria per measurement category.

Proposal 3: Define a separate capability for supported number of events triggered L1 measurement reporting.

Issue 2-10: whether and how to coordinate with R19 MIMO

<Candidate options> - Option 1: (MTK) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events. - Optoin 2: Define separate capabilities for MIMO and mobility. (Samsung, CMCC, Apple) - Option 3: Discuss the coordination between LTM and MIMO events at the end of the Wis. (Nokia) <Recommended WF> - Continue discussion.

Although event triggered L1 measurement reporting are considered for both Rel-19 MIMO and Rel-19 Mobility, Rel-19 MIMO and Rel-19 Mobility are still separate features. And they are two parallel on-going work items, there may be some potential troubles with mixed requirements. So, we think it is better to define separate capabilities for MIMO and mobility.

Proposal 4: Define separate capabilities for MIMO and mobility.

Issue 2-11: L1 filtering assumption in event evaluation

<Candidate options> - Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW) - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC) <Recommended WF> Continue discussion.

On L1 filtering assumption in event evaluation. In RAN2 #126 meeting, it was agreed that RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. And In RAN1 #118bis, RAN1 had agreed that no L1 specified filtering for time and spatial domain is introduced. So, we think in event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

Proposal 5: In the event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: The event triggered reporting delay for L1 reporting is related to the duration of TTT.

Proposal 2: Introduce requirements for reporting criteria per measurement category.

Proposal 3: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 4: Define separate capabilities for MIMO and mobility.

Proposal 5: In the event triggered L1 measurement in mobility, do not consider L1 filter for requirements.

- References

[1] R4-2420101, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.