3GPP TSG-RAN WG4 Meeting #114							        	R4-2501423

Athens, GR, 17th-21st February 2025

Title:	RRM Core requirements on conditional intra-CU LTM

Source:	China Telecom

Agenda item:			7.27.2.3

Document for:	Discussion

- Introduction

In RAN2#128 meeting, some conclusions on conditional intra-CU LTM have been duplicated as follows[1]. From this meeting, we will continue to discuss the RRM Core requirements on conditional intra-CU LTM.

RAN2#128 1. The triggering condition of conditional LTM can be based on L3 measurement. 2. CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. 3. The L1 execution condition of a candidate cell is associated to only one triggering event. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 5. To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. 7. For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. 8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. 10. Candidate cell TA is maintained by a new timer. 11. For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

- Discussion

General on conditional LTM

Issue 1-1-2: Measurement requirements on conditional LTM

< Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting.

For this issue, RAN2 had achieved some conclusions in RAN2#128, which are duplicated as follow,

RAN2#128 1. The triggering condition of conditional LTM can be based on L3 measurement. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

RAN2 had agreed to introduce L3 measurements to CLTM, so we think L3 measurement based conditional LTM for FR1 should be supported in RAN4.

Proposal 1: Support L3 measurement based conditional LTM for FR1.

DL-sync, UL-sync and Early TCI state activate

Issue 1-2-1: Early UL Synchronization

For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

In RAN2, both UE based TA measurement and PDCCH ordered RACH are introduced for early UL Synchronization in CLTM. And in R18 LTM, UE based TA measurement is also agreed in RAN2, however RAN4 did not define the relate requirements due to time limit.

What is more, the benefactions of CLTM are shorter latency, less signaling and higher robustness, in such a condition, it seems that UE based TA measurement is better than PDCCH ordered RACH. So, we think it is necessary to define RRM requirements for UE based TA measurement.

Proposal 2: Define RRM requirements for UE based TA measurement.

Conditional Intra-CU LTM delay

Firstly, we should define the starting point of conditional intra-CU LTM delay. As a reference, the definitions the starting point and ending point of LTM delay and CHO delay are copied as follows

CHO: When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. LTM: LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

So, we think the starting point of conditional intra-CU LTM delay is When the UE receives a RRC message implying conditional intra-CU LTM. If we want to align with the definition of LTM delay, we can treat it as the starting point of conditional intra-CU LTM delay is the end of the last TTI containing the RRC command for conditional intra-CU LTM.

Proposal 3: The starting point of initial conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

For subsequent conditional LTM, we could follow the approach for subsequent CPAC, the starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Proposal 4: The starting point of subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Base on the definition of the conditional intra-CU LTM delay as shown above. The components of Conditional intra-CU LTM delay can contain these as follows

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +Tearly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM, and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmits the first UL message on the target cell.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

- Tearly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Proposal 5:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Due to this is the first meeting on conditional intra-CU LTM in all RAN WGs. We should revisit this issue when other WGs achieve further conclusions.

Proposal 6：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

- Summary

In this paper, we provide our views on conditional intra-CU LTM. From this discussion we have derived the following proposals:

Proposal 1: Support L3 measurement based conditional LTM for FR1.

Proposal 2: Define RRM requirements for UE based TA measurement.

Proposal 3: The starting point of initial conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 4: The starting point of subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Proposal 5:

DIntra-CLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.

Where:

- TRRC is the RRC procedure delay.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional intra-CU LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

- Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

- TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

- Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further.

Further, the components of Tinterrupt could be defined as Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

Proposal 6：Revisit conditional intra-CU LTM delay when other WGs achieve further conclusions.

- References

[1] RP-242790, Status Report to TSG, Apple Inc.

[2] R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom.